3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211869

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.14.8.2.2

Source:	Apple Inc.

Title:	Discussion on PDCCH demod requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#103-e demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present our views on the open issues on PDCCH demodulation requirements.

# 2. Discussion

In [1] we have several agreements on PDCCH demodulation requirements. We address some of the open issues below.

Issue 3-1-1: Whether to define requirements for multi-slot PDCCH monitoring

- Option 1: Yes

  - Option 1a: Define one requirement for both UE supporting monitor one slot and two slots every four slots.

- Option 2: No

  - Option 2a: Implicitly test this in PDSCH requirements.

For operation in 52.6 - 71 GHz with 480kHz SCS and 960KHz support of multi-slot PDCCH is mandatory. Also, the UE is not expected to receive and monitor PDCCH every slot at high SCS. We don’t see a way around but to define requirements with multi-slot PDCCH monitoring for 480KHz SCS at least. Hence, we propose to define requirements with multi-slot PDCCH monitoring alone for FR2-2 with 480kHz SCS. We can use PDCCH monitoring every 1 slot every 4 slots, since that is mandatory for UE supporting 480KHz SCS.

Proposal #1: Define requirements for PDCCH demod with 480KHz SCS with multi-slot PDCCH monitoring – 1 slot every 4 slots.

The initial test cases for PDCCH were agreed in [1]. We think it would be impractical and unnecessary to define requirements for every combination of parameters and could use the same approach as Rel-15 to define a reduced set, while tests all AL and combinations of interleaver size/ REG bundle size.

The initial set of tests in the WF is a comprehensive set and can define requirements in FR2-2 with those parameters.

We provide results for alignment for the initial set of test cases in the table below.

Table 1: Initial Simulation results for PDCCH demod in FR2-2

| Test number | CBW/SCS | CORESET duration | CORESET RB | Intl. size | REG bundle size | Agg level | Prop. Condition | DCI format | Payload size | Ant. Config and corr Mtx | 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA10-200 | 1_0 | 40 | 1x2 Low | 5.6 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1.2 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low | -1.1 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | -4.4 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1.6 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low | -1.3 |

Proposal #2: Define PDCCH demod requirements for initial set of testcases agreed for FR2-2.

# 3. Conclusion

In this paper we present our views on the open issues for UE demodulation requirements definition for 52.6-71GHz. Our observations and proposals are captured below:

Proposal #1: Define requirements for PDCCH demod with 480KHz SCS with multi-slot PDCCH monitoring – 1 slot every 4 slots.

Proposal #2: Define PDCCH demod requirements for initial set of testcases agreed for FR2-2.

# Reference

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm Incorporated.