3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211870

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.14.8.2.4

Source:	Apple Inc.

Title:	Discussion on CQI reporting requirements for 52.6 - 71 GHz

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#103-e demodulation requirements for 52.6 - 71 GHz was discussed and way forward [1] was agreed. In this contribution we present our views on CQI reporting requirements.

# 2. Discussion

For CQI reporting requirements it was agreed to only define requirements in AWGN channel for FR2-2.

Issue 5-1-1: CBW/SCS Combination

- Option 1: Only 100MHz/120kHz

- Option 2: 100MHz/120kHz and 400MHz/480kHz

CQI reporting requirements in AWGN are typically introduced for a combination of SNR points to cover different modulation. With 480KHz SCS/400MHz the testable SNR is very limited. Also, testing CQI reporting with 120KHz SCS is sufficient and defining requirements with 480kHz doesn’t provide any extra coverage in our understanding. Hence, we propose to define requirements with 120KHz SCS/ 100MHz CBW for FR2-2.

Proposal #1: Define CQI reporting requirement with 120KHz SCS/ 100MHz CBW for FR2-2.

Issue 5-1-4: SNR value

- Option 1: Not consider SNR related to 64QAM with following reasons:

  - The lowest SNR with CQI reporting related to 64QAM is 12dB which is rather higher than maximum achievable SNRBB specified by RF part.

  - Phase noise has big impact on the BLER performance when SNR is equal or larger than 14dB which CQI reporting index is larger than 12.

- Option 2: Decide based on simulation results.

Introducing CQI reporting in higher SNR corresponding to 64QAM range is not feasible and practical given the testable SNR constraints in band n263. Hence, for CQI reporting requirements we propose to limit to QPSK and 16QAM SNR ranges for requirements in FR2-2 with 120KHz SCS.

Proposal #2: Limit CQI requirements in FR2-2 to SNR ranges corresponding to QPSK and 16QAM.

# 3. Conclusion

In this paper we present our views on the open issues for UE demodulation requirements definition for 52.6-71GHz. Our observations and proposals are captured below:

Proposal #1: Define CQI reporting requirement with 120KHz SCS/ 100MHz CBW for FR2-2.

Proposal #2: Limit CQI requirements in FR2-2 to SNR ranges corresponding to QPSK and 16QAM.

# Reference

- R4-2210665, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, Qualcomm Incorporated.