3GPP TSG-RAN4 Meeting #104-e	R4-2211893

# Online, 15 – 26 August, 2022

Agenda item:	9.9.2.1

Source:	Apple

Title:	On Pre-MG performance

Document for:	Discussion

# Introduction

Performance of Pre-MG was extensively discussed in RAN4#103e, with agreement captured in [1]. Test case configuration related agreements are duplicated below for information.

Sub-topic 2-1 Test case configuration Issue 2-1-1 Test cases for the different deployment scenarios Agreements: - Only need to define the test cases for SA Issue 2-1-2 Non-DRX or DRX Agreements: - Only define the test cases for Non-DRX cases Issue 2-1-3 CA or single carrier Agreements: - Define the test cases for single carrier cases firstly. The final agreements on TC list in issue 2-2-5 can be updated accordingly. - Whether the other additional test cases for CA can be FFS in the next meeting. Issue 2-1-4 Pre-MG activation mechanism Agreements: - Both UE autonomous and network-controlled pre-MG (de)activation mechanisms shall be verified. Issue 2-1-5 Which types of measurement reference signal measured Agreements: - The SSB reference signal can be configured for Pre-MG test cases. Issue 2-1-6 Which types of trigger events Agreements: - Define the test cases for BWP switching trigger event firstly. - Whether the other additional test cases for MO addition/remove can be FFS in the next meeting. Issue 2-1-7 per-UE/per-FR gap patterns Agreements: - per-UE gap pattern can be configured in Pre-MG testing Issue 2-1-8 Gap pattern Agreements: - #0 in FR1 and #13 for FR2 Issue 2-1-9 Testing procedure Agreements: - Both the activation/deactivation and measurement reporting can be fulfilled within one testing procedure. Issue 2-1-10 Test cases on FR1/FR2 Agreements: - Define the different TCs for FR1 and FR2

As can be observed, there are still some FFS left. In this contribution, we will further discuss the open issues and provide proposals accordingly.

# Discussion

The first issue is whether to introduce test case for CA:

Issue 2-1-3 CA or single carrier

Agreements:

- Define the test cases for single carrier cases firstly. The final agreements on TC list in issue 2-2-5 can be updated accordingly.

- Whether the other additional test cases for CA can be FFS in the next meeting.

Our preference is not to introduce additional test cases for CA. Determination of Pre-MG status is quite similar for single carrier and CA cases. For both network controlled and UE autonomous activation/deactivation, if UE can survive single carrier test case, we see no issue for the UE to survive under CA environment. In CA scenario, UE only needs to repeat the status checking for single CC case multiple times. On the hand, it is better to make all test cases applicable for non-CA capable UE, e.g. RedCap UE.

Based on above analysis, we propose not to introduce test cases for CA.

Proposal 1: not introduce additional test cases for Pre-MG in CA.

Another issue is about trigger events:

Issue 2-1-6 Which types of trigger events

Agreements:

- Define the test cases for BWP switching trigger event firstly.

- Whether the other additional test cases for MO addition/remove can be FFS in the next meeting.

With target to simplify test cases design, we suggest RAN4 focusing on BWP switching first, which is the most typical use case of this functionality. On the hand, MO addition/remove is handled via RRC reconfiguration. If UE fails to determine Pre-MG status after RRC reconfiguration, UE may not even be able to pass other Pre-MG test case since UE has to check initial status of Pre-MG after RRC configuration.

Proposal 2: no consider other additional test cases for MO addition/remove for Pre-MG in R17.

# Conclusion

In this contribution, we further discuss the open issues in Pre-MG topic. After discussion, the following conclusions are provided:

Proposal 1: not introduce additional test cases for Pre-MG in CA.

Proposal 2: no consider other additional test cases for MO addition/remove for Pre-MG in R17.

# References

- R4-2210586, WF on R17 NR MG enhancements – Pre-configured MG, Intel