3GPP RAN WG4 Meeting #104-e		R4-2211950

Electronic Meeting, Aug 15-Aug 26, 2022

Agenda item:	9.14.3.1

Source:	Murata Manufacturing Co., Ltd.

Title:	Multi-band relaxation for FR2-2 handheld UE

Document for:	Approval

- Introduction

In RAN4#103-e we discussed MBR value and agreed below [1].

Agreement: - MBR to be included in the requirements so this will be in draftCR - Value of MBR is [1] dB

In previous meeting we estimated feasible MBR value, but there seems no technical discussion [2]. In this contribution, we checked whether agreed value is feasible by simulation.

- Discussion

In order to evaluate the multi band relaxation value, we investigated the following three antenna types for this discussion.

- Multi band antenna which supports 28 GHz range and 60 GHz range

- Single band antenna which supports 28 GHz range

- Single band antenna which supports 60 GHz range

2.1	Analysis model of antenna

The simulated antenna structure is shown in Figure 1. There are 4 stack antennas which cover both 28 GHz range and 60 GHz range. In the middle of the stack antennas, there are 4 antennas which cover the 60 GHz range.

Figure 1. The simulated antenna for UE

In designing the antenna, we followed minimum peak EIRP discussion, for example, antenna size, number of antenna elements and so on. We used only 1 panel for CDF simulation.

2.2	Simulation result

The following result shows antenna gain at peak and spherical coverage.

Table 1. The simulation result of peak gain and spherical coverage

|  | Peak gain [dBi] |  | Spherical coverage [dBi] |  | Gain drop [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 28 GHz range | 60 GHz range | 28 GHz range | 60 GHz range | 28 GHz range | 60 GHz range |
| 28 GHz single band antenna | 10.1 | - | 1.4 | - | 8.8 | - |
| 60 GHz single band antenna | - | 14.9 | - | -0.5 | - | 15.4 |
| Multi band antenna | 9.2 | 14.7 | -0.5 | -0.8 | 9.7 | 15.6 |
| degradation | 0.9 | 0.2 | 1.9 | 0.3 | 0.9 | 0.2 |

Table 2 shows the specification of multi band relaxation.

Table 2. The specification of multi band relaxation ([1], [3])

| Band | MBP,n [dB] | MBS,n [dB] |
| --- | --- | --- |
| n257 | 0.7 | 0.7 |
| n263 | [1.0] | [1.0] |

Comparing Table 1 and Table 2, the degradation values of 28 GHz range are larger than specification of MBR.

28 GHz peak gain of multi band antenna is 0.9 dB lower than single band antenna. This degradation is a bit larger than our expectation. According to our analysis, 60 GHz antenna is coupled with 28 GHz antenna and it is one of the degradation factor. However, the difference between degradation and MBR specification is 0.2 dB. It is very small so we believe the current MBR specification value of band n257 should not be changed.

The degradation value of 28 GHz spherical coverage is 1.2 dB larger than MBR value but the gain drop of simulation is 1.2 dB smaller than specification. So Multi band antenna has enough gain for spherical coverage.

Table 3. The specification of Min peak EIRP and Spherical coverage ([3], [4], [5])

| Band | Min peak EIRP [dBm] | Spherical coverage [dBm] | Gain drop [dB] |
| --- | --- | --- | --- |
| n257 | 22.4 | 11.5 | 10.9 |
| n263 | 14.1 | 2.3 | 11.8 |

In 60 GHz range, the degradation values are smaller than the MBR values, but gain drop is 3.8 dB larger than specification because there is only 1 antenna panel in the simulation. In the RAN4#102-e discussion, gain drop was agreed by averaging 1 panel value 14.1 dB and 2 panels value 9.5 dB. If we use 2 antenna panels, spherical coverage will be 4.6 dB better, and we can meet specification value.

Observation 1:	We could design feasible multi band antenna which covers 28 GHz range and 60 GHz range by simulation.

Observation 2:	From the simulation result, the degradation value of peak gain is 0.2 dB and that of spherical coverage is 0.3 dB in the 60 GHz range.

From above simulation result and consideration, it seems that the MBR value can be smaller than 1.0 dB. However, considering the difficulty of suppressing frequency roll-off over the wide frequency range of n263, we could understand the necessity to add some margin. So, we believe it is feasible to fix the MBR value of band n263 to 1.0 dB.

Proposal 1:	Fix the MBR value of band n263 to 1.0 dB.

Table 4. Proposed MBR value

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n263 | 1.0 | 1.0 |

- Conclusions

Observation 1:	We could design feasible multi band antenna which covers 28 GHz range and 60 GHz range by simulation.

Observation 2:	From the simulation result, the degradation value of peak gain is 0.2 dB and that of spherical coverage is 0.3 dB in the 60 GHz range.

Proposal 1:	Fix the MBR value of band n263 to 1.0 dB.

- References

R4-2210578, “WF on 60 GHz UE Requirements,” Qualcomm.

R4-2210267, “Email discussion summary for [103-e][132] NR_ext_to_71GHz_Part_2,” Qualcomm.

TS 38.101-2 V17.6.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

R4-2207697, “Draft CR to 38.101-2 on band n263 Tx aspects,” Apple

R4-2211268, “Big CR on extending NR to 71GHz for TS 38.101-2,” Intel