3GPP TSG-RAN4 Meeting #104-e 		R4-2211965

E-meeting, 15th Aug. – 26th Aug, 2022

Title:	Discussion on RRM requirements for the combination of multiple concurrent gaps, pre-configured MG and NCSG

Source:	Xiaomi

Agenda Item:	11.10.2

Document for:	Discussion

- Introduction

In RAN#95-e plenary meeting, the WI of further enhancements on NR and MR-DC measurement gaps and measurements without gaps was approved in [1]. One of the objectives is to define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or multiple concurrent MGs and/or NCSG. In this contribution, we would like to discuss the related requirements and provide our proposals.

- Discussion

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or multiple concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MGs and multiple concurrent MGs (i.e., concurrent MGs where at least one of the gaps is a pre-configured gap) - Case 2: NCSG and multiple concurrent MGs (i.e., concurrent MGs where at least one of the gaps is NCSG) - Note: Prioritization among other possible combinations of pre-configured MG, concurrent MG and NCSG can be discussed in WI phase

Scenario

In this WI, RAN4 will define the joint requirements for multiple concurrent gaps, pre-configured MG and NCSG, and, case 1 and case 2 are prioritized. However, from our perspective, the case of concurrent gaps including pre-configured MG and NCSG is equally important as case 1 and case 2. For UE supporting NCSG, NW can configure pre-configured MG and NCSG to further reduce the scheduling gap of serving cell compared with the configuration of case 2.

Proposal 1: RAN4 define the joint requirements for UE configured with pre-configured MG and NCSG.

Association between the supported measurement type and MGs

In multiple concurrent MGs, the indication of association between the measurement type and the measurement gap should be configured to UE. When joint operation is configured, some of the measurement type may not be associated to a certain measurement gap, e.g. NCSG cannot be configured for CSI-RS based inter-frequency measurement, if UE is configured with concurrent gap and NCSG or pre-configured gap and NCSG, CSI-RS based inter-frequency measurement can only be associated to concurrent gap or pre-configured gap. Thus, the association between the supported measurement types and MGs are summarized in following table 1.

|  |  | Concurrent gap | NCSG | Pre-MG |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | activated | deactived |
| NR SSB based measurement | Intra-f w gap | Y | Y | Y |  |
|  | Intra-f w/o gap |  |  |  | Y |
|  | inter-f w gap | Y | Y | Y |  |
|  | inter-f w/o gap |  |  |  | Y |
| NR CSI-RS based measurement | Intra-f |  |  |  | Y |
|  | Inter-f | Y |  | Y |  |
| E-UTRAN Inter-RAT measurement |  | Y | Y |  |  |
| E-UTRA Inter-RAT RSTD measurement |  | Y |  | Y |  |
| E-UTRA Inter-RAT E-CID measurement |  |  |  | Y |  |
| NR PRS based measurement |  | Y |  |  |  |

Proposal 2: When UE is configured with joint configuration of multiple concurrent gaps, pre-configured MG and NCSG, the association between the supported measurement types and MGs are summarized in table 1.

|  |  | Concurrent gap | NCSG | Pre-MG |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | activated | deactived |
| NR SSB based measurement | Intra-f w gap | Y | Y | Y |  |
|  | Intra-f w/o gap |  |  |  | Y |
|  | inter-f w gap | Y | Y | Y |  |
|  | inter-f w/o gap |  |  |  | Y |
| NR CSI-RS based measurement | Intra-f |  |  |  | Y |
|  | Inter-f | Y |  | Y |  |
| E-UTRAN Inter-RAT measurement |  | Y | Y |  |  |
| E-UTRA Inter-RAT RSTD measurement |  | Y |  | Y |  |
| E-UTRA Inter-RAT E-CID measurement |  |  |  | Y |  |
| NR PRS based measurement |  | Y |  |  |  |

Measurement gap collision

When UE is configured with concurrent gap and pre-configured MG, the collision may happen if there is overlapping in time domain between concurrent gap and pre-configured MG. And the existing proximity condition and priority rule are applied when collision happens between the activated pre-configured MG and concurrent gap.

Proposal 3: The existing proximity condition and priority rule are applied when collision happens between the activated pre-configured MG and concurrent gap.

When collision between pre-configured MG and concurrent MG happens and the priority of pre-configured MG is higher than the concurrent MG, as shown in figure 1, the collided concurrent gap occasion should be dropped, however, if the status of pre-configured MG is changed from activated to deactivated, then the dropped concurrent gap occasion is available and the UE should measure with the dropped concurrent gap occasions.

Figure 1: Gap collision between pre-configured MG and concurrent gap

Proposal 4: When collision between pre-configured MG and concurrent MG happens and the priority of pre-configured MG is higher than the concurrent MG, and if the status of pre-MG is changed from activated to deactivated, then the UE shall measure within the dropped concurrent gap occasions.

When UE is configured with concurrent gap and NCSG, the proximity condition and gap collision may need to be revisited, since it is assumed that UE perform the measurement with NCSG in idle Rx chain. If the MGL of legacy gap is overlapped with the ML of NCSG, even through UE can simultaneously perform the measurement with legacy gap and NCSG, the reception in ML would be interrupted due to RF retuning time or the reception in MGL would be interrupted by VIL, as shown in figure 2 and 3. Thus, we think the existing priority rule can be applied for the collision between legacy gap and NCSG. And the proximity condition for gap collision may not be needed, since UE may use the different Rx chain to perform the measurement with legacy gap and NCSG respectively.

Figure 2: ML of NCSG is interrupted by RF retuning

Figure 3: Collision happens when VIL1 or VIL2 is overlapped with MGL

Observation 1: The existing proximity condition for gap collision is not needed for NCSG and concurrent gap or activated pre-configured MG.

Proposal 5: When the MGL of concurrent gap or the activated pre-configured MG is overlapped with the ML of NCSG, or when VIL1/VIL2 of NCSG is overlapped with the MGL of concurrent gap or the activated pre-configured MG, the NCSG and concurrent gap or activated pre-configured MG is considered as gap collision.

Proposal 6: The existing priority rule is applied when collision happens between concurrent gap/activated pre-configured MG and NCSG.

CSSF

In Rel-17 multiple concurrent gap, it was agreed that the CSSF should be calculated separately for each concurrent gap and the associated measurement types. And we think the same principle can be applied to the joint configuration for concurrent gaps, pre-configured MG and NCSG.

Proposal 7: CSSF is calculated separately for each concurrent gap and the associated measurement types, and the concurrent gap can be the legacy gap or activated pre-configured MG or NCSG.

- Conclusion

In this contribution, we discuss the related requirements for the joint configuration of concurrent gap, pre-configured MG and NCSG and provide our proposals as follows.

Proposal 1: RAN4 define the joint requirements for UE configured with pre-configured MG and NCSG.

Proposal 2: When UE is configured with joint configuration of multiple concurrent gaps, pre-configured MG and NCSG, the association between the supported measurement types and MGs are summarized in table 1.

|  |  | Concurrent gap | NCSG | Pre-MG |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | activated | deactived |
| NR SSB based measurement | Intra-f w gap | Y | Y | Y |  |
|  | Intra-f w/o gap |  |  |  | Y |
|  | inter-f w gap | Y | Y | Y |  |
|  | inter-f w/o gap |  |  |  | Y |
| NR CSI-RS based measurement | Intra-f |  |  |  | Y |
|  | Inter-f | Y |  | Y |  |
| E-UTRAN Inter-RAT measurement |  | Y | Y |  |  |
| E-UTRA Inter-RAT RSTD measurement |  | Y |  | Y |  |
| E-UTRA Inter-RAT E-CID measurement |  |  |  | Y |  |
| NR PRS based measurement |  | Y |  |  |  |

Proposal 3: The existing proximity condition and priority rule are applied when collision happens between the activated pre-configured MG and concurrent gap.

Proposal 4: When collision between pre-configured MG and concurrent MG happens and the priority of pre-configured MG is higher than the concurrent MG, and if the status of pre-MG is changed from activated to deactivated, then the UE shall measure within the dropped concurrent gap occasions.

Observation 1: The existing proximity condition for gap collision is not needed for NCSG and concurrent gap or activated pre-configured MG.

Proposal 5: When the MGL of concurrent gap or the activated pre-configured MG is overlapped with the ML of NCSG, or when VIL1/VIL2 of NCSG is overlapped with the MGL of concurrent gap or the activated pre-configured MG, the NCSG and concurrent gap or activated pre-configured MG is considered as gap collision.

Proposal 6: The existing priority rule is applied when collision happens between concurrent gap/activated pre-configured MG and NCSG.

Proposal 7: CSSF is calculated separately for each concurrent gap and the associated measurement types, and the concurrent gap can be the legacy gap or activated pre-configured MG or NCSG.

- Reference

[1]	RP-221018	New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps,	MediaTek Inc, Intel Corporation