3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212017

# Electronic Meeting, 15th-26th, Aug, 2022

Source: 	Samsung

Title: 	On how to handle the fallbacks of the proposed band combination

Agenda Item:	10.1.1

Document for:	Discussion

- Introduction

The fallbacks should be carefully checked and treated when proposing the higher order band combination. Unfortunately in practice, the proponents always neglect them, and it wastes a lot of time for the rapporteurs to flag the missing fallbacks.

In this paper, we provide our suggestions on how to handle this issue.

- 2. Discussion

Actually, some approved principles and guidelines have already been clearly recorded in TR 38.862 clause 6.2.1.1 [1], cited as below:

#4 Proponents should prepare and submit the corresponding contributions, e.g. draft CR, TP before RAN4#X meeting. If a draft CR or TP is depending on approval of lower order fallbacks submitted at the same meeting, this need to be clearly mentioned in the cover sheet of the draft CR or in the heading of the TP.

#6 The contributions will be discussed during RAN4#X meeting. If there are no technical concerns and if all the needed fallbacks are completed, the band combinations can be approved. And the final decision will be made by chairman.

The common understanding is that the fallbacks shall be specified in advance or at least at the same meeting, the proposed higher order band combination could be approved when all the fallbacks are completed in advance or at least at the same meeting.

In the basket WID, rapporteur is supposed to clearly list all the fallback types in the “Justification” clause, including below items for an X DL/ Y UL band combination if any:

- Constituent LTE inter band CA

- Constituent NR inter band CA – FR1 part

- Constituent NR inter band CA – FR2 part

- (X-1) DL with Y UL (X-1≥Y, X≥3)

- X DL with Y-1 UL (2≤Y≤3)

Proposal 1: In the basket WID, rapporteur is supposed to clearly list the precondition to propose the band combination, at least including all the fallback types of this WID, while all the fallbacks shall be completed and specified in advance or at least at the same meeting.

Proposal 2: Proponents should follow the approved guideline in TR 38.862, i.e.

#4 Proponents should prepare and submit the corresponding contributions, e.g. draft CR, TP before RAN4#X meeting. If a draft CR or TP is depending on approval of lower order fallbacks submitted at the same meeting, this need to be clearly mentioned in the cover sheet of the draft CR or in the heading of the TP.

- 3. Conclusion

Proposal 1: In the basket WID, rapporteur is supposed to clearly list the precondition to propose the band combination, at least including all the fallback types of this WID, while all the fallbacks shall be completed and specified in advance or at least at the same meeting.

Proposal 2: Proponents should follow the approved guideline in TR 38.862, i.e.

#4 Proponents should prepare and submit the corresponding contributions, e.g. draft CR, TP before RAN4#X meeting. If a draft CR or TP is depending on approval of lower order fallbacks submitted at the same meeting, this need to be clearly mentioned in the cover sheet of the draft CR or in the heading of the TP.

- 4. Reference

[1] TR 38.862 V17.1.0