3GPP TSG-RAN4 Meeting #104-e	R4-2212022

# Electronic Meeting, Aug 15-26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 15.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cat F Rel-15 Draft CR to 38.101-1 update of simultaneous RxTx capability for band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI, Huawei, Hisilicon, CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Below capability description (approved as R4-2206485) is captured in current spec: “If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.” For example if CA_A-B, CA_A-C, CA_B-C are mandatory support simultaneous RxTx capability, when CA_A-B-C is working, the band pairs CA_A-B, CA_A-C, CA_B-C within the band combination are simultaneous RxTx, accordingly it could be deducted CA_A-B-C is working under mandatory Simultaneous Rx/Tx mode. It is also true for higher order band combinations. Therefore, we propose below description be added into section 5.2A and 5.2C to make the capability more specific and clear. This description shall be applied from Rel-15 TS 38.101 series, exactly as the above one. “If the mandatory simultaneous Rx/Tx capability applies for all the subsets of a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for this higher order band combination.” The benefit is operators could stop fixing the problem for higher order combinations by adding note which is inefficient. With the proposed description, the capability for higher order combination is clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following description is added into section 5.2A and 5.2C: “If the mandatory simultaneous Rx/Tx capability applies for all the subsets of a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for this higher order band combination.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Simultaneous RxTx capability for higher order band combinations is unclear, and inefficient “Adding note” solution still continues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A, 5.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The updated capability despciption for DC combinations is included in Rel-17 Cat F CR R4-2212025 section 5.2B, since higher order DC combiantions with mandatory simultaneous RxTx capability are specified as of Rel-17 of 38.101-1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchaged sections are omitted>

## 5.2A	Operating bands for CA

5.2A.1	Intra-band CA

NR intra-band contiguous carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1, where all operating bands are within FR1.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n411 | n41 |
| CA_n771 | n77 |
| CA_n781 | n78 |
| CA_n791 | n79 |
| NOTE 1: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

If the mandatory simultaneous Rx/Tx capability applies for all the subsets of a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for this higher order band combination.

Table 5.2A.2-1: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3-n771 | n3, n77 |
| CA_n3-n781 | n3, n78 |
| CA_n3-n791 | n3, n79 |
| CA_n8-n751 | n8, n75 |
| CA n8-n781 | n8, n78 |
| CA_n8-n791 | n8, n79 |
| CA_n28-n752 | n28, n75 |
| CA_n28-n781 | n28, n78 |
| CA_n41-n78 | n41, n78 |
| CA_n75-n781 | n75, n78 |
| CA_n76-n781 | n76, n78 |
| CA_n77-n79 | n77, n79 |
| CA_n78-n793 | n78, n79 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 4: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. |  |

5.2B	Void

5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, where all operating bands are within FR1.

If the mandatory simultaneous Rx/Tx capability applies for a band combination, the mandatory simultaneous Rx/Tx capability also applies for the band combination when the applicable band combination is a subset of a higher order band combination.

If the mandatory simultaneous Rx/Tx capability applies for all the subsets of a higher order band combination, the mandatory simultaneous Rx/Tx capability also applies for this higher order band combination.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier. |  |

<Unchaged sections are omitted>