3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211511

# Electronic Meeting, August 15 – August 26, 2022

3GPP TSG RAN WG1#109-e			R1-2205582

e-Meeting, May 9th – 20th, 2022

Title:	LS on TCI assumption for RSSI measurement for FR2-2

Release:	17

Work Item:	NR_ext_to_71GHz-Core

Source:	Qualcomm

To:	TSG RAN WG4, TSG RAN WG2

Cc:

Contact person:	Jing Sun

jingsun@qti.qualcomm.com

# 1	Overall description

In RAN1 #109e, RAN1 discussed UE TCI assumption for RSSI measurements in FR2-2.

Agreement (RAN1#108-e)

For the QCL Type-D of L3-RSSI measurement for unlicensed operation in FR2-2, if explicit TCI state is configured, use the TCI state.

- Use the QCL type-D of the latest PDSCH reception or latest CORESET monitoring for RSSI measurement, if the explicit TCI state is not configured.

- A dynamic update mechanism for TCI-State in RMTC-Config is not further considered in Rel.17

- The explicit TCI state is configured at least in RMTC-Config

- Note: For inter-frequency L3-RSSI measurement, the TCI state configured is with respect to the target frequency TCI state

- Note2: For a given L3-RSSI measurement occasion, the UE needs to identify the last PDSCH reception or last configured CORESET monitoring (which ever is later) before the L3-RSSI measurement occasion, and use the QCL Type-D of that for L3-RSSI monitoring

The following is a recommended text proposal to be considered in TS 38.133.

## *** < Beginning of TP#0 for TS 38.133 v17.5.0> ***

### 9.2A.7	Intra-frequency RSSI and Channel occupancy measurements

#### 9.2A.7.1	Intra-frequency RSSI measurements

An RSSI measurement is defined as an intra-frequency measurement provided that the RSSI measurement bandwidth is fully contained within the current carrier bandwidth of the UE.

The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more serving carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity.

For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier.

*** < Unchanged parts are ommitted> ***

### 9.3A.8	Inter-frequency RSSI measurements

An RSSI measurement is defined as an inter-frequency measurement provided that the RSSI measurement bandwidth is not contained within the current carrier bandwidth of the UE.

The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more inter-frequency carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity. The requirements apply if rmtc-SubframeOffset [2] is configured.

For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell, UE can assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state of the active BWP of the carrier on which the RMTC configuration is provided. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided.

*** < End of TP#0 for TS 38.133 v17.5.0> ***

# 2	Actions

To TSG RAN4

ACTION: 	RAN1 respectfully asks RAN4 to consider the above recommendations and capture them in RAN4 spec.

To TSG RAN2

ACTION: 	RAN1 respectfully asks RAN2 to consider the text proposal above as a recommendation to RAN4 spec (i.e., TS 38.133) and, if deemed necessary, specify it into relevant RAN2 spec.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #110	22 - 26 August 2022	 		Toulouse, FR

TSG RAN WG1 Meeting #110-bis-e	10 - 19 October 2022	 		Online