3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211524

# Electronic Meeting, August 15 – August 26, 2022

3GPP TSG-RAN WG2 Meeting #118-e			Tdoc R2- 2206662

Electronic Meeting, May 9th - 20th, 2022

Title:	Reply LS on NCD-SSB issues for RedCap UE

Response to:	LS on NCD-SSB issues for RedCap UE (R4-2207104)

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN2

To:	RAN4

Cc:

Contact Person:

#### Name:	Emre A. Yavuz

E-mail Address:	emre dot yavuz at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 would like to thank RAN4 for the LS on NCD-SSB issues for RedCap UEs and provide the following replies for the questions RAN4 has asked:

Regarding the measurement related questions:

Q1 Is it possible to configure CD-SSB and/or multiple NCD-SSB(s) for serving cell measurements?

Answer: RAN2 has confirmed in RAN2#118-e that when active BWP of a RedCap UE is associated with NCD-SSB, it is up to network to configure the UE to perform serving cell measurements on NCD-SSB or CD-SSB. RAN2 has agreed that a BWP specific serving cell measurement object, i.e., servingCellMO defined under BWP-DownlinkDedicated, can be configured when BWP is associated with NCD-SSB. A RedCap UE uses this servingCellMO for serving cell measurements when configured in the active BWP, otherwise, it uses the legacy servingCellMO defined under ServingCellConfig. It is possible to configure CD-SSB and/or multiple NCD-SSB for serving cell measurements, but the UE shall perform the measurements using the configured serving cell measurement object of the active BWP which can either be associated with CD-SSB or NCD-SSB.

Q2 If yes, is it feasible from RAN2’s signaling design perspective to explicitly indicate which SSB is the reference SSB to define intra-frequency measurement?

Answer: From RAN2 signaling point of view, a BWP-specific servingCellMO can be configured under BWP-DownlinkDedicated. The SSB defined in this servingCellMO is the reference SSB to be used for serving cell measurements when the UE is in this active BWP; if the field is absent, SSB defined in servingCellMO under ServingCellConfig is the reference SSB to be used for serving cell measurements. This reference SSB is used to define intra-frequency measurements.

Regarding the handover related questions:

Q3 Are the following Handover scenarios valid from RAN2’s perspective?

  - Handover to a target cell’s specific Redcap BWP associated with NCD-SSB directly other than to the initial BWP associated with CD-SSB

  - Handover to a target cell’s initial BWP and further switch to the specific Redcap BWP to send the RACH

    - the specific Redcap BWP associated with NCD-SSB

    - the specific Redcap BWP without presence of NCD-SSB

Answer: From RAN2 standpoint, handover scenario a) is supported whereas scenario b) is not supported.

2. Actions:

To RAN4

ACTION: 	RAN2 kindly asks RAN4 to take the information above into consideration.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #119	22nd - 26th Aug 2022		e-Meeting

TSG-RAN WG2 Meeting #119bis-e	10th - 26th Oct 2022 		e-Meeting