3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212165

# Electronic Meeting, August 15 - 26, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Band n256 requirement gap for out of-band blocking

Agenda Item:	9.11.4.2	RX requirements	[NR_NTN_solutions-Core]

Document for:	Approval

# Introduction

In the RAN4#103e meeting the NTN UE specification [1] was finalized. In particular, an agreement was reached on band n256 OOB blocking to enable the implementation using an n65 duplexer. However, the agreed modifications for range 2 creates a gap between range 1 and range for the lower frequency range. In this contribution, we detail the issue and propose a solution.

# Discussion

## Gap in OOB blocking requirement

Table 7.6.3-2 is reproduced here from the latest 38.101-5 specification [1]. As is highlighted in yellow, the modified frequencies for range 2 create a 40MHz gap between range 1 and range 2 where no blocking performance is specified for the lower frequency range.

Table 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | [-30] | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For band n256 in Range 2 requirement, the applicable lower frequency range should be modified as -[145] < f – FDL_low ≤ [-100]. NOTE 2: For band n256 in Range 3 requirement, the applicable lower frequency range should be modified as 1 ≤ f ≤ FDL_low –[145] NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

## Proposed fix for the OOB blocking requirement

The easiest way to fix the problem is to extend the range 1 to -100MHz from the nominal -60MHz. Since this may become very cumbersome to further implement using notes, it may be easier to create a specific row for n256 as has already been done for band specific blocking requirements in 38.101-1 (band n71 for example).

Proposal on corrected table:

Table 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -303 | -154 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | [-100] < f – FDL_low < -15 or 15 < f – FDL_high < 60 | [-145] < f – FDL_low ≤ [-100] or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – [145] or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Void NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

# Conclusions

In this contribution, we observe that the current OOB blocking range for n256 has a 40MHz gap between Range 1 and 2, in order to solve the issue, we propose the following.

Proposal on corrected table:

Table 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -303 | -154 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | [-100] < f – FDL_low < -15 or 15 < f – FDL_high < 60 | [-145] < f – FDL_low ≤ [-100] or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – [145] or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Void NOTE 3: For band n256 in Range 2 requirement, the Pinterferer should be modified as [-30] NOTE 4: For band n256 in Range 3 requirement, the Pinterferer should be modified as [-15] |  |  |  |  |  |

# References

- R17 TS 38.101-5-h00, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 17)