# 3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2212808

Electronic Meeting, Aug. 15th – 26th, 2022

Source: 	vivo

Title: 	Discussion and reply LS on UE power limitation for STxMP in FR2

Agenda Item:	13.1.2

Document for:	Discussion

Introduction

A LS has been received from RAN1 for studies of simultaneous UL transmission across multiple UE panels (STxMP), in which the following two assumptions on power limitation were listed, and some questions were raised.

- Assumption 1: Power limitation per panel for STxMP

- Assumption 2: A total power limitation per UE over all UE panels used for STxMP

Above power limitation includes both total radiated power and EIRP, and scenarios of these assumptions include at least single carrier scenario in FR2.

RAN1 seeks a few answers from RAN4 on the following questions in order to proceed further on the study of UE power control for STxMP.

Question 1: From RAN4 perspective, is Assumption 1 is feasible?

Question 2: From RAN4 perspective, is Assumption 2 is feasible?

Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

In this paper, these questions are discussed, and some tentative answers were provided.

Discussion

RAN4 requirements for FR2

Currently, there are two main radiated power requirements used, which is TRP (Total Radiated Power) and EIRP (Effective Isotropic Radiated Power). These concepts are widely used in the industry and also in 3GPP. In RAN4 FR2 UE RF requirements, there exists maximum limitations for both of them, such as the following for PC3:

Table 6.2.1.3-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |
| n262 | 23 | 43 |

In the configured transmitted power part, there are following restrictions:

measured peak EIRP PUMAX,f,c:

PPowerclass + PIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

measured total radiated power:

PTMAX,f,c ≤ TRPmax

Among the two metrics, EIRP is mainly used with Pcmax/Pumax concept which is related to the UE power control and configuration behaviour, and this is actually “perceived” by receiver. By comparison, the only use case of PTMAX,f,c is   to satisfy the TRPmax requirements.

Though we do not have a strict definition for EIRP in the spec, it is the common understanding that peak EIRP is hypothetical power that would have to be radiated by an isotropic antenna to give the equivalent signal strength as the actual source antenna in the direction of the antenna’s strongest beam. One of the key parts of this definition, is the current common understanding of EIRP is combined with one beam direction.

Observation 1: Current RAN4 requirements have the upper limitations for max TRP and max EIRP.

Observation 2: Current EIRP concept is intrinsically combined with one beam direction.

Regulatory Requirements

The current max TRP and max EIRP regulatory requirements are defined in FCC 47 CFR Part 30, which basically covered bands in FR2 range, the sub-section 30.202 states:

(b) For mobile stations, the average power of the sum of all antenna elements is limited to a maximum EIRP of +43 dBm.

It should be noted that the regulatory requirements are quite general, which incorporate all the antennas in a mobile station (for a band).

For the TRP, there is no regulation requirements defined as such. However, this value of 23dBm has been used for Rel-15 co-existence study, and revising this value would mean that new assessments are needed, and basic requirements such as ACLR/ACS need to be revised. So, it is also preferred not to be changed.

Observation 3a: Regulatory requirements exist for EIRP, which are per-UE based for a band.

Observation 3b: Though no regulation basis for TRP, the current TRP requirements are also preferred to be kept as per-UE, otherwise more assessment would be needed for basic requirements.

Per-panel vs. Per-UE

Current implementation for FR2 is per-panel based, but there are no definition or specific requirements for a “panel” in the spec, since it is highly implementation specific. It is also likely to keep this informal “rule” in the future.

Observation 4: There is no specific “panel” based requirements in the spec.

Although multiple panels are already assumed at a UE since Rel-15, it is also assumed that only one panel would be active for transmission before Rel-18. So, there is no difference between per-panel and per-UE for transmission until now. However, for Rel-18, with the possible introduction of STxMP feature and simultaneous transmission from multiple panels, there could be difference between per-panel and per-UE, and the LS seems already have some different understandings.

For example, for single panel transmission, it naturally has a maximum transmission power. In case of simultaneous transmission by two panels, there would be a combined effect of two panels, e.g. two simultaneous Tx beams. In this case, it seems the concept of TRP can be easily extended by accounting all the transmission power of two beams. However, for EIRP it is still not that clear how the extension should be done, since the current EIRP concept corresponds to one direction, while there would be two simultaneous Tx beams for STxMP. One possible way is to assume a combined beam, however considering the  multi-TRP scenario, it is likely that a more typical scenario is that the two beams have considerable separation and more suitable to be treated separately, thus making a “combined beam” approach more difficult. In other words, assume two equally strong beam are transmitted with significantly distant directions, does it mean from EIRP point of view, the peak EIRP is the same as the case of only one beam is transmitted? Though seems strange, in this case, the sum of the peak EIRP of the two panels, may result in a same peak EIRP.

Observation 5: TRP concept can easily be extended to simultaneous multiple transmissions from multiple panels, while EIRP may need further clarification when extended to simultaneous multi-beam/multi-panel transmission. The current concept may lead to sum of two equal Tx beams to different directions have a same peak EIRP as the case of one beam transmission.

Questions and Answers

Here are some analyses on the questions in the LS.

Question 1: From RAN4 perspective, is Assumption 1 is feasible?

From feasibility perspective, it seems that the answer could be yes. Any panels should naturally satisfy a reasonable power limitation. In addition, the power control is actually based on a panel in current spec.

Question 2: From RAN4 perspective, is Assumption 2 is feasible?

Assumption 2 seems a mandatory case from regulatory point of view as pointed out previously. Besides, from feasibility point of view, based on current TRP and EIRP, it seems that it is also feasible. For example, in order to control TRP, the actual conducted power to different panels can be turned down. In order to control EIRP, the beam direction and power can be adjusted to avoid to be combined into a beam stronger than the regulatory requirements. Admittedly there would be some complexity for this kind of combination, but considering the targeted mTRP scenario, it is likely that a separate case is typical thus the complexity of combination may not be too serious.

In this sense, from feasibility perspective, it seems that the answer could be also be yes.

Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

For EIRP, the total power limitation per UE should be the same, since it is derived from regulatory requirements. Sum of the EIRP should also follow the current per-UE requirements.

In addition, some concept may need to be further clarified to better adapt to multi-panel / multi-beam transmissions. For example, since EIRP is based on one direction, sum of two beam with different directions may still comply with the same peak EIRP.

For TRP, though no regulatory requirements, the current TRP requirements are used for assumption of co-existence study, and is also preferred to be kept as per-UE, otherwise more assessment would be needed for basic requirements.

It should be noted that the current understanding of “sum of per-panel power limitation” is the “limitation of sum of per-panel power”, which is an actual reachable upper limit of the sum, not simple arithmetic summation of two per-panel power limitations which is 3dB larger than the per-UE limitation.

Observation 6: The current understanding of “sum of per-panel power limitation” is the “limitation of sum of per-panel power”, it is an actual reachable upper limit of the sum, not simple arithmetic summation of two per-panel power limitations which is 3dB larger than the per-UE limitation.

Since this may be confused, it is suggested to add this information as a note in the reply LS.

Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

It is believed that both assumptions are feasible, and both assumptions shall apply to a same UE. The per-panel power limitation can be the same to total power limitation, while both of them should be satisfied simultaneously. E.g., the sum of per-panel power limitation can be larger than the total power limitation per UE. However, it should be emphasized that the sum of the per-panel power should satisfy the per-UE requirement but as said, they should be satisfied simultaneously.

Conclusion

In this paper, some discussion was done and a list of observations and proposals are provided.

Observation 1: Current RAN4 requirements have the upper limitations for max TRP and max EIRP.

Observation 2: Current EIRP concept is intrinsically combined with one beam direction.

Observation 3a: Regulatory requirements exist for EIRP, which are per-UE based for a band.

Observation 3b: Though no regulation basis for TRP, the current TRP requirements are also preferred to be kept as per-UE, otherwise more assessment would be needed for basic requirements.

Observation 4: There is no specific “panel” based requirements in the spec.

Observation 5: TRP concept can easily be extended to simultaneous multiple transmissions from multiple panels, while EIRP may need further clarification when extended to simultaneous multi-beam/multi-panel transmission. The current concept may lead to sum of two equal Tx beams to different directions have a same peak EIRP as the case of one beam transmission.

Observation 6: The current understanding of “sum of per-panel power limitation” is the “limitation of sum of per-panel power”, it is an actual reachable upper limit of the sum, not simple arithmetic summation of two per-panel power limitations which is 3dB larger than the per-UE limitation.

A draft reply LS was attached.

- References

[1] RP-213577, Multi-carrier enhancements for NR, RAN#94e

[2] 38.101-2

[3] FCC 47 CFR Part 30

- Annex A

# 3GPP TSG-RAN WG4 Meeting # 104-e 	R4-21xxxxx

Electronic Meeting, Aug. 15th – 26th, 2022

Title:	[Draft] reply LS on UE power limitation for STxMP in FR2

Response to:	(R4-2211516_ R1-2205639) LS on UE power limitation for STxMP in FR2

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on UE power limitation for STxMP in FR2. Regarding the questions, RAN4 is currently have the following answers:

Question 1: From RAN4 perspective, is Assumption 1 is feasible?

Answer: Yes.

Question 2: From RAN4 perspective, is Assumption 2 is feasible?

Answer: Yes.

Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class?

For both TRP and EIRP, both the total power limitation per UE, and the upper limit of sum of per-panel powers should be the same to (no greater than) the existing power limitation for a given power class.

Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

It is believed that both assumptions are feasible, and both assumptions shall be applied to a same UE. The per-panel power limitation can be the same to total power limitation, while both of them should be satisfied simultaneously. E.g., the limitation of sum of per-panel power should be always the same to the per UE power limitation.

Note 1: The current understanding of “sum of per-panel power limitation” is the “limitation of sum of per-panel power”, which is an actual reachable upper limit of the sum, not simple arithmetic summation of two per-panel power limitation which is 3dB larger than the per-UE limitation.

Note 2: EIRP is defined based on one direction. For example, sum of two beam with different directions would still comply with the same peak EIRP.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above answers into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #104-bis-e	10 - 16 Oct. 2022        E-meeting