3GPP TSG-RAN WG4 Meeting#104-e                	R4-2212830

Electronic Meeting, 15 August – 26 August 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUSCH demodulation performance of Rel-17 NR coverage enhancements: simulation results

Agenda item: 	9.16.2.1

Document for: 	Discussion

Introduction

In this contribution, we provide the results of our simulation campaign considering the agreements in RAN4#103-e captured in [1][2][3] for both PUSCH TBoMS and PUSCH JCE.

PUSCH enhancements

TBoMS

Test parameters

We provide in this section the simulation results of TBoMS using the parameters below:

Table 1: TBoMS simulation parameters.

| Parameters | TBoMS (FDD and TDD) |
| --- | --- |
| Metric | SNR at 70% throuhput |
| MCS | MCS2 |
| Resource allocation | 5 PRB |
| Number of slots | 2 slots for TDD 4 slots for FDD |
| TDD pattern / FR | 30 kHz : 7D1S2U, S=6D:4G:4U For 15/60/120kHz SCS, 3D1S1U, S=10D:2G:2U |
| DMRS | DMRS 1+1 only and no PT-RS configured |
| Inter Intra slot frequency hopping | Disabled |
| Channel condition | higher speed channel model TDLB100-400 Low for FR1 and TDLA30-300 Low for FR2 |
| Antenna configuration | 1T2R |
| RV sequence | [0 2 3 1] |
| PUSCH mapping type | Cover both PUSCH mapping type A and type B for FR1 // Type B for FR2 |
| HARQ | 4 |
| Repetition | No repetition |
| transform precoding | CP-OFDM |
| DMRS/PTRS | DMRS 1+1 only and no PT-RS configured |
| DM-RS configuration type | 1 |
| DM-RS duration | single-symbol DM-RS |
| Additional DM-RS position | pos1 |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
| DM-RS port | 0 |
| DM-RS sequence generation | NID0=0, nSCID =0 |
| Start symbol | 0 |
| Allocation length | 14 for FR1 //10 for FR2 |
| Frequency density (KPT-RS) | Disabled |
| Time density (LPT-RS) | Disabled |

Simulation results: SNR at 70% throughput

The Table 2 gives the SNR at 70% throughput for two TBoMS configurations: TBoMS over 2 slots and TBoMS over 4 slots using Rx, different SCS, and both PUSCH mapping type 1 and PUSCH mapping type B.

Table 2: SNR at 70% throughput in case of TBoMS.

| TBoMS over |  |  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 4 available slots | 1T2R | FDD 15kHz SCS | TDLB100-400 Low | PUSCH Mapping type A | -4,37 | -1,87 |
|  |  |  |  | PUSCH Mapping type B | -3,37 | -0,87 |
| 2 available slots |  | TDD 15kHz SCS |  | PUSCH Mapping type A | -2,37 | 0,13 |
|  |  |  |  | PUSCH Mapping type B | -1,37 | 1,13 |
| 4 available slots |  | FDD 30kHz SCS |  | PUSCH Mapping type A | -0,37 | 2,13 |
|  |  |  |  | PUSCH Mapping type B | 0,63 | 3,13 |
| 2 available slots |  | TDD 30kHz SCS |  | PUSCH Mapping type A | 1,63 | 4,13 |
|  |  |  |  | PUSCH Mapping type B | 2,63 | 5,13 |
| 2 available slots |  | TDD 60kHz SCS | TDLA30-300 Low | PUSCH Mapping type B | -4,40 | -1,3 |
| 2 available slots |  | TDD 120kHz SCS |  | PUSCH Mapping type B | -4,20 | -1,1 |

PUSCH JCE

Test parameters

We provide in this section the simulation results of PUSCH JCE using the parameters below:

Table 3: PUSCH JCE simulation parameters.

| PUSCH Parameters | JCE (FDD and TDD) |
| --- | --- |
| Metric | SNR at 70% throughput |
| MCS | MCS4 |
| Frequency resource allocation | Full applicable bandwidth: Minimum and largest bandwidth for each SCS . |
| Number of slots cTDW=aTDW | TDD: 2 slots // FDD: 8 slots |
| TDD pattern / FR | FR1 SCS 15/30 kHz: 7D1S2U, S=6D:4G:4U (TDD/FDD) FR2 60/120kHz: DDSUU, S=10G:2G:2U (TDD) |
| DM-RS configuration type | 1 |
| DM-RS duration | single-symbol DM-RS |
| Additional DM-RS position | For FR1: DMRS 1+1 and DMRS 1+0 |
|  | For FR2: DMRS 1+1 and no PT-RS configured for FR2 |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
| DM-RS port | 0 |
| DM-RS sequence generation | NID0=0, nSCID =0 |
| Inter slot frequency hopping | Disabled |
| Channel condition | TDLA30-10 for FR1 and TDLA30-75 for FR2 |
| Antenna configuration | 2 Rx 4Rx 8Rx |
| RV sequence | [0 3 0 3] for TDD [0 0 0 0] for FDD . |
| PUSCH mapping type | PUSCH mapping type A and type B for FR1 // only Type B for FR2 |
| Repetition | Rep type A only same as aTDW (2 slots) for TDD // 8 slots for FDD |
| transform precoding | CP-OFDM |
| Time resource allocation | 14 symbols |
| CFO | Do not consider CFO modelling |
| Phase offset model | with ideal phase offset |
| PTRS Frequency density (KPT-RS) | Disabled |
| PTRS Time density (LPT-RS) | Disabled |
| Code block group based PUSCH transmission | Disabled |

Simulation results: SNR at 70% throughput

The Table 4, Table 5 and Table 6 provide the SNR gain at 70% throughput in case of PUSCH with JCE using 1T2R, 1T4R and 1T8R respectively.

Table 4: SNR gain at 70% throughput using 1T2R

| JCE over |  |  |  | PUSCH mapping type | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | 1+0 | type A | -9,37 | -6,87 |
|  |  |  |  | type B | -9,43 | -6,93 |
|  |  |  | DMRS 1+1 | type A | -10,13 | -7,63 |
|  |  |  |  | type B | -10,12 | -7,62 |
|  | [FDD 15kHz SCS 50MHz] |  | DMRS 1+0 | type A | -10,08 | -7,58 |
|  |  |  |  | type B | -10,06 | -7,56 |
|  |  |  | DMRS 1+1 | type A | -10,12 | -7,62 |
|  |  |  |  | type B | -10,12 | -7,62 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | type A | -4,06 | -1,56 |
|  |  |  |  | type B | -4,08 | -1,58 |
|  |  |  | DMRS 1+1 | type A | -4,58 | -2,08 |
|  |  |  |  | type B | -4,64 | -2,14 |
|  | [TDD 15kHz SCS 50MHz] |  | DMRS 1+0 | type A | -4,73 | -2,23 |
|  |  |  |  | type B | -4,71 | -2,21 |
|  |  |  | DMRS 1+1 | type A | -4,91 | -2,41 |
|  |  |  |  | type B | -4,92 | -2,42 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | type A | -10 | -7,5 |
|  |  |  |  | type B | -10,01 | -7,51 |
|  |  |  | DMRS 1+1 | type A | -10,66 | -8,16 |
|  |  |  |  | type B | -10,62 | -8,12 |
|  | [FDD 30kHz SCS 100MHz] |  | DMRS 1+0 | type A | -10,48 | -7,98 |
|  |  |  |  | type B | -10,45 | -7,95 |
|  |  |  | DMRS 1+1 | type A | -11,24 | -8,74 |
|  |  |  |  | type B | -11,22 | -8,72 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | type A | -4,47 | -1,97 |
|  |  |  |  | type B | -4,46 | -1,96 |
|  |  |  | DMRS 1+1 | type A | -4,88 | -2,38 |
|  |  |  |  | type B | -4,86 | -2,36 |
|  | [TDD 30kHz SCS 100MHz] |  | DMRS 1+0 | type A | -4,91 | -2,41 |
|  |  |  |  | type B | -4,9 | -2,4 |
|  |  |  | DMRS 1+1 | type A | -5,41 | -2,91 |
|  |  |  |  | type B | -5,43 | -2,93 |
| 2 slots | TDD 60kHz SCS 50MHz | TDLA30-75 Low | DMRS 1+1 | type B | -4,37 | -1,27 |
|  | [TDD 60kHz SCS 100MHz] |  |  | type B | -4,63 | -1,53 |
|  | TDD 120kHz SCS 50MHz |  | DMRS 1+1 | type B | -4,94 | -1,84 |
|  | [TDD 120kHz SCS 200MHz] |  |  | type B | -5,13 | -2,03 |

Table 5: SNR gain at	 70% throughput using 1T4R.

| JCE over |  |  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | DMRS 1+0 | type A | -13,08 | -10,58 |
|  |  |  |  | type B | -12,98 | -10,48 |
|  |  |  | DMRS 1+1 | type A | -13,79 | -11,29 |
|  |  |  |  | type B | -13,72 | -11,22 |
|  | [FDD 15kHz SCS 50MHz] |  | DMRS 1+0 | type A | -13,19 | -10,69 |
|  |  |  |  | type B | -13,16 | -10,66 |
|  |  |  | DMRS 1+1 | type A | -13,83 | -11,33 |
|  |  |  |  | type B | -13,82 | -11,32 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | type A | -7,84 | -5,34 |
|  |  |  |  | type B | -7,76 | -5,26 |
|  |  |  | DMRS 1+1 | type A | -8,39 | -5,89 |
|  |  |  |  | type B | -8,32 | -5,82 |
|  | [TDD 15kHz SCS 50MHz] |  | DMRS 1+0 | type A | -7,93 | -5,43 |
|  |  |  |  | type B | -7,92 | -5,42 |
|  |  |  | DMRS 1+1 | type A | -8,48 | -5,98 |
|  |  |  |  | type B | -8,48 | -5,98 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | type A | -13,31 | -10,81 |
|  |  |  |  | type B | -13,36 | -10,86 |
|  |  |  | DMRS 1+1 | type A | -14,06 | -11,56 |
|  |  |  |  | type B | -14,11 | -11,61 |
|  | [FDD 30kHz SCS 100MHz] |  | DMRS 1+0 | type A | -13,5 | -11 |
|  |  |  |  | type B | -13,49 | -10,99 |
|  |  |  | DMRS 1+1 | type A | -14,27 | -11,77 |
|  |  |  |  | type B | -14,25 | -11,75 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | type A | -8,01 | -5,51 |
|  |  |  |  | type B | -8,02 | -5,52 |
|  |  |  | DMRS 1+1 | type A | -8,51 | -6,01 |
|  |  |  |  | type B | -8,52 | -6,02 |
|  | [TDD 30kHz SCS 100MHz] |  | DMRS 1+0 | type A | -8,27 | -5,77 |
|  |  |  |  | type B | -8,29 | -5,79 |
|  |  |  | DMRS 1+1 | type A | -8,84 | -6,34 |
|  |  |  |  | type B | -8,84 | -6,34 |

Table 6: SNR gain at 70% throughput using 1T8R.

| JCE over |  |  |  |  | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | DMRS 1+0 | type A | -15,92 | -14,42 |
|  |  |  |  | type B | -15,89 | -14,39 |
|  |  |  | DMRS 1+1 | type A | -16,86 | -15,36 |
|  |  |  |  | type B | -16,84 | -15,34 |
|  | [FDD 15kHz SCS 50MHz] |  | DMRS 1+0 | type A | -15,66 | -14,16 |
|  |  |  |  | type B | -15,62 | -14,12 |
|  |  |  | DMRS 1+1 | type A | -15,91 | -14,41 |
|  |  |  |  | type B | -15,92 | -14,42 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | type A | -10,6 | -9,1 |
|  |  |  |  | PUSCH Mapping type B | -10,64 | -9,14 |
|  |  |  | DMRS 1+1 | type A | -11,44 | -9,94 |
|  |  |  |  | type B | -11,45 | -9,95 |
|  | [TDD 15kHz SCS 50MHz] |  | DMRS 1+0 | type A | -10,75 | -9,25 |
|  |  |  |  | type B | -10,74 | -9,24 |
|  |  |  | DMRS 1+1 | type A | -11,45 | -9,95 |
|  |  |  |  | type B | -11,43 | -9,93 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | type A | -16,15 | -14,65 |
|  |  |  |  | type B | -16,13 | -14,63 |
|  |  |  | DMRS 1+1 | type A | -17,06 | -15,56 |
|  |  |  |  | type B | -17,01 | -15,51 |
|  | [FDD 30kHz SCS 100MHz] |  | DMRS 1+0 | type A | -16,2 | -14,7 |
|  |  |  |  | type B | -16,22 | -14,72 |
|  |  |  | DMRS 1+1 | type A | -17,19 | -15,69 |
|  |  |  |  | type B | -17,16 | -15,66 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | type A | -11 | -9,5 |
|  |  |  |  | type B | -10,99 | -9,49 |
|  |  |  | DMRS 1+1 | type A | -11,68 | -10,18 |
|  |  |  |  | type B | -11,67 | -10,17 |
|  | [TDD 30kHz SCS 100MHz] |  | DMRS 1+0 | type A | -10,9 | -9,4 |
|  |  |  |  | type B | -10,93 | -9,43 |
|  |  |  | DMRS 1+1 | type A | -11,68 | -10,18 |
|  |  |  |  | type B | -11,65 | -10,15 |

Conclusion

In this contribution we have provided the results of our simulation campaign on both PUSCH TBoMS and PUSCH JCE for NR_Cov_enh Rel-17 BS demodulation performance requirements.s

References

- R4-2203030 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2207210 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2210666 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement