| 3GPP TR 38.718-03-01 V0.1.0 (2022-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	8

4.1	Introduction	8

4.2	TR Maintenance	8

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part	8

5.x	CA_nX-nY-nZ	8

5.x.1	Common for 1 band UL and 2 bands UL CA	8

5.x.1.1	Operating bands for CA	8

5.x.1.2	Channel bandwidths per operating band for CA	9

5.x.1.3	∆TIB,c and ∆RIB,c values	9

5.x.2	Specific for 2 bands UL CA	9

5.x.2.1	UE co-existence studies	9

5.x.2.2	REFSENS requirements	9

5.1	CA_n1-n41-n79	10

5.1.1	Common for 1 band UL and 2 bands UL CA	10

5.1.1.1	Operating bands for CA	10

5.1.1.2	Channel bandwidths per operating band for CA	10

5.1.1.3	∆TIB,c and ∆RIB,c values	11

5.1.2	Specific for 2 bands UL CA	11

5.1.2.1	UE co-existence studies	11

5.1.2.2	REFSENS requirements	11

5.2	CA_n41-n77-n79	12

5.2.1	Common for 1 band UL and 2 bands UL CA	12

5.2.1.1	Operating bands for CA	12

5.2.1.2	Channel bandwidths per operating band for CA	12

5.2.1.3	∆TIB,c and ∆RIB,c values	13

5.2.2	Specific for 2 bands UL CA	13

5.2.2.1	UE co-existence studies	13

5.2.2.2	REFSENS requirements	13

5.3	CA_n3-n28-n40	14

5.3.1	Common for 1 band UL and 2 bands UL CA	14

5.3.1.1	Operating bands for CA	14

5.3.1.2	Channel bandwidths per operating band for CA	14

5.3.1.3	∆TIB,c and ∆RIB,c values	14

5.3.2	Specific for 2 bands UL CA	14

5.3.2.1	UE co-existence studies	14

5.3.2.2	REFSENS requirements	14

5.4	  CA_n3-n8-n41	15

5.4.1	Common for 1 band UL and 2 bands UL CA	15

5.4.1.1	Operating bands for CA	15

5.4.1.2	Channel bandwidths per operating band for CA	15

5.4.1.3	∆TIB,c and ∆RIB,c values	15

5.4.2	Specific for 2 bands UL CA	15

5.4.2.1	UE co-existence studies	15

5.4.2.2	REFSENS requirements	15

5.5	CA_n1-n3-n26	16

5.5.1	Common for 1 band UL and 2 bands UL CA	16

5.5.1.1	Operating bands for CA	16

5.5.1.2	Channel bandwidths per operating band for CA	16

5.5.1.3	∆TIB,c and ∆RIB,c values	16

5.5.2	Specific for 2 bands UL CA	17

5.5.2.1	UE co-existence studies	17

5.5.2.2	REFSENS requirements	17

5.6	CA_n1-n26-n78	17

5.6.1	Common for 1 band UL and 2 bands UL CA	17

5.6.1.1	Operating bands for CA	17

5.6.1.2	Channel bandwidths per operating band for CA	17

5.6.1.3	∆TIB,c and ∆RIB,c values	18

5.6.2	Specific for 2 bands UL CA	18

5.6.2.1	UE co-existence studies	18

5.6.2.2	REFSENS requirements	18

5.7	CA_n3-n26-n78	19

5.7.1	Common for 1 band UL and 2 bands UL CA	19

5.7.1.1	Operating bands for CA	19

5.7.1.2	Channel bandwidths per operating band for CA	19

5.7.1.3	∆TIB,c and ∆RIB,c values	19

5.7.2	Specific for 2 bands UL CA	19

5.7.2.1	UE co-existence studies	19

5.7.2.2	REFSENS requirements	20

5.8	CA_n1-n7-n26	20

5.8.1	Common for 1 band UL and 2 bands UL CA	20

5.8.1.1	Operating bands for CA	20

5.8.1.2	Channel bandwidths per operating band for CA	20

5.8.1.3	∆TIB,c and ∆RIB,c values	20

5.8.2	Specific for 2 bands UL CA	21

5.8.2.1	UE co-existence studies	21

5.8.2.2	REFSENS requirements	21

5.9	CA_n3-n7-n26	21

5.9.1	Common for 1 band UL and 2 bands UL CA	21

5.9.1.1	Operating bands for CA	21

5.9.1.2	Channel bandwidths per operating band for CA	22

5.9.1.3	∆TIB,c and ∆RIB,c values	22

5.9.2	Specific for 2 bands UL CA	22

5.9.2.1	UE co-existence studies	22

5.9.2.2	REFSENS requirements	22

5.10	CA_n7-n26-n78	23

5.10.1	Common for 1 band UL and 2 bands UL CA	23

5.10.1.1	Operating bands for CA	23

5.10.1.2	Channel bandwidths per operating band for CA	23

5.10.1.3	∆TIB,c and ∆RIB,c values	23

5.10.2	Specific for 2 bands UL CA	24

5.10.2.1	UE co-existence studies	24

5.10.2.2	REFSENS requirements	24

6	Dual Connectivity: Specific Band Combination Part	24

6.x	DC_nX-nY-nZ	24

6.x.1	Configurations for DC_nX-nY-nZ	25

Annex <X> (informative): Change history	26

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC for 3 bands DL with up to 2 bands UL under Rel-18 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 3 bands DL with up to 2 bands UL for the Rel-18 band combinations.

This TR contains the RF requirements of band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

4	Background

4.1	Introduction

The present document is a technical report for NR inter-band CA and DC for 3 bands DL with up to 2 bands UL under Rel-18 time frame. The document covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination) including:

1:  Common issues for both 1 band UL and 2 bands UL NR CA, including ∆TIB and ∆RIB requirements.

2:  2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

It shall be noted that no new issue for inter-band NR DC combination, and the 2 bands UL NR CA specific issues shall be re-used.

4.2	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

Editor's note: It is not recommended to bring TP to TR for the following cases:

1.   NR CA configurations with additional BCS other than BCS0 (such as BCS1) if there is no additional technical issue.

2.   High order DL NR CA configurations, such as DL NR CA configuration CA_nXA-nYA-nZC

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part

5.x	CA_nX-nY-nZ

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| nX |  | – |  |  | – |  |  |
| nY |  | – |  |  | – |  |  |
| nZ |  | – |  |  | – |  |  |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nXA-nYA-nZA | -* or CA_nXA-nYA CA_nXA-nZA CA_nYA-nZA | nX | 5, 10, 15, 20 | 0 |
|  |  | nY | 5, 10, 15, 20, 25, 30 |  |
|  |  | nZ | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

Editor's note*: ‘-’ is for 1UL

Editor's note: The table format can be referred to Table 5.5A.3.2-1 in TS38.101-1

5.x.1.3	∆TIB,c and ∆RIB,c values

Editor’s note: for the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1, the table below is from the latest Rel.17 38.101-1, note that the table format might be changed in Rel.18.

For CA_nX-nY-nZ, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_nx-ny-nz |  |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_nx-ny-nz |  |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.x.2	Specific for 2 bands UL CA

Editor's note: Text will be added if 2 bands UL CA are supported, otherwise all the clauses shall be void.

5.x.2.1	UE co-existence studies

Editor's note:  No co-existence study tables are recommended to be included, since it was already incorporated in the corresponding 2DL fallback band combination TR.

5.x.2.2	REFSENS requirements

Editor's note: Text will be added on reference sensitivity exceptions if IMD issue due to dual uplink operation falling into DL of the third band are identified.

5.1	CA_n1-n41-n79

5.1.1	Common for 1 band UL and 2 bands UL CA

5.1.1.1	Operating bands for CA

Table 5.1.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

5.1.1.2	Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported bandwidths per CA band combination of band n1+n41+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n41A-n79A | CA_n1A-n41A CA_n1A-n79A CA_n41A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

5.1.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n41-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n41-n79 | 0.5 | 0.5 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.1.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n41-n79 |  | - |  | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.1.2	Specific for 2 bands UL CA

5.1.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n1-n41, CA_n1-n79 and n41-n79 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings.

- 2nd and 5th order IMD generated by dual uplink of Band n1 + Band n41 may fall into own Rx of Band n79.

- 2nd and 5th order IMD generated by dual uplink of Band n1 + Band n79 may fall into own Rx of Band n41.

- 2nd and 5th order IMD generated by dual uplink of Band n41 + Band n79 may fall into own Rx of Band n1.

5.1.2.2	REFSENS requirements

Table 5.1.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

Table 5.1.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n79 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | 19.0 | TDD | IMD21 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | TDD | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | 29.4 | TDD | IMD21 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n79 | 4690 | 40 | 216 | 4690 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 29.9 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.2	CA_n41-n77-n79

5.2.1	Common for 1 band UL and 2 bands UL CA

5.2.1.1	Operating bands for CA

Table 5.2.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

5.2.1.2	Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination of band n41+n77+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

5.2.1.3	∆TIB,c and ∆RIB,c values

For CA_n41-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.2.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n41-n77-n79 | 0.3 | 0.8 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.2.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n41-n77-n79 | 0.5 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.2.2	Specific for 2 bands UL CA

5.2.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n41-n77, CA_n41-n79 and n77-n79 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings. Note that there is no IMD impact on CA_n41-n77 to n79 Rx and that on CA_n41-n79 to n77 Rx since CA_n77-n79 is synchronous operation.

- 3rd, 4th and 5th order IMD generated by dual uplink of Band n77 + Band n79 may fall into own Rx of Band n41.

5.2.2.2	REFSENS requirements

Table 5.2.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

Table 5.2.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77-n79 | n77 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | 10.7 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 5.3	CA_n3-n28-n40

### 5.3.1	Common for 1 band UL and 2 bands UL CA

#### 5.3.1.1	Operating bands for CA

Table 5.3.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n3-n28-n403 | n3, n28, n40 |  |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |

#### 5.3.1.2	Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported bandwidths per CA band combination of band n3+n28+n40

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n40A | CA_n3A-n28A CA_n3A-n40A CA_n28A-n40A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n40 | 20, 40 |  |

#### 5.3.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n28-n40, the TIB,c and RIB,c values are given in the tables below, reuse the values of CA_3-28-40.

Table 5.3.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n28-n40 | 0.5 | 0.3 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.3.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n28-n40 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.3.2	Specific for 2 bands UL CA

#### 5.3.2.1	UE co-existence studies

For CA combinations with 3 bands DL and 2bands UL, only IMD issues due to dual Tx operation of two bands falling to the third band Rx need to be considered.

Based on the calculation, there is no additional IMD issue.

#### 5.3.2.2	REFSENS requirements

Based on co-existence studies on 5.3.2.1, no need to define exceptional REFSENS requirements.

  - 5.4			CA_n3-n8-n41

    - 5.4.1	Common for 1 band UL and 2 bands UL CA

      - 5.4.1.1	Operating bands for CA

Table 5.4.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

      - 5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination of band n3+n8+n41

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |

      - 5.4.1.3	∆TIB,c and ∆RIB,c values

For CA_n3A-n8A-n41A, the TIB,c and RIB,c values have already been included in the TS38.101-1.

    - 5.4.2	Specific for 2 bands UL CA

      - 5.4.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n8A, CA_n3A-n41A and CA_n8A-n41A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD 2 and IMD3 caused by n3+n8 may fall into the its own band n41 Rx

IMD 2 and IMD3 caused by n3+n41 may fall into the its own band n8 Rx

IMD2 caused by n8+n41 may fall into the its own band n3 Rx

      - 5.4.2.2	REFSENS requirements

For IMD2 caused by n8+n41, the IMD2 frequency range is calculated by the lower edge and upper edge of the bands, which is 1581MHz~1810MHz (i.e. f41-f8) , while the band n3 DL frequency range is 1805MHz~1880MHz, the overlapping frequency range is only 5MHz. However, considering the minimum channel bandwidths of band n8 and n41, which are 5MHz and 10MHz, respectively. The IMD2 frequency range is 1588.5 MHz ~ 1802.5 MHz, which means there are no test points of the IMD2 product fall into the own band n3 DL frequency range. Therefore, there is no need to define such MSD values.

For the other IMDs, the MSD requirement are defined in table 5.4.2.2-1:

Table 5.4.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n8-n41 | n3 | 1722.5 | 5 | 25 | 1817.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | 28.0 | FDD | IMD24 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 26.0 | FDD | IMD24 |
|  | n41 | 2516 | 10 | 50 | 2516 | N/A | FDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.5	CA_n1-n3-n26

5.5.1	Common for 1 band UL and 2 bands UL CA

5.5.1.1	Operating bands for CA

Table 5.5.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n3-n26 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |

5.5.1.2	Channel bandwidths per operating band for CA

Table 5.5.1.2-1: Supported bandwidths per CA band combination of band n1+n3+n26

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n26A | CA_n1A-n3A CA_n1A-n26A CA_n3A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n26 | 5, 10, 15, 20 |  |

5.5.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n26, the TIB,c and RIB,c values are resused from CA_1-3-26 and are given in the tables below.

Table 5.5.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n3-n26 | 0.3 | 0.3 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.5.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n3-n26 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.5.2	Specific for 2 bands UL CA

5.5.2.1	UE co-existence studies

UL n1-n26 does not affect DL n3.

UL n3-n26 does not affect DL n1.

UL n1-n3 does not affect DL n26.

5.5.2.2	REFSENS requirements

Based on the co-existence studies there are no need to defined MSD values.

5.6	CA_n1-n26-n78

5.6.1	Common for 1 band UL and 2 bands UL CA

5.6.1.1	Operating bands for CA

Table 5.6.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n26-n78 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.6.1.2	Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination of band n1+n26+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n26A-n78A | CA_n1A-n26A CA_n1A-n78A CA_n26A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.6.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n26-n78, the TIB,c and RIB,c values are resused from CA_n1-n5-n78 and are given in the tables below.

Table 5.6.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n26-n78 | 0.6 | 0.6 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.6.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n26-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.6.2	Specific for 2 bands UL CA

5.6.2.1	UE co-existence studies

UL n1-n26 gives IMD3 into DL n78.

UL n1-n78 gives IMD5 into DL n26.

UL n26-n78 gives IMD3 into DL n1.

5.6.2.2	REFSENS requirements

Based on the co-existence studies there are a need to define MSD values. MSD values from CA_n1-n5-n78 are reused.

Table 5.6.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n26-n78 | n1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n26 | 840 | 5 | 25 | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n26 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | TDD | IMD3 |

5.7	CA_n3-n26-n78

5.7.1	Common for 1 band UL and 2 bands UL CA

5.7.1.1	Operating bands for CA

Table 5.7.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n26-n78 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination of band n3+n26+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n26A-n78A | CA_n3A-n26A CA_n3A-n78A CA_n26A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.7.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n26-n78, the TIB,c and RIB,c values are resused from CA_n3-n5-n78 and are given in the tables below.

Table 5.7.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n26-n78 | 0.6 | 0.6 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.7.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n26-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.7.2	Specific for 2 bands UL CA

5.7.2.1	UE co-existence studies

UL n3-n26 gives IMD3 and IMD5 into DL n78.

UL n3-n78 gives no IMD into DL n26.

UL n26-n78 gives IMD3 into DL n3.

5.7.2.2	REFSENS requirements

Based on the co-existence studies there are a need to define MSD values. MSD values from CA_n3-n5-n78 are reused.

Table 5.7.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n26-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3408 | 10 | 50 | 3408 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3512 | 10 | 50 | 3512 | 4.5 | TDD | IMD5 |
|  | n3 | 1767 | 5 | 25 | 1862 | 15.7 | FDD | IMD3 |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |

5.8	CA_n1-n7-n26

5.8.1	Common for 1 band UL and 2 bands UL CA

5.8.1.1	Operating bands for CA

Table 5.8.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n7-n26 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |

5.8.1.2	Channel bandwidths per operating band for CA

Table 5.8.1.2-1: Supported bandwidths per CA band combination of band n1+n7+n26

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n7A-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n7B-n26A | CA_n1A-n26A CA_n1A-n7A CA_n7A-n26A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |

5.8.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n7-n26, the TIB,c and RIB,c values are resused from CA_1-3-26 and are given in the tables below.

Table 5.8.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n7-n26 | 0.5 | 0.6 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.8.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n7-n26 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.8.2	Specific for 2 bands UL CA

5.8.2.1	UE co-existence studies

UL n1-n7 gives IMD5 into DL n26.

UL n1-n26 does not affect DL n7.

UL n7-n26 does not affect DL n1.

5.8.2.2	REFSENS requirements

Based on the co-existence studies there are a need to define MSD values. MSD values from CA_1-7-26 are reused.

Table 5.8.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n7-n26 | n1 | 1965 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n26 | 830 | 5 | 50 | 875 | 3.5 | FDD | IMD5 |

5.9	CA_n3-n7-n26

5.9.1	Common for 1 band UL and 2 bands UL CA

5.9.1.1	Operating bands for CA

Table 5.9.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n7-n26 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |

5.9.1.2	Channel bandwidths per operating band for CA

Table 5.9.1.2-1: Supported bandwidths per CA band combination of band n3+n7+n26

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n7B-n26A | CA_n3A-n26A CA_n3A-n7A CA_n7A-n26A CA_n7B | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n26 | 5, 10, 15, 20 |  |

5.9.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n7-n26, the TIB,c and RIB,c values are resused from CA_1-3-26 and are given in the tables below.

Table 5.9.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n3-n7-n26 | 0.5 | 0.5 | 0.3 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.9.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n3-n7-n26 | - | - | - |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.9.2	Specific for 2 bands UL CA

5.9.2.1	UE co-existence studies

UL n3-n7 gives IMD3 into DL n26.

UL n3-n26 gives IMD2 and IMD3 into DL n7.

UL n7-n26 does not affect DL n3.

5.9.2.2	REFSENS requirements

Based on the co-existence studies there are a need to define MSD values. MSD values from CA_3-7-26 are reused.

Table 5.9.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n26 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n7 | 2560 | 10 | 50 | 2680 | N/A | FDD | N/A |
|  | n26 | 835 | 5 | 25 | 880 | 17.5 | FDD | IMD3 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n7 | 2505 | 10 | 50 | 2625 | 29.0 | FDD | IMD24 |
|  | n26 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |

5.10	CA_n7-n26-n78

5.10.1	Common for 1 band UL and 2 bands UL CA

5.10.1.1	Operating bands for CA

Table 5.10.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n7-n26-n78 | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.10.1.2	Channel bandwidths per operating band for CA

Table 5.10.1.2-1: Supported bandwidths per CA band combination of band n7+n26+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n26A-n78A | CA_n7A-n26A CA_n7A-n78A CA_n26A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.10.1.3	∆TIB,c and ∆RIB,c values

For CA_n7-n26-n78, the TIB,c and RIB,c values are resused from CA_n5-n7-n78 and are given in the tables below.

Table 5.10.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n7-n26-n78 | 0.6 | 0.6 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.10.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n7-n26-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.10.2	Specific for 2 bands UL CA

5.10.2.1	UE co-existence studies

UL n7-n26 gives IMD2 into DL n78.

UL n7-n78 gives IMD2 and IMD5 into DL n26.

UL n26-n78 gives IMD2 into DL n7.

5.10.2.2	REFSENS requirements

Based on the co-existence studies there are a need to define MSD values. MSD values from CA_n5-n7-n78 are reused.

Table 5.10.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n26-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 30.2 | FDD | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n26 | 830 | 5 | 25 | 875 | 3.3 | FDD | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | FDD | IMD2 |
|  | n26 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n26 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | TDD | IMD2 |

6	Dual Connectivity: Specific Band Combination Part

6.x	DC_nX-nY-nZ

Editor's note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means pure TP to TR to included NR DC configuration is not allowed.

6.x.1	Configurations for DC_nX-nY-nZ

Table 6.x.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA-nZ | DC_nXA-nYA DC_nXA-nZA DC_nYA-nZA |

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104e | R4-2213383 |  |  |  | TR skeleton | 0.0.0 |
| 2022-08 | RAN4 #104e | R4-2213619 |  |  |  | - R4-2212449, TP for TR 38.xxx-xx-xx: CA_n1-n41-n79, SoftBank Corp., LG Electronics - R4-2212456, TP for TR 38.xxx-xx-xx: CA_n41-n77-n79, SoftBank Corp., LG Electronics - R4-2212545, TP for TR 38.XXX-XX-XX: CA_n3A-n28A-n40A, Samsung, KDDI - R4-2212725, TP for TR38.xxx-xx-xx_3DL/2UL CA_n3A-n8A-n41A, ZTE Corporation - R4-2213107, TP for TR 38.818-03-01: CA_n1-n3-n26, Ericsson, Telstra - R4-2213109, TP for TR 38.818-03-01: CA_n1-n26-n78, Ericsson, Telstra - R4-2213111, TP for TR 38.818-03-01: CA_n3-n26-n78, Ericsson, Telstra - R4-2213108, TP for TR 38.818-03-01: CA_n1-n7-n26, Ericsson, Telstra - R4-2213110, TP for TR 38.818-03-01: CA_n3-n7-n26, Ericsson, Telstra - R4-2213112, TP for TR 38.818-03-01: CA_n7-n26-n78, Ericsson, Telstra | 0.1.0 |