3GPP TSG-RAN WG4 Meeting #104-e	R4-2213512

Electronic Meeting, 15 – 26 August, 2022

Title: 	Discussion on test cases for pre-MG

Source: 	Huawei, HiSilicon

Agenda item:	9.9.2.1

Document for:	Discussion

Introduction

RRM test cases for pre-MG are discussed in RAN4#103-e, and the outcomes are captured in WF [1]. Based on [1] the following issues are to be further discussed.

- CA or single carrier

- Types of trigger events

- Test with inter-frequency measurement

In this paper we will provide our views on above issues for pre-MG test cases.

Discussion

CA or single carrier

### Issue 2-1-3 CA or single carrier Agreements: - Define the test cases for single carrier cases firstly. The final agreements on TC list in issue 2-2-5 can be updated accordingly. - Whether the other additional test cases for CA can be FFS in the next meeting.

In our view, there are indeed some additional work that the UE should do for CA either for rule based or RRC based activation and deactivation, but they are functional. For example, for RRC based activation and deactivation, UE may need to combine the status indication from activated CCs with that from deactivated CCs. We do not see any performance issue in this bit combining operation. On the other hand, the RRM test should focus on the performance issues. Therefore, we do not see strong need to define performance tests for CA scenario.

Proposal 1: Do not define additional test cases for CA.

Types of trigger events

### Issue 2-1-6 Which types of trigger events Agreements: - Define the test cases for BWP switching trigger event firstly. - Whether the other additional test cases for MO addition/remove can be FFS in the next meeting.

For rule based activation and deactivation, other trigger event than BWP switch is defined, e.g. MO addition/remove. In our view, however, test with RRC based trigger event like addition/removal of MO is not much different from existing test with legacy MG (i.e. configuring the MO and MG in the same RRC message), and we do not see strong need to test it.

It is noted that BWP switching is the most type use case for pre-MG and also the most challenging scenario for pre-MG activation and deactivation.

Proposal 2: Do not define additional test cases for other trigger event like MO addition/remove.

Test with inter-frequency measurement

### Issue 2-2-3 Test cases for the reporting delay of inter-frequency measurement with gap Agreements: - Whether the other additional test cases for the inter-frequency measurement report event can be FFS in the next meeting.

In our view, the issue is related to previous issue in section 2. 2. If RAN4 agrees to test other trigger events for pre-MG activation and deactivation like MO addition/remove, it is meaningful to have the test case for inter-frequency measurement, i.e. an inter-frequency MO is added so that the pre-MG status is changed. However, as discussed in section 2.2, we do not see the need to define test cases for other trigger events than BWP switch, so we suggest to not define additional test cases for inter-frequency measurement.

Proposal 3: do not define additional test cases for the inter-frequency measurement.

Conclusions

In this paper we provided our views on issues for pre-MG test cases.

Proposal 1: Do not define additional test cases for CA.

Proposal 2: Do not define additional test cases for other trigger event like MO addition/remove.

Proposal 3: do not define additional test cases for the inter-frequency measurement.

Reference

- R4-2210586, WF on R17 NR MG enhancements – Pre-configured MG, Intel