3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213792

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Simulation results for RedCap CQI requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.18.5.2.1

Document for:	Discussion

Background

As per [1], in this paper we provide our simulation results for RedCap CQI requirements

Simulation results for static channel

The simulation assumptions are captured in appendix.

The simulation results for case with 2T2R, rank2 are shown in Table 2-1.

Table 2-1: Simulation results for case with 2T2R rank 2.

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100% | 0 | 0 | 100% |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 87.5% |
| 12 | 10 | 100% | 0 | 0 | 88.9% |
| 14 | 12 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 0 | 24.0% |
| 18 | 13 | 100% | 0 | 0 | 53.8% |

Simulation results under fading channel

The simulation results for FR1 FDD 1RX are captured in Table 2-2:

Table 2-1: Simulation results for FR1 FDD 1RX

| SNR | TP ratio of follow CQI/ Median CQI | Median CQI | Prob of(Median CQI-1<CQI< Median CQI+1) | BLER with follow CQI |
| --- | --- | --- | --- | --- |
| 0 | 1.40 | 4 | 50.2% | 0.23 |
| 2 | 1.25 | 5 | 49.2% | 0.76 |
| 4 | 1.26 | 6 | 46.3% | 0.24 |
| 6 | 1.24 | 7 | 44.0% | 0.23 |
| 8 | 1.19 | 8 | 39.6% | 0.20 |
| 10 | 1.27 | 9 | 42.7% | 0.19 |
| 12 | 1.35 | 10 | 40.2% | 0.19 |
| 14 | 1.32 | 11 | 43.8% | 0.18 |
| 16 | 1.33 | 12 | 51.0% | 0.17 |
| 18 | 1.39 | 13 | 61.5% | 0.14 |

The simulation results for FR1 FDD 2RX are captured in Table 2-3:

Table 2-2: Simulation results for FR1 FDD 2RX

| SNR | TP ratio of follow CQI/ Median CQI | Median CQI | Prob of(Median CQI-1<CQI< Median CQI+1) | BLER with follow CQI |
| --- | --- | --- | --- | --- |
| 0 | 1.05 | 5 | 54.6 | 0.30 |
| 2 | 1.08 | 6 | 52.1% | 0.28 |
| 4 | 1.12 | 8 | 43.1% | 0.31 |
| 6 | 1.12 | 9 | 47.3% | 0.29 |
| 8 | 1.14 | 9 | 52.7% | 0.23 |
| 10 | 1.39 | 11 | 48.1% | 0.21 |
| 12 | 1.45 | 12 | 55.2% | 0.19 |
| 14 | 1.54 | 13 | 67.1% | 0.15 |
| 16 | 1.29 | 13 | 71.5% | 0.10 |
| 18 | 1.29 | 14 | 64.2% | 0.07 |

The simulation results for FR1 TDD 1RX are captured in Table 2-4:

Table 2-3: Simulation results for FR1 TDD 1RX

| SNR | TP ratio of follow CQI/ Median CQI | Median CQI | Prob of(Median CQI-1<CQI< Median CQI+1) | BLER with follow CQI |
| --- | --- | --- | --- | --- |
| 0 | 1.27 | 4 | 70% | 0.17 |
| 2 | 1.24 | 5 | 72.1% | 0.19 |
| 4 | 1.30 | 6 | 67.5% | 0.17 |
| 6 | 1.32 | 6 | 45% | 0.18 |
| 8 | 1.14 | 8 | 62.5% | 0.19 |
| 10 | 1.18 | 9 | 58.8% | 0.18 |
| 12 | 1.18 | 10 | 56.7% | 0.21 |
| 14 | 1.18 | 11 | 70.8% | 0.20 |
| 16 | 1.23 | 12 | 68.8% | 0.17 |
| 18 | 1.28 | 13 | 79.2% | 0.16 |

The simulation results for FR1 TDD 2RX are captured in Table 2-4:

Table 2-4: Simulation results for FR1 TDD 2RX

| SNR | TP ratio of follow CQI/ Median CQI | Median CQI | Prob (Median CQI-1<CQI< Median CQI+1) | BLER with follow CQI |
| --- | --- | --- | --- | --- |
| 0 | 1.02 | 5 | 70% | 0.21 |
| 2 | 1.05 | 6 | 67.9% | 0.20 |
| 4 | 1.11 | 8 | 57.5% | 0.28 |
| 6 | 1.21 | 9 | 67.5% | 0.25 |
| 8 | 1.15 | 9 | 64.2% | 0.24 |
| 10 | 1.22 | 11 | 65.4% | 0.27 |
| 12 | 1.32 | 12 | 74.2% | 0.23 |
| 14 | 1.08 | 12 | 86.3% | 0.14 |
| 16 | 1.08 | 13 | 80.8% | 0.11 |
| 18 | 1.27 | 14 | 95% | 0.09 |

Conclusion

In this paper, we provide our initial simulation results for CQI requirements

Reference

[1]   WF on RedCap demodulation and CQI reporting requirements. Ericsson

Appendix

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 ULA high | Reuse |

CQI reporting requirements in fading condition

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high | New |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 ULA high | New |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 ULA high | Reuse |