3GPP TSG-RAN4 Meeting #104-e		R4-2214059

# Electronic Meeting, 15-26 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2564 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Satellite access band grouping for RRM requirements in TS 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The frequency bands for satellite access are missing for RRM requirements are defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following FR1 bands are defined for the satellite access in tabel 7.3.2-1 of TS 38.101-5: - n255 and n256 The above bands are defined and introduced in new FR1 band group in new section 3.5.2A and Table 3.5.2A-1 in TS 38.133 for the applicability of RRM requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements for satellite access in FR1 will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.5.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CG-SDT	Configured Grant Small Data Transmisison

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

HARQ	Hybrid Automatic Repeat Request

HO	Handover

GAP	Refers to any of the measurement gap pattern, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-DC	NR-NR Dual Connectivity

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurenent Gap

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRM	Radio Resource Management

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SL-RSSI	Sidelink Received Signal Strength Indicator

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TRP	Transmission-Reception Point

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

VIL	Visible Interruption Length

VIRP				Visible Interruption Repetition Period

----------------------NEXT CHANGE----------------------------

### 3.5.2A	NR operating bands for satellite access in FR1

NR frequency bands grouping for satellite access in FR1 is specified in Table 3.5.2A-1.

Table 3.5.2A-1: NR frequency band groups for satellite access in FR1

| Group | NR FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NR_FDD_SAB_FR1_A | n255, n256 |
| B | NR_FDD_SAB_FR1_B |  |
| C | NR_FDD_SAB_FR1_C |  |
| D | NR_FDD_SAB_FR1_D |  |
| E | NR_FDD_SAB_FR1_E |  |
| F | NR_FDD_SAB_FR1_F |  |
| G | NR_FDD_SAB_FR1_G |  |
| H | NR_FDD_SAB_FR1_H |  |
| I | NR_FDD_SAB_FR1_I |  |
| J | NR_FDD_SAB_FR1_J |  |

----------------------END OF CHANGE----------------------------