3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214190

Electronic Meeting, Aug 15th – 26th, 2022

Agenda item:			9.24.7

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [104-e][330] NB-IOT_MTC_Demod

Document for:	Information

Introduction

Based on the discussion during RAN4#103-e:

- R4-2210534 Email discussion summary for [103-e][331] NB-IOT_MTC_Demod, Moderator(Huawei)

- R4-2210674 WF on Rel-17 NB-IOT and eMTC performance requirements, Huawei.

Great progress was made, the only left open issues are simulation results alignment and CR drafting.

During this meeting, we will continue the discussion as per the following consideration:

1st round discussion:

- Collect companies’ view on some open issues raised for this meeting

- Simulation results alignment

- Draft CR review

2nd round discussion:

- Try to finalize all open issues

- Complete the simulation results alignment

- Endorse the draft CR

Topic #1: UE requirements for Rel-17 NB-IOT

Companies’ contributions summary

| T-doc number | Company | Title | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2212205 | Qualcomm Incorporated | DraftCR – Test cases for NB-IoT DL 16-QAM demodulation performance | Draft CR |
| R4-2212896 | Ericsson | Test setup of NB-IoT channel quality reporting requirements | Proposal 1: For NB-IoT CQI reporting test in TDD, set the UL-DL configuration and special subframe configuration parameters same as the NPDSCH demodulation requirements: • Uplink-downlink configuration: 4 (DSUUD DDDDD) • Special subframe configuration: 4 Proposal 2: For NB-IoT CQI reporting test in TDD, configure the following NPDCCH/NPDSCH parameters: • Rmax (npdcch-NumRepetitions): 4 • G (nPDCCH-startSF-USS): 2 • Repetition numbers of NPDCCH and NPUSCH: 1 • ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE • CQI reporting period: 80ms |
| R4-2212897 | Ericsson | draft CR: channel quality reporting requirements for NB-IoT | Draft CR |
| R4-2213798 | Huawei, HiSilicon | Discussion and simulation results on NPDSCH requirements with 16QAM | Proposal 1: Use uplink downlink configuration 4 and special subframe configuration 4 for TDD test. Proposal 2: Use Idelay=1, G=8, α_offset=0 and Rmax=1 for scheduling pattern and PDCCH search space configuration |
| R4-2213799 | Huawei, HiSilicon | Simulation results on CQI requirements for Rel-17 NB-IOT | Simulation results for CQI and proposal: Proposal 1: Use uplink downlink configuration 4 and special subframe configuration 4 for TDD CQI test. Proposal 2: RAN4 should design the TDD CQI pattern satisfying that continuous 4 scheduled NPDSCH are not overlapped with special slot. Proposal 3: FDD CQI pattern introduced in [2] should be updated based on the agreement that 4 sub-frames are used for CQI calculation. |
| R4-2213802 | Huawei, HiSilicon | Summary of Rel-17 NB-IOT and eMTC simulation results | Summary of simulation results |

Open issues summary

Sub-topic 1-1 NPDSCH demodulation test

Background information:

As per the approved WF R4-2207200, the following agreements are reached in last RAN4#102-e meeting:

Duplex mode for NPDSCH performance with 16QAM - Introduce NPDSCH demodulation test with 16QAM for HD-FDD and TDD Whether the Rel-17 NB-IOT requirements are applicable for TDD: - Yes

Issue 1-1-1: TDD pattern

- Proposals

  - Option 1: Uplink-downlink configuration 4 and special subframe configuration 4 (Huawei)

  - Other options

- Recommended WF

  - Option 1 is agreeable?

Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements

- Proposals

  - Option 1: Use Idelay=1, G=8, αoffset=0, Rmax=1 (Huawei)

  - Other options

- Recommended WF

  - TBA

Sub-topic 1-2 CQI test

Background information:

Issue 1-2-1: TDD pattern

As per the approved WF R4-2207200, the following agreements are reached in last RAN4#102-e meeting:

CQI reporting test for TDD RAN4 defines the CQI reporting test for TDD

- Proposals

  - Option 1: Uplink-downlink configuration 4 and special subframe configuration 4 (Ericsson, Huawei)

  - Other options

- Recommended WF

  - Same TDD pattern as NPDSCH demodulation requirements, i.e. Option 1?

Issue 1-2-2: Scheduling pattern for TDD CQI reporting test

As per the approved WF R4-2207200, the following agreements are reached in last RAN4#102-e meeting:

Scheduling pattern for HD-FDD CQI reporting test - Rmax (npdcch-NumRepetitions): 4 - G (nPDCCH-startSF-USS): 2 - Repetition numbers of NPDCCH and NPUSCH: 1 - ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE - CQI reporting period: 40ms - CQI delay: 14ms

- Proposals

  - Option 1:  (Ericsson)

- Rmax (npdcch-NumRepetitions): 4

- G (nPDCCH-startSF-USS): 2

- Repetition numbers of NPDCCH and NPUSCH: 1

- ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

- CQI reporting period: 80ms

  - Option 2:

-	Rmax (npdcch-NumRepetitions): 4

-	G (nPDCCH-startSF-USS): 2

-	Repetition numbers of NPDCCH and NPUSCH: 1

-	ISF=4 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE

-	CQI reporting period: 80ms

- Recommended WF

  - Moderator’s observation: the difference between Option 1 and Option 2 is the ISF value.

Issue 1-2-3: Scheduling pattern for FDD CQI reporting test

As per the approved WF R4-2207200, the following agreements are reached in last RAN4#102-e meeting:

Scheduling pattern for HD-FDD CQI reporting test - Rmax (npdcch-NumRepetitions): 4 - G (nPDCCH-startSF-USS): 2 - Repetition numbers of NPDCCH and NPUSCH: 1 - ISF=1 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE - CQI reporting period: 40ms - CQI delay: 14ms

- Proposals

  - Option 1:  (Huawei)

- ISF=4 for NPDSCH to transmit MAC CE

  - Other options

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

Sub topic 1-1: NPDSCH demodulation test

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: TDD pattern Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements |
| Ericsson | Issue 1-1-1: TDD pattern We agree with Option 1. Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements We prefer to keep the current configuration: G=4 and Rmax=8 for NB-IoT TDD. We need explanation from the proponent of option 1 why the existing configuration cannot be used. |
| Huawei | Issue 1-1-1: TDD pattern We agree with Option 1. Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements Option 1 and current configuration have same USS period. Our preference is to reduce the NPDCCH search time since NPDCCH repetition is 1, so we prefer to configure Rmax=1 and G=8 |
| Ericsson3 | Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements To Huawei, thank for your explanation. Some of the exiting TDD NPDSCH requirements set very low SNR e.g., -10.2dB. Then RAN4 set Rmax=8 to avoid the case UE cannot decode NPDCCH in Rel-15. For this case, on the other hand, the required SNR will be 20dB. We think UE can decode NPDCCH without errors, so we are fine to set Rmax=1/G=8. In this case, is it possible to set Noc=-99dBm/15kHz only during the test? |
| Qualcomm | Issue 1-1-1: TDD pattern Option 1. Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements Prefer to keep the same parameter settings for G and Rmax as in the baseline test. We don’t see a need to change them. For Idelay, two values are specified for the first and second HARQ processes in the proposed new test case. |

Sub topic 1-2: CQI test

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1: TDD pattern Issue 1-2-2: Scheduling pattern for TDD CQI reporting test Issue 1-2-3: Scheduling pattern for FDD CQI reporting test |
| Ericsson | Issue 1-2-1: TDD pattern We support Option 1. Issue 1-2-2: Scheduling pattern for TDD CQI reporting test Option 1: ISF=1 If we agree with the UL-DL subframe configuration 4 in Issue 1-2-1, 7 subframes are allocated for DL subframe. According to the scheduling pattern shown in R4-2212896 (see below), UE can stay on DL carrier and monitor at least 4 consecutive DL subframes. We don’t need to use ISF=4 for MAC CE transmission. Since the configuration of ISF=1 is used for MAC CE transmission, we think UE can measure NRS after MAC CE reception during the DL subframes. Issue 1-2-3: Scheduling pattern for FDD CQI reporting test Option 2: ISF=1 Same comment as TDD. According to the scheduling pattern shown in R4-2212896 (see below), UE can stay on DL carrier and monitor at least 4 consecutive DL subframes. We don’t need to use ISF=4 for MAC CE transmission. |
| Huawei | Issue 1-1-1: TDD pattern We support option 1 Issue 1-2-2: Scheduling pattern for TDD CQI reporting test @Ericsson: Thanks for explanation, we still have concern that how can CQI accuracy requirements be met with 90% by using only one slot NRS samples? According our simulation results, at least 4 slots NRS samples are used for CQI calculation to meet 90% metric. Even UE can stay on DL carrier and monitor at least 4 consecutive DL subframes, there is only one slot when NRS is transmitted. Issue 1-2-3: Scheduling pattern for FDD CQI reporting test Similar concern as issue 1-2-2 |
| Ericsson3 | Issue 1-2-2: Scheduling pattern for TDD CQI reporting test Issue 1-2-3: Scheduling pattern for FDD CQI reporting test To Huawei, our intention was to configure ‘NB-IoT downlink subframes’ to all the DL subframes so that UE may assume NRS are transmitted before/after NDPSCH transmission, according to TS36.211 10.2.6. However we found the existing NB-IoT UE demodulation requirements do not configure ‘NB-IoT downlink subframes’. In this case we tend to agree with Huawei it is safer to schedule NPDSCH with 4 subframes. According to our scheduling pattern (two figures above), it is possible to configure ISF=4, with keeping the same CQI reporting period. In summary, we are ok to set ISF=4 for both HD-FDD/TDD CQI reporting tests. |
| Qualcomm | Issue 1-2-1: TDD pattern Support Option 1. Issue 1-2-2: Scheduling pattern for TDD CQI reporting test Option 2 (ISF = 4) is fine. Issue 1-2-3: Scheduling pattern for FDD CQI reporting test Option 1 (ISF = 4) is fine. |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212205 (Qualcomm) for NPDSCH performance test | Huawei: 1 For the coversheet, the part “Other specs affected ” is missed 2 For table 8.12.1.1.5-1: NPDCCH repetition number should be 1 according to previous agreements. 3: For table 8.12.1.1.5-1: Rmax should be 4, G should be 2. 4: For table 8.12.1.1.1-2: We propose to configure Idelay to 1 for first HARQ processes and to 2 for the second HARQ processes to follow the existing configurations. For FRC table, target coding rate seems to be 2/3 not 0.78 Similar views with Table 8.12.1.2-2 |
|  | Qualcomm: @Huawei, thanks for your comments. We’ll check and revise the CR as needed. |
| R4-2212897 (Ericsson) for CQI reporting test | Huawei: Why configure Idelay=0? |
|  | Ericsson3: To Huawei. Thank you for your checking. It should be 1. On top of that, according to Issue 1-2-2/1-2-3, we would like to add the following configurations: downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. |
|  | Qualcomm: OCNG Pattern NB.OP.1 TDD does not seem to be defined in 36.101. ?? |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-Topic# | Status summary |
| --- | --- |
| #1-1 NPDSCH demodulation test | Issue 1-1-1: TDD pattern Agreements: Uplink-downlink configuration 4 and special subframe configuration 4 Issue 1-1-2: Scheduling pattern for TDD NPDSCH demodulation requirements Moderator: To move forward, moderator would like to suggest to reuse the existing G=4 and Rmax= 8 for TDD NPDSCH demodulation requirements. Idelay for the first and second HARQ processes can be further discussed in the revised CR. Agreements: Use G=4, Rmax=8 |
| #1-2 CQI test | Issue 1-2-1: TDD pattern Agreements: Use Uplink-downlink configuration 4 and special subframe configuration 4 Issue 1-2-2: Scheduling pattern for TDD CQI reporting test Agreements: - Rmax (npdcch-NumRepetitions): 4 - G (nPDCCH-startSF-USS): 2 - Repetition numbers of NPDCCH and NPUSCH: 1 - ISF=4 for NPDSCH and IRU=NPUSCH format 1 to transmit MAC CE - CQI reporting period: 80ms Issue 1-2-3: Scheduling pattern for FDD CQI reporting test Agreements: Configure ISF=4 for PDSCH with MAC CE |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212205 (Qualcomm) for NPDSCH performance test | To be revised |
| R4-2212897 (Ericsson) for CQI reporting test | To be revised |

Discussion on 2nd round (if applicable)

Topic #2: BS requirements for Rel-17 NB-IOT

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Title | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2213075 | Nokia, Nokia Shanghai Bell | BS demodulation performance for NB-IoT 16QAM | Simulation results and proposals: Proposal 1: Include simulation result of 5.7 dB to final simulation summary for NPUSCH format 1 16QAM alignment results. Proposal 2: Use impairment margin of 2.5 dB for NPUSCH format 1 16QAM requirements. |
| R4-2213076 | Nokia, Nokia Shanghai Bell | Introduction of NPUSCH format 1 16QAM test requirements | Draft CR for TS 36.141 |
| R4-2213662 | Samsung | Simulation results for Rel-17 NB-IoT | Simulation results and proposal: Proposal 1: RAN4 applies the following FRC for UL 16 QAM requirement. Reference channel; A16-x. Subcarrier spacing (kHz); 15. Number of allocated subcarriers; 12. Diversity; No. Modulation; 16QAM. ITBS / IRU; 15 / 0. Payload size (bits); 280. Allocated resource unit; 1. Code rate (target); 1/3. Code rate (effective); 0.528. Transport block CRC (bits); 24. Code block CRC size (bits); 0. Number of code blocks - C; 1. Total number of bits per resource unit; 576. Total symbols per resource unit; 144. Channel estimation length (ms) Note 1; 2. |
| R4-2213663 | Samsung | Draft CR on NPUSCH format1 demodulation requirement for TS 36.104 | Draft CR for TS 36.104 |
| R4-2213800 | Huawei, HiSilicon | Simulation results on NPUSCH requirements with 16QAM | Simulation results for NPUSH format 1 |

Open issues summary

Sub-topic 2-1 Test setup

Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements

- Proposals

  - Option 1: 2.5dB (Nokia)

  - Option 2: Depends on companies’ implementation

- Recommended WF

  - Moderator: usually company add the impairment margin depends on their implementation.

Issue 2-1-2: FRC

- Proposals

  - Option 1: (Samsung)

| Reference channel | A16-x |
| --- | --- |
| Subcarrier spacing (kHz) | 15 |
| Number of allocated subcarriers | 12 |
| Diversity | No |
| Modulation | 16QAM |
| ITBS / IRU | 15 / 0 |
| Payload size (bits) | 280 |
| Allocated resource unit | 1 |
| Code rate (target) | 1/3 |
| Code rate (effective) | 0.528 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Total number of bits per resource unit | 576 |
| Total symbols per resource unit | 144 |
| Channel estimation length (ms) Note 1 | 2 |

  - Other options

- Recommended WF

  - Companies are encouraged to check if the FRC in Option 1 is agreeable?

Companies views’ collection for 1st round

Open issues

Sub topic 2-1: Test setup

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements Issue 2-1-2: FRC |
| Ericsson | Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements We have the same understanding as the moderator. It is up to the source company how much margin to be added. Issue 2-1-2: FRC - Target code rate should be 0.54 (instead of 1/3). - Since NPUSCH format 1 uses IRU=1 with 12 tones without repetitions, the required number of subframes is 1, we can set channel estimation length to 1 ms, that is, we don’t see any reasons to spend 2 ms for channel estimation. |
| Samsung | Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements Same understanding as the moderator, the margin pending on source company Issue 2-1-2: FRC We are ok to change the channel estimation length to 1ms. For target code rate, we need to double check, 1/3 is the coding rate for TBCC |
| Ericsson2 | Issue 2-1-2: FRC To Samsung: Regarding the target code rate, in our understanding, we set the code rate in TBS table in TS36.213 Table 16.4.1.5.1-1. For example, A15-1, A15-2, and A16-5, set target code rate to 2/3 although it uses TBCC. But we can wait for other companies inputs. Another option is we remove this row because other FRC tables do not specify the target code rate. |
| Huawei | Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements Same views with Ericsson and Samsung Issue 2-1-2: FRC For target rate, we propose to set it to 1/2. For channel estimation length, we are OK to change it to 1ms. |
| Nokia | Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements We are fine with moderator’s proposal. We updated the impairment margin for our simulation results in the simulation summary (Excel sheet). Issue 2-1-2: FRC For target code rate, we agree with Huawei to set it to 1/2. The effective code rate then is 0.53. For channel estimation we are fine to set it to 1 ms. These numbers are used for new FRC A16-7 in draft CR36.141 in R4-2213076. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213663 (Samsung) for NPUSCH format 1 in TS 36.104 | Ericsson: We prefer to have different table for 16QAM since this is new feature from Rel-17, as same as R4-2213076. |
|  | Samsung: we are fine to have a different table, |
|  | Huawei: We share the same views with Ericsson |
|  | Nokia: We agree with Ericsson and Huawei, the table needs to be changed. |
| R4-2213076 (Nokia) for NPUSCH format 1 in TS 36.141 | Huawei: Do we need to change the entire table A.16.1.1-1 to revision mode? We just add one new row to legacy table |
|  | Nokia: Thanks for your comments. We can update the draft CR accordingly. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-Topic# | Status summary |
| --- | --- |
|  | Issue 2-1-1: Impairment margin for NPUSCH format 1 performance requirements Agreements: No need to mandate the impairment margin and leave it to source companies. Issue 2-1-2: FRC Agreements: - Set the channel estimation length to 1ms. - Use following FRC for NPUSCH format 1 except the target code rate Reference channel; A16-x. Subcarrier spacing (kHz); 15. Number of allocated subcarriers; 12. Diversity; No. Modulation; 16QAM. ITBS / IRU; 15 / 0. Payload size (bits); 280. Allocated resource unit; 1. Code rate (target); TBD. Code rate (effective); 0.528. Transport block CRC (bits); 24. Code block CRC size (bits); 0. Number of code blocks - C; 1. Total number of bits per resource unit; 576. Total symbols per resource unit; 144. Channel estimation length (ms); 1. Target code rate: - Option 1: 0.54 (Ericsson) - Option 2: 1/2 (Huawei, Nokia) - Option 4: Remove the target code rate setting in FRC table (Ericsson) Recommendation in 2nd round: Further discuss in the 2nd round. Candidates for target code rate: - Option 1: 0.54 (Ericsson) - Option 2: 1/2 (Huawei, Nokia) - Option 4: Remove the target code rate setting in FRC table (Ericsson) |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2213663 (Samsung) for NPUSCH format 1 in TS 36.104 | To be revised |
| R4-2213076 (Nokia) for NPUSCH format 1 in TS 36.141 | To be revised |

Discussion on 2nd round (if applicable)

Topic#3: Demodulation requirements for Rel-17 eMTC

Companies’ contributions summary

| T-doc number | Company | Title | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2213801 | Huawei, HiSilicon | CR: Introduction of eMTC PDSCH requirements with 14 HARQ processes | Draft CR |

Open issues summary

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213801 (Huawei, HiSilicon) for PDSCH with 14 HARQ processes | Ericsson: 1. Table 8.11.1.1-1: Put also parameter name ce-HARQ-AckBundling for HARQ bundling. See below. 2. Table 8.11.1.1.3.1-1a: Note 5 should also mention ‘Test 7 is applicable for UE in half-duplex FDD operation’, see blow, because 8.11.1.1 is applicable for both full-duplex FDD and half-duplex FDD operations. 3. Table A.3.3.2.1-4: Since Note 3 is applicable for R.79 FDD and R.104 FDD, we propose to the new text is put as ‘Note 6’ and it is put only in R.79 FDD. See below. |
|  | Huawei: Thanks for Ericsson’s carefully checking, we agree with all the suggestions. |
|  | Qualcomm: Cover sheet: Test specification should be TS 36.521-1 |

Summary for 1st round

Open issues

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2213801 (Huawei, HiSilicon) for PDSCH with 14 HARQ processes | To be revised |

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | Way forward for performance requirements of Rel-17 NB-IOT and eMTC | Huawei, HiSilicon | Capture the agreements reached in this meeting for those open issues |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2212205 |  | DraftCR – Test cases for NB-IoT DL 16-QAM demodulation performance | Qualcomm Incorporated | Revised |  |
| R4-2212896 |  | Test setup of NB-IoT channel quality reporting requirements | Ericsson | Noted |  |
| R4-2212897 |  | draft CR: channel quality reporting requirements for NB-IoT | Ericsson | Revised |  |
| R4-2213798 |  | Discussion and simulation results on NPDSCH requirements with 16QAM | Huawei, HiSilicon | Noted |  |
| R4-2213799 |  | Simulation results on CQI requirements for Rel-17 NB-IOT | Huawei, HiSilicon | Noted |  |
| R4-2213802 |  | Summary of Rel-17 NB-IOT and eMTC simulation results | Huawei, HiSilicon | Noted | To be uploaded at the end of the meeting |
| R4-2213075 |  | BS demodulation performance for NB-IoT 16QAM | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213076 |  | Introduction of NPUSCH format 1 16QAM test requirements | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2213662 |  | Simulation results for Rel-17 NB-IoT | Samsung | Noted |  |
| R4-2213663 |  | Draft CR on NPUSCH format1 demodulation requirement for TS 36.104 | Samsung | Revised |  |
| R4-2213800 |  | Simulation results on NPUSCH requirements with 16QAM | Huawei, HiSilicon | Noted |  |
| R4-2213801 |  | CR: Introduction of eMTC PDSCH requirements with 14 HARQ processes | Huawei, HiSilicon | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

- Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |
| Huawei, HiSilicon | Ke Li | like54@hisilicon.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)