3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214270

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.14.6

Source:	Moderator (Huawei)

Title:	Email discussion summary for [104-e][218] NR_ext_to_71GHz_RRM_1

Document for:	Information

Introduction

This email discussion document focuses on maintenance of RRM requirements for NR operation extension to 71GHz including general requirements, timing requirements and impact of LBT on RRM requirements.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

  - General requirements

  - Timing requirements

  - LBT impact on RRM requirements

  - CR/Draft CR

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| Ericsson | Magnus Larsson | magnus.k.larsson@ericsson.com |
| Nokia | Erika Almeida | Erika.almeida@nokia.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |
| CATT | Lingyu Gao | gaolingyu@catt.cn |
| LG Electronics | Jin-Yup Hwang | jinyup.hwang@lge.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: General requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211689 | CATT | Observation 1: The main difference between the two options is how the frame boundary alignment tolerance of PDSCH symbols is calculated when deriveSSB-IndexFromCell is disabled. Proposal 1: It is reasonable and more convenient to derive the frame boundary alignment tolerance of PDSCH symbols in option 1 based on the assumed propagation delay difference. Observation 2: When deriveSSB-IndexFromCell is enabled, if the PDSCH symbols need to be modified, then the calculation of SSB symbols also needs to be modified for 960kHz SCS, and the L3 measurement scheduling restriction requirements for FR2-2 also need further corrections. Observation 3: It is reasonable to use the SSB symbol offset between maximal and minimal of 3 least significant bits of SSB index as reference to calculation. Proposal 2: As long as the difference between the SSB index numbers of the reference cell and the target cell is less than 7, the time difference between the reference cell and the target cell can be as many as dozens of symbols instead of 7 960kHz PDSCH symbols. Observation 4: The maximum frame boundary alignment tolerance of UE seems to satisfy the condition that the difference between the SSB index numbers of the reference cell and the target cell is less than 7, so it can be assumed that the UE can derive the SSB index of the neighbor cell according to the timing of the serving cell. |
| R4-2211878 | Apple | Proposal 1: RAN4 to decide the scaling factor for RRC_IDLE, with the proposed values in the above table as a starting point for consideration. Proposal 2: RAN4 to decide the scaling factor N1 for RRC_IDLE state for PC1 and PC2. |
| R4-2211879 | Apple | Proposal 1: RAN4 to send an LS to RAN1 seeking clarification on both case 1 and case 2. Proposal 2: RAN4 defers the implementation of the recommended TP until RAN4 receives clarification from RAN1 and concludes the discussion. |
| R4-2212214 | LG Electronics Inc. | LS on Rx beam sweeping factor for RRM measurements |
| R4-2212215 | LG Electronics Inc. | Proposal 1: For QCL-ed assumption performing RSSI measurement, - in FR2-2 intra-frequency, - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier. - in FR2-2 inter-frequency, - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided. |
| R4-2212402 | MediaTek inc. | Proposal 1: If TCI is not provided for the inter-frequency RSSI measurement, the reference TCI should be on the same FR2-2 band as the target inter-frequency RSSI measurement. |
| R4-2212912 | Nokia, Nokia Shanghai Bell | Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |
| R4-2212956 | Huawei, HiSilicon | Proposal 1: RAN4 should inform RAN2 to extend maxNumberRxBeam to 12 according to RAN4’s agreement. Proposal 2: Define frame boundary alignment tolerance of PDSCH symbols as 7 symbols for (960, 960) when deriveSSB-IndexFromCell is disabled. Proposal 3: Capture the TP in RAN1 LS in TS 38.133. Proposal 4: Introduce scheduling restriction on DL during RSSI measurement in FR2-2. Observation 1: The demand on hardware and memory for cell detection in 480 kHz and 960 kHz is increased compared with FR2-1 but not significantly. Observation 2: For powerful UE supporting 480 kHz and 960 kHz, the delay will be even longer than 120 kHz. Proposal 5: Introduce UE capability on relaxed requirements for cell detection. |
| R4-2213023 | vivo | Observation 1: The L3 measurement scheduling restriction requirements and the frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is enabled based on the assumption of 1km. Observation 2: If the propagation difference needs to be revised to 1.5km, deriveSSB-IndexFromCell may be not always enabled for 480kHz. The assumption of propagation difference shall be the same whether deriveSSB-IndexFromCell is enabled or disabled. Proposal 1: Frame boundary alignment tolerance of PDSCH symbols shall be 6 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. Proposal 2: For cell re-selection, the scaling factor N1 of FR2-2 should be magnified by 1.5 times compared with FR2-1. The requirements for measurements of serving cell, intra-frequency NR cells and inter-frequency NR cells can be defined as below: Table 4.2.2.2-1: Nserv DRX cycle length [s]; Scaling Factor (N1); Nserv [number of DRX cycles]. FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; M1*N1*4. 0.64; 5; 8; M1*N1*4. 1.28; 4; 6; N1*2. 2.56; 3; 5; N1*2. Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length.. Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 5; 8; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 6; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 5; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. Note 3: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.. Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 5; 8; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 6; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 5; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length.. Proposal 3: The requirements of PSS/SSS detection period and SSB measurement period for FR2-2 for intra-frequency measurement can be defined as below: Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Mpss/sss_sync_w/o_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps =60, 120 and 180 for 120kHz, 480kHz and 960kHz respectively. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps =36, 72 and 108 for 120kHz, 480kHz and 960kHz respectively. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps =36, 72 and 108 for 120kHz, 480kHz and 960kHz respectively. Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Mmeas_period_w/o_gaps : For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps =60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps =36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps =36. Proposal 4: The requirements of PSS/SSS detection period and SSB measurement period for FR2-2 for inter-frequency measurement can be defined as below: Table 9.3.4-2: Time period for PSS/SSS detection, (Frequency range FR2) Condition NOTE1,2; TPSS/SSS_sync_inter. No DRX; Max(600ms, Ceil(Kgap Mpss/sss_sync_inter) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(600ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(Kgap Mpss/sss_sync_inter) DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured.. Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96 samples, 192 samples and 288 samples for 120kHz, 480kHz and 960kHz respectively. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60 samples, 120 samples and 180 samples for 120kHz, 480kHz and 960kHz respectively. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60 samples, 120 samples and 180 samples respectively. Table 9.3.5-2: Measurement period for inter-frequency measurements with gaps (Frequency FR2) Condition NOTE1,2; T SSB_measurement_period_inter. No DRX; Max(400ms, Ceil(Kgap Mmeas_period_inter) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(400ms, Ceil(1.5 * Kgap Mmeas_period_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(Kgap Mmeas_period_inter) DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent gaps, the MRGP above is the MRGP of the measurement gap associated with the target frequency layer to be measured if concurrent measurement gaps are configured.. Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64 samples. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40 samples. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40 samples. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mmeas_period_inter =96 samples. For a UE supporting FR2-2 power class 2, Mmeas_period_inter=60 samples. For a UE supporting FR2-2 power class 3, Mmeas_period_inter =60 samples. |
| R4-2213351 | Ericsson | Proposal 1: Define frame boundary alignment tolerance of PDSCH when deriveSSB-IndexFromCell is disabled following Option2: SSB SCS (KHz); Data SCS (KHz); Frame boundary alignment tolerance of PDSCH symbols (deriveSSB-IndexFromCell disabled). 960; 120; 1 120KHz symbol. 960; 480; 3 480KHz symbol. 960; 960; 7 960KHz symbol. Proposal 2: Only follow Option 1a and suggest shifting SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index. Proposal 3: In response to LS R1-2205582 from RAN1 on TCI assumption for RSSI measurement for FR2-2, kindly ask RAN1 to further clarify the reference serving cell for TCI assumption and answer our questions as discussed above. |
| R4-2212911 | Nokia, Nokia Shanghai Bell | Include the following text in clause 9.2A.7.1: For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier. Include the following text in clause 9.3A.8: For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell, UE can assume that the measurement resources are QCL-TypeD to the DL RS associated with the TCI state of the active BWP of the carrier on which the RMTC configuration is provided. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided. |
| R4-2213024 | vivo | Proposal 1: For LS from RAN1 related to TCI assumption for RSSI measurement, the following highlighted part can be omitted and the remaining part is enough to indicate the TCI state of RSSI measurement resources for unlicensed operation in FR2-2. |

Open issues summary

Sub-topic 1-1 Frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled

Issue 1-1-1: Frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled

- Proposals

  - Option 1: Frame boundary alignment tolerance of PDSCH symbols shall be 6 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. (CATT, vivo)

  - Option 2: Frame boundary alignment tolerance of PDSCH symbols shall be 7 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. (Ericsson, Huawei)

- Recommended WF

  - Discuss above options.

Comments collection

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1-1: We support option 2. Companies supported option 1 argued that the scheduling restriction may be impacted if change the tolerance of PDSCH symbols from 6 to 7. But we have different understandings, the scheduling restrictions of 7 symbols (6+1) can still hold where the tolerance of PDSCH is 7. However, if majority views is to keep the original conclusion. We can compromise to option 1. |
| Ericsson | Issue 1-1-1: We prefer Option2. When deriveSSB-IndexFromCell is enable, it implies network is aware of the timing offset between cells and network can secure and guarantee deriveSSB-IndexFromCell. But if deriveSSB-IndexFromCell is disable, timing offset between cells is unknown and cannot be a prerequisite of another requirement. |
| Nokia | We prefer Option 2. |
| Qualcomm | We prefer to keep the original agreement – Option 1 which is based on all the assumptions that were considered while deciding that deriveSSB-IndexFromCell is always enabled for 480kHz SCS |
| Apple | We prefer Option 1. |
| CATT | We support option 1 to keep the previous agreement. If it is considered that the UE can use the PBCH DMRS to derive the LSB of the SSB index, this detection approach can only resolve SSB index ambiguity between 8 SSB indexes. So we consider that as long as the difference between the SSB index numbers of the reference cell and the target cell is less than 7, the time difference between the reference cell and the target cell can be as many as dozens of symbols, it is different with 7 960KHz PDSCH symbols. For option 1, if companies think the assumed propagation delay difference is not enough, we can appropriately increase the assumed cell radius. However, we have already defined many requirements based the previous assumption. |
| vivo | We support Option 1 which is based on the assumption of 1km of propagation difference among gNB. In our understanding, the requirements of L3 measurement scheduling restriction and the frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is enabled which have been defined are also based on the assumption of 1km. If the assumption of 1km needs to be revised to 1.5km as described in Option 2, deriveSSB-IndexFromCell may be not always enabled for 480kHz which is not aligned with the current spec. We suggest that the assumption of propagation difference shall be the same whether deriveSSB-IndexFromCell is enabled or disabled. In addition, as mentioned in the last meeting, if the UE determine the SSB index by only decoding the PBCH DMRS, as long as the SSB index number difference between reference cell and target cell is less than 7, the timing difference between reference cell and target cell may be up to dozens of symbols instead of 7 symbols. |

Sub-topic 1-2 QCL-ed assumption for RSSI

Background: A LS is sent from RAN1 (R1-2205582) about QCL-ed assumption for RSSI measurement in FR2-2, and text proposals are provided for infra-frequency/inter-frequency RSSI measurement. Based on submitted contributions in this meeting (R4-2212215/ R4-2212402/ R4-2212956/ R4-2213351/ R4-2212911/ R4-2211879), companies mainly have concerns on the inter-frequency case where the TCI determination seems not clear from RAN1’s LS. The discussion is divided into infra-frequency and inter-frequency. LS can be triggered to other working group if necessary.

Issue 1-2-1: Intra-frequency QCL-ed assumption

- Proposals

  - Option 1: Original TP in RAN1 LS (Nokia, Huawei)

For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier.

  - Option 2: Minor changes in wording (R4-2212402)

For performing RSSI measurement in FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a serving cell in FR2-2.

- Recommended WF

  - The situation is clear for intra-frequency RSSI measurement. Companies please check whether the original TP or updated TP can be agreeable.

Comments collection

| Company | Comments |
| --- | --- |
| LGE | Issue 1-2-1: Slightly prefer option 1, but both option 1 and option 2 are fine for us |
| MTK | Support Option 2. The TCI needs to refer to FR2-2 serving cells if the TCI is not explicitly configured, considering FR1-FR2-2 CA. Using TCI in FR1 is not desired. |
| Huawei | We are fine with either option 1 and option 2. |
| Ericsson | Issue 1-2-1: We agree on Option 2. Maybe serving cell can be FR2-1 considering CBM between FR2-1 and FR2-2 in some cases? |
| Nokia | Issue 1-2-1: Intra-frequency QCL-ed assumption We are ok with Option 1 or Option 2. Our understanding is that scenarios with FR2-1 and FR2-2 are not in the scope of the work item, according to the agreements in RAN4 #101. |
| Qualcomm | We prefer option 1 as it clearly mentions which carrier should be the reference. |
| Apple | From specification text’s perspective, Option 2 is better. |
| vivo | We are fine with option 1. For option 2, we suggest to further revise to ‘the a serving cell in FR2-2. |

Issue 1-2-2: Inter-frequency QCL-ed assumption

- Sub-issue 1: Reference serving cell when the TCI state is provided in the RMTC configuration

  - Option 1: The QCL reference serving cell is determined by serving cell ID indicated in TCI-StateInfo in RMTC-Config. (Ericsson)

- Recommended WF

  - Based on contributions (R4-2212215/R4-2212402/R4-2213351), “carrier on which the RMTC configuration is provided” is not clear and the corresponding part is redundant. Companies please check whether option 1 is the common understanding.

- Sub-issue 2:  When the TCI state is provided in the RMTC configuration

  - Option 1: Original TP in RAN1 LS (Nokia, Huawei)

  - Option 2: (MTK, LGE, vivo)

For performing RSSI measurement in FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration of the inter-frequency RSSI measurement.

- Recommended WF

  - If Option 1 in Sub-issue 1 is the common understanding, please check whether TP in option 2 is agreeable.

- Sub-issue 3:  When the TCI state is not provided in the RMTC configuration

  - Option 1: Original TP in RAN1 LS (Nokia, Huawei)

  - Option 2: (MTK)

If no TCI state is provided in the RMTC configuration of the inter-frequency RSSI measurement, UE shall assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration on the same FR2-2 band as the inter-frequency RSSI measurement.

If no TCI state is provided in any of RMTC configuration on the same FR2-2 band, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a serving cell on the same FR2-2 band.

- Recommended WF

  - Companies please check whether option 2 is agreeable for the case when TCI is not proved in RMTC.

- Sub-issue 4: Whether further clarifications are needed from RAN1

  - Proposal 1: (Apple)

Case 1) If a UE has serving cell in FR2-1 but no serving cell in FR2-2, it is not clear if the UE should use the beam in FR2-1 for FR2-2 RSSI measurement. If so, there still could be some implementation difficulties to “use the beam in FR2-1” since the UE antenna panel for FR2-1 and FR2-2 may have different configurations in terms of the number of antenna elements per panel and panel location in the UE, and thus it may not always be possible to keep the QCL type-D relation. In this case, should UE be allowed to conduct beam sweeping?

Case 2) If a UE has no serving cell in FR2-1 or FR2-2, it is not clear if that explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI? In our understanding, there are two options:

- Option 1: UE performs RX beam sweeping first on the TCI state associated RS. Once the RX beam is determined, UE performs RSSI measurement and reports it.

- Option 2: UE performs RSSI measurement by using Rx beam sweeping, and determines which RSSI measurement to report. For example, the UE can report the RSSI measurement results based on the strongest Rx beam (i.e., with highest RSSI value). The beauty of this option is it combines beam sweeping and RSSI measurement in the same procedure and hence may save time in measurement.

- Recommended WF

  - Discuss the cases in proposal 1. As the question in proposal 1 is related to the sub-issue 1/2/3, companies please check whether consensus can be reached in sub-issue 1/2/3 which may eliminate the ambiguity in above proposal 1.

Comments collection

| Company | Comments |
| --- | --- |
| LGE | Issue 1-2-2: Sub-issue 1 We are the same understanding with option 1, and “carrier on which the RMTC configuration is provided” is not clear Sub-issue 2 Support option 2. In option 2, we think the yellow highlight “of the inter-frequency RSSI measurement” is redundant since this description is for inter-frequency RSSI measurement. For “can” or “shall”, can follow the same word for intra-frequency RSSI measurement in Issue 1-2-1. Sub-issue 3 Prefer original TP from RAN1 LS in case no TCI state is provided. For option 2. If further clarification are needed, RAN4 can send LS to RAN1 with Sub-issue 4. Sub-issue 4 For case 1, there is no FR2-1+FR2-2 deployment scenarios for this WI in our understanding, so we think that the case 1 can be skipped. For case 2, when UE has no serving cell in FR2-2 (i.e., serving cell is FR1 only), option 2 can be considered. However, before concluding this issue in RAN4, we prefer to send LS to RAN1 for further clarification. |
| MTK | Sub-issue 1: Option 1 is reasonable, for the case the TCI is provided. Sub-issue 2: support Option 2. “of the inter-frequency RSSI measurement” is to clarify if there are multiple RSSI is configured, e.g. 1 intra-freq. RSSI and 1 inter-freq. RSSI Sub-issue 3: support Option 2. The original TP wording, i.e. ““carrier on which the RMTC configuration is provided” is not clear and the corresponding part is redundant” is not clear and the corresponding part is redundant. Sub-issue 4: Support Option 1. Option 2 can be up to UE implementation. Prefer to discuss this issue within RAN4 first. |
| Huawei | Sub-issue 1: We also agree that the current description in RAN1 LS about the determination of reference Cell is not clear. And we also agree with that the reference cell shall follow the serving cell ID in RMTC configuration introduced by RAN2. Sub-issue 2 If it is agreed that the reference cell of TCI state is indicated by serving cell ID in RMTC, then this case is clear when TCI is explicitly configured. With the clarification from MTK, we are fine with option 2. Sub-issue 3 We are fine with option 2 which makes sense. Minor suggestion on the wording “in FR2-2” is enough as inter-band is not considered in Rel-17. And we think it is related to case 1/2 in sub –issue 4. Sub-issue 4 Regarding case1/2, we share similar views as LGE that they can be merged to one case that there is no serving cell in FR2-2. For the case when there is serving cell in FR2-2, We believe sub-issue 1/2/3 can already fix the issue (regardless of TCI is configured or not). The only unclear case is when there is no serving cell in FR2-2, we think following solutions can be considered: 1. Do not define requirements for this case. 2. Allow beam sweeping time in the delay requirements 3. Add conditions for these case (e.g. the associated RS is known and beam sweeping is not needed) |
| Ericsson | Issue 1-2-2: Sub-issue 1 We agree on recommended WF just remove it. Sub-issue 2 We can agree on Option 2 Sub-issue 3 We generally support Option 2 and same question in Issue 1-2-1. |
| Nokia | Issue 1-2-2: Sub-issue 1: In our view Option 1 clarifies the issue. Sub-issue 2: Yes, the TP in Option 2 is agreeable. Sub-issue 3: We are fine with Option 2, with the change proposed by Huawei. Sub-issue 4: We agree with LGE and Huawei. We don’t think case 1 is relevant (according to the deployment scenarios agreed in RAN4 #101). We can clarify case 2 with RAN1. |
| Qualcomm | We think we need to break down the scenarios in this case. 1st scenario – TCI state is provided in the RMTC configuration 2nd scenario – TCI state is provided on an inter-freq FR2-2 carrier 3rd scenario – TCI state is provided on an inter-freq FR1 carrier 4th scenario – TCI state is not provided For scenario 2, the UE should use the DL-RS associated with the TCI state of the carrier on which TCI state is provided as the QCL source. For scenario 3 and 4, UE should use the latest received PDSCH and the latest monitored CORESET in the active BWP of a carrier in FR2-2 as the QCL source. So we propose an alternate TP: - For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. - If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell, UE can assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state of the active BWP of the FR2-2 carrier on which the RMTC configuration is provided. - If no TCI state is provided in the RMTC configuration or if TCI state is provided in the RMTC configuration on a non FR2-2 carrier and the configured RSSI measurement resources are not confined within the bandwidth of any serving cell UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a carrier in FR2-2 on which the RMTC configuration is provided - Otherwise, no beam-related requirements for RSSI |
| Apple | Sub-issue 1: Option 1 is OK. We can also clarify with RAN1. Sub-issue 2: Option 2 is OK. Sub-issue 3: it is not clear “If no TCI state is provided in the RMTC configuration of the inter-frequency RSSI measurement, UE shall assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration on the same FR2-2 band as the inter-frequency RSSI measurement.” Sub-issue 4: We are OK to not consider case 1 and to clarify case 2 with RAN1. |
| vivo | Issue 1-2-2: Sub-issue 1 We agree with Option 1. ‘carrier on which the RMTC configuration is provided’ is not clear. The reference serving cell can be indicated in RMTC configuration. Sub-issue 2 Support Option 2. In our understanding, inter-frequency RSSI measurement has assumed that the RSSI measurement bandwidth is not contained with the current bandwidth of the UE. In addition, we suggest if ‘of the inter-frequency RSSI measurement’ needs to be added in the text, it may be better to add in the start of text ‘For performing inter-frequency RSSI measurement in FR2-2’. And for intra-frequency RSSI measurement, we can follow the same rule ‘For performing intra-frequency RSSI measurement in FR2-2’. Sub-issue 3 For the TCI state is not provided in the RMTC configuration, we suggest the following TP: If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a serving cell on the carrier on which the RMTC configuration is provided. Sub-issue 4 For the case where the UE has no serving cell on the carrier, we suggest that there is no need to define the requirement. |

Issue 1-2-3: Scheduling restriction on DL during RSSI measurement in FR2-2

- Proposals

  - Option 1: Introduce scheduling restriction on DL during RSSI measurement in FR2-2. (Huawei)

- Recommended WF

  - Discuss above option.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1: |
| MTK | Open to discuss. Our understanding is the scheduling restriction apply only for the case the provided TCI is different from the serving cell's RS/channels |
| Huawei | We support option 1. For intra-frequency case, only scheduling restriction on UL is considered currently which is adopted from NR-U. When TCI for RSSI is different from DL data, UE is not required to receive data simultaneously. To MTK: Yes, itis for the case when TCI different from serving cell’s RS/channel |
| Ericsson | Issue 1-2-1: Reasonable to add scheduling restriction |
| Nokia | We also believe that it is reasonable to introduce scheduling restriction, but we would prefer a clearer agreement, for example: “Introduce scheduling restriction on DL during RSSI measurement in FR2-2, for the case in which the TCI state provided in the RMTC configuration differs from the TCI state of the active BWP” |
| Apple | We are open to further discussion. |

Sub-topic 1-3 Cell detection relaxation

Issue 1-3-1: Cell detection relaxation

- Proposals

  - Option 1: Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR =  1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz.(Nokia)

  - Option 2: Introduce UE capability on relaxed requirements for cell detection. ( Huawei)

- Recommended WF

  - Discuss above options.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1: |
| MTK | Can support Option 1 |
| Huawei | We would like companies to considered option 2. As analysed in our paper, at worst case, the number of samples that UE needs to processed is increase by 30-40% compared with 120 KHz. Relaxation factor of 2 and 3 will make the delay too long. And it will result that for more powerful UE supporting 480/960 kHz, the delay will be even longer than 120 KHz. A compromised solution is to introduce a capability for this. When UE is not capable of this capability, KFR = 2 or 3 as agreed in previews meeting. We provide the capability table as follow for reference: |
| Ericsson | Issue 1-3-1: We can agree on Option 2 to avoid all UEs using too long detection time, Option 2 can at least help some advanced UEs change cells more effectively. |
| Nokia | We agree to Option 2. Our proposal is an implementation of the agreement in the last meeting, but we agree that this can be a UE capability. |
| Qualcomm | We support Option 1 which is based on the agreement from last meeting. We are defining minimum RAN4 requirements here. A UE can always do better if it can, we don’t think a UE capability is needed for this. |
| Apple | We support Option 1. |
| CATT | Option 2 is more flexible to realize cell detection relaxation. |
| vivo | For Option 1, we understand considering the impact on Rx beam sweeping factor and HW/memory for FR2-2, it is enough to define the different values of Mpss/sss for 120kHz, 480kHz and 960kHz instead of introducing the new factor KFR. However, we are also fine with Option 1. For Option 2, we prefer to not introduce UE capability. As mentioned from Qualcomm, we only defining the minimum requirements. |

Sub-topic 1-4 Imapct of beam sweeping factor

Issue 1-4-1: LS on extension on maxNumberRxBeam

- Proposals

  - Option 1: Inform RAN2 about RAN4 agreement to extend the maximum value of maxNumberRxBeam to 12 for FR2-2.(LGE, Huawei)

- Recommended WF

  - Please LGE provide the capability table in this thread and also in A.7 Rel-17 feature list. It is expected to have a stable version in the first week which could be included in the LS of feature list to RAN2.

Comments collection

| Company | Comments |
| --- | --- |
| LGE | Issue 1-4-1: Based on moderator suggestion, we provide the capability table below, and we will also add the table in feature list thread before end of 1st round. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 15. NR_ext_to_71GHz; 15-x; Support of extended maxNumberRxBeam for FR2-2; Recommended CSI-RS resource repetition number per resource set. The candidate value set is {2,3,4,5,6,7,8,9,10,11,12}; Yes; No; Per band; N/A; Applicable to FR2-2 only; N/A; Mandatory with capability signalling. |
| Huawei | Thanks LEG for providing the table. We support option 1 and the capability table proposed by LGE. |
| Ericsson | Issue 1-4-1: Agree on Recommended WF |
| Nokia | We agree to the proposed WF |
| Apple | We support Option 1. |
| CATT | Issue 1-4-1: Agree with the Recommended WF |
| vivo | Agree with the proposed WF. |

Issue 1-4-2: specification updating on Rx beam sweeping factor

- Recommended WF

  - Similar changes are proposed in R4-2211878/ R4-2213023/ R4-2212957 /R4-2212957. Companies please directly comment on the CRs in 1.2.5.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-4-2: |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212216 (LGE) CR on QCL-ed assumption for RSSI measurement in FR2-2 | Nokia: This change is overlapping with Nokia’s CR in R4-2212390 / Mediatek’s CR in R4-2212403 and Qualcomm’s CR R4-2214077, and depends on the conclusion of issue 1-2. |
|  | Moderator : It is suggested to revise this CR to capture QCL assumptions for RSSI measurement |
| R4-2212403 (MediaTek inc.) CR on TS38.133 RRM requirements in FR2-2 | Nokia: This change is overlapping with Nokia’s CR in R4-2212390 / LGE’s CR R4-221216 and Qualcomm’s CR R4-2214077, and depends on the conclusion of issue 1-2. |
|  | Company B |
|  | Huawei: For RSSI part, it depends on conclusion of pending issues. |
|  | Moderator: It is suggested to revise this CR to capture change #1. RSSI parts can be merged to 2216 |
| R4-2212682 (Nokia, Nokia Shanghai Bell) Draft CR Measurement procedure updates for FR2-2 | Huawei: The changes on TSSB_time_index_intra are similar as R4-2213025 |
|  | Moderator: It is suggested to revise this CR to merge the changes in 3025. |
| R4-2212957 (Huawei, HiSilicon) Draft CR on beam sweeping factor for RRM requirements of FR2-2 | Nokia Last row on Table 6.2.1.2.1-2 should be < -8 not ≥ -8 9.2.5 – we also have a text proposal for this clause. We do not have agreements for increasing Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps, and there is already a factor that we agreed for relaxation as discussed on Issue 1-3-1. For that clause we would prefer to use our version of the text in R4-2212682. Clause 9.3.4 - We do not have agreements for increasing Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps, and there is already a factor that we agreed for relaxation as discussed on Issue 1-3-1. For that clause we prefer the only the relaxation factor is included and Mpss/sss_sync_w/o_gaps not to be changed Style: Tables Table 4.2.2.3-1, Table 4.2.2.4-1 , Table 4.2.2.9.2-1 (and others) have different styles. We do not have agreements yet for the SCell activation in FR2-2. In clause 8.3A.2, NRxBeam is being used, but we agreed in RAN4 #102 to use N instead. But we do not have agreements yet for the SCell activation in FR2-2. |
|  | Moderator: It is suggested to revise this CR for changes exempt for clause 4 and 9 which will be captured in 3025/2682 |
| R4-2213025 (vivo) CR on introduction of cell Re-selection, inter-frequency and intra-frequency measurments requirements for FR2-2 | Huawei: Fine with changes on updating Rx sweeping factor for clause 4 and 9. The changes on TSSB_time_index_intra are similar as R4-2212682. |
|  | Nokia: This CR is overlapping with Huawei’s CR R4-2212957. Table 4.2.2.9.2-1 has multiple styles. The proposal for Mpss/sss_sync_w/o gaps is overlapping with the proposal in sub-topic 1-3. If the proposal there is accepted, the numbers in this CR should be reviewed |
|  | Moderator: It is suggested to revise this CR to keep the changes for clause 4. Other changes will be merged to 2682 |
| R4-2214077 (Qualcomm) Draft CR for RSSI measurements for FR2-2 | Huawei: Depends on the conclusion of pending issues. |
|  | Nokia: This change is overlapping with Nokia’s CR R4-2212390 / LGE’s CR R4-221216 and Mediatek’s CR in R4-2212403, and depends on the conclusion of issue 1-2. |
|  | Moderator: It is suggested to merge this CR to 2216. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Issue 1-1-1: Frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled 7 companies comment in 1st round. 3 companies support option 2 and 4 companies support option 1. Tentative agreements: NA Candidate options: - Option 1: Frame boundary alignment tolerance of PDSCH symbols shall be 6 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. - Option 2: Frame boundary alignment tolerance of PDSCH symbols shall be 7 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. Recommendations for 2nd round: Continue discussion in 2nd round |
| Sub-topic#1-2 | Issue 1-2-1: Intra-frequency QCL-ed assumption 8 companies comment in 1st round and the issue is discussed in GTW Aug-18 with following agreement: Agreement: - For performing intra-frequency RSSI measurement in FR2-2, UE canshall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE canshall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier in FR2-2. Candidate options: NA Recommendations for 2nd round: NA |
|  | Issue 1-2-2: Inter-frequency QCL-ed assumption 8 companies comment in 1st round and the issue is discussed in GTW Aug-18 with following agreement: Agreement: - The QCL reference serving cell is determined by serving cell ID indicated in TCI-StateInfo in RMTC-Config, when the TCI state is provided in the RMTC configuration. Agreement: - For performing inter-frequency RSSI measurement in FR2-2, - if the TCI state is provided in RMTC configuration of FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. - FFS on the following bullet - if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided Agreement: - RAN4 to clarify the the case with RAN1 when there is no serving cell in FR2-2. - If a UE has no serving cell in FR2-2, it is not clear if that explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI? Candidate options: NA Recommendations for 2nd round: Discuss the follow FFS point: - if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided Discuss LS reply to RAN1 |
| Sub-topic#1-3 | Issue 1-3-1: Cell detection relaxation 8 companies comment in 1st round. 3 company support option 2 and 5 company support option 1. Tentative agreements: NA Candidate options: - Option 1: Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. - Option 2: Introduce UE capability on relaxed requirements for cell detection. Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic#1-4 | LS on extension on maxNumberRxBeam 8 companies comment in 1st round and the issue is discussed in GTW Aug-18 with following agreement: Agreement: - Inform RAN2 and RAN1 about RAN4 agreement to extend the maximum value of maxNumberRxBeam to 12 for FR2-2. - The revised LS R4-2212214 is agreeable. Tentative agreements: NA Candidate options: NA Recommendations for 2nd round: NA |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 1-1 Frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled

Issue 1-1-1: Frame boundary alignment tolerance of PDSCH symbols when deriveSSB-IndexFromCell is disabled

- Proposals

  - Option 1: Frame boundary alignment tolerance of PDSCH symbols shall be 6 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. (CATT, vivo, Apple, QC)

  - Option 2: Frame boundary alignment tolerance of PDSCH symbols shall be 7 for the combination of 960kHz SSB SCS and 960kHz Data SCS when deriveSSB-IndexFromCell is disabled. (Ericsson, Huawei, Nokia)

- Recommended WF

  - Discuss above options.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: |
| Apple | We prefer Option 1. |
| Ericsson | Issue 1-1-1: It is rationale to take propagation delay offset based on network deployment scenario as assumption to define requirement which is enabled by network, i.e. deriveSSB-IndexFromCell is enabled. On the contrary, if requirement which isn’t controlled or ensured by network, assumption on hypothetical network deployment scenario is not reliable. Here is the basic reasoning as follows: ‘deriveSSB-IndexFromCell is enabled’ relies on certain network deployment scenario, but ‘deriveSSB-IndexFromCell is disabled’ doesn’t need to rely on certain network deployment scenario. And in legacy requirements, we did not assume network deployment scenario when ‘deriveSSB-IndexFromCell is disabled’. |
| Qualcomm | Issue 1-1-1: Support Option 1. |
| vivo | Issue 1-1-1: Support Option 1. deriveSSB-IndexFromCell is always enabled for 480kHz based on the assumption of Option 1. We understand there is no difference on the propagation difference whether deriveSSB-IndexFromCell is enabled or disabled. |
| CATT | Issue 1-1-1: Option 1. As we commented in 1st round, if it is considered that the UE can use the PBCH DMRS to derive the LSB of the SSB index, this detection approach can only resolve SSB index ambiguity between 8 SSB indexes. So we consider that as long as the difference between the SSB index numbers of the reference cell and the target cell is less than 7, the time difference between the reference cell and the target cell can be as many as dozens of symbols, it is different with 7 960KHz PDSCH symbols. |

Sub-topic 1-2 QCL-ed assumption for RSSI

Issue 1-2-2: Inter-frequency QCL-ed assumption

Agreement in GTW:

Agreement:

- The QCL reference serving cell is determined by serving cell ID indicated in TCI-StateInfo in RMTC-Config, when the TCI state is provided in the RMTC configuration.

Agreement:

- For performing inter-frequency RSSI measurement in FR2-2,

  - if the TCI state is provided in RMTC configuration of FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration.

  - FFS on the following bullet

    - if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided

Agreement:

- RAN4 to clarify the the case with RAN1 when there is no serving cell in FR2-2.

  - If a UE has no serving cell in FR2-2, it is not clear if that explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI?

- Proposals

    - Option 1: if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided

- Recommended WF

  - Discuss the FFS point.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-2: |
| MTK | This sentence can be removed “[or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier]”, because TCI can only be provided in FR2-2 RMTC. Regarding [a or the], prefer to “a”, because “the” may cause misunderstanding that the carrier is for this inter-frequency. |
| LGE | We are fine to remove the [or if TCI state is only … ]. For further clarification, if no TCI state is provided and there are multiple serving cells in FR2-2, which serving cell should UE assume RSSI measurement resources are QCLed with TypeD to received PDSCH or monitored CORESET? Or is the QCLed TypeD assumption to received PDSCH or monitored CORESET for all serving cells in FR2-2 same since common beam management is assumed? |
| Apple | We agree that [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] should be removed for clarity. Regarding “[a or the] carrier in FR2-2”, we propose to use “a serving cell in FR2-2”. In the case of no serving cell in FR2-2, in GTW Aug. 18 it was agreed to send an LS to RAN4 seeking clarification. |
| Huawei | For the case when TCI state is only provided in a non-FR2-2 carrier, according to companies comments during GTW, it is stated in RAN2 spec that it is only applicable to shared spectrum in FR2-2 as shown below. From our understanding, it means the TCI will only be configured when the RSSI measurement in on FR2-2, and it does not preclude the reference cell is a non-FR2-2 cell. We are also fine to remove it as it can be covered by the case mentioned in LS (not serving cell in FR2-2). For [a or the], we share similar views as MTK. We don’t fully understand what “the carrier ” refers to. |
| Qualcomm | We prefer to keep “[or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier]”. As pointed out by Huawei, the TCI state can be provided on a non-FR2-2 carrier. We are fine to remove it if we receive a clarification on the scenario based on the LS. |
| vivo | We are fine with removing the [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier]. For the part of [a or the], we propose to use ‘the serving cell by which the RMTC configuration is provided’. In our understanding, it is necessary to clear that which serving cell should UE assume RSSI measurement resources are QCLed with TypeD to received PDSCH or monitored CORESET. For different serving cells, the latest PDSCH and PDCCH scheduling situation may be different. We believe that UE can refer to the serving cell by which the RMTC configuration is provided. |

Issue 1-2-3: Scheduling restriction on DL during RSSI measurement in FR2-2

- Proposals

  - Option 1: Introduce scheduling restriction on DL during RSSI measurement in FR2-2.

- Recommended WF

  - Discuss above option.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-3: |
| MTK | Fine with Option 1 with the clarification for the case in which the TCI state provided in the RMTC configuration differs from the TCI state of RS/channel on the active BWP. Open for the wording refinement. |
| Apple | We agree that scheduling restriction is needed for the case mentioned above, “the TCI state provided in the RMTC configuration differs from the TCI state of RS/channel on the active BWP” |
| Huawei | We are fine with MTK modification. We have provide a CR with following wording: “- The UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on symbols which are overlapping in time with RSSI measurement symbols configured by RMTC if the RSSI measurement resources are not QCL-ed with typeD to the DL RS in the active TCI state of PDCCH/PDSCH. ” However, with some further discussion, we think some the impact of beam switching is not considered. For 480/960KHz, even the measurement resource are QCL-ed with PDCCH/PDSCH, UE may need to perform beam switching to the configured TCI. So we would like to add the following case : For 480 kHz and 960kHz, The UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on symbols which are overlapping in time with the first and last RSSI measurement symbols configured by RMTC. |
| Ericsson | Agree on Option1 and MTK’s suggestion on clarification. |
| vivo | We are fine with Option 1. |

Sub-topic 1-3 Cell detection relaxation

Issue 1-3-1: Cell detection relaxation

- Proposals

  - Option 1: Introduce the factor KFR to account for the cell-detection relaxation agreed in RAN4 #103, where: For FR2-1, KFR = 1. For FR2-2: KFR =  1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz.(Nokia)

  - Option 2: Introduce UE capability on relaxed requirements for cell detection. ( Huawei)

- Recommended WF

  - Discuss above options.

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1: |
| MTK | Support Option 1 and prefer not to define UE capability as it is minimum requirement. |
| Apple | Option 1. |
| Huawei | We recognize that it is in late stage to consider the UE capability. But as explained in 1st discussion, current requirement reflect that the delay of 480/960 KHz is times longer than that 120 Khz when 480/960 KHz are optionally supported by UE. |
| Ericsson | Support Option 2 to allow flexible use case on cell change and mobility. The current cell detection time is a bit too long to support UE’s medium level mobility, e.g. pedestrian. |
| Qualcomm | Support option 1. No UE capability needed as RAN4 defines minimum UE requirements |
| Nokia | We support option 1. |
| vivo | Support Option 1. |
| CATT | Option 2 is more flexible, but we can compromise to Option 1. |

Topic #2: Timing requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211874 | Apple | Proposal 1: Option 1 of the UE timing advance adjustment accuracy for 480/960kHz SCS is agreed. |
| R4-2212899 | Ericsson | MRTD and MTTD definition and values as agreed in WF are implemented in CR [2] R4-2012900. |

Open issues summary

Sub-topic 2-1  Timing Advance adjustment accuracy

Background:  Way forward in RAN4#103-e R4-2210590

Way forward: (To be handled during maintenance phase) The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in the table below: - Option 1: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±12 Tc]; [±8 Tc]. - Option 2: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±8 Tc]; [±4 Tc].

Issue 2-1-1: Timing Advance adjustment accuracy

- Proposals

  - Option 1: Option 1 of the UE timing advance adjustment accuracy for 480/960kHz SCS is agreed. (Apple)

- Recommended WF

  - Check whether option 1 is agreeable.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: |
| Ericsson | Issue 2-1-1: We prefer option 2, since this is a linear scale from SCS = 120 kHz to SCS = 480 kHz and SCS = 960 kHz. The UL uncertainty budget is already very tight with almost no room for RF channel delay spread. |
| Nokia | We prefer Option 2 from the Way Forward. As mentioned for many meetings we think this accuracy is important to keep network efficiency. So we prefer to keep the previous tentative agreement. |
| Apple | To Ericssion: there is no scaling from 15kHz SCS to 30kHz SCS. We do need to consider the implementation issues. We’d also like to note that the current values are tentative agreement. |
| CATT | Prefer Option 2 to make progress. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211875 (Apple) Draft CR on UE timing advance adjustment accuracy for Rel-17 NR extension to 71GHz | Ericsson: (Same comment as issue 2-1-1. We prefer option 2 adjustment uncertainty = +/-8Tc for SCS = 480 kHz and 4 Tc for SCS = 960 kHz, since this is a linear scale from SCS = 120 kHz to SCS = 480 kHz and SCS = 960 kHz. The UL uncertainty budget is already very tight with almost no room for RF channel delay spread. |
|  | Nokia: we don’t agree changing the TA accuracy requirements. We prefer to keep the existing values. |
| R4-2212683 (Nokia, Nokia Shanghai Bell) Draft CR Timing RRM requirements for FR2-2 | Ericsson: Changes in this CR are OK, but Ericsson’s CR in R4-2212900 also add the new subframe definitions into the 7.5.1 and 7.6.1 sections. We can merge CRs. |
|  | Company B |
|  | Huawei: Similar changes in 2683/2900/2985/4078 where coordination is needed. |
|  | Moderator: It is suggested to merge this CR into 4078. |
| R4-2212900 (Ericsson) MRTD and MTTD values and definitions in CA and DC | Nokia We are fine with eh chances. We have a CR with similar changes R4-2212683, I think those could be merged. |
|  | Moderator: It is suggested to merge this CR into 4078. |
| R4-2212958 (Huawei, HiSilicon) Draft CR on timing requirements of FR2-2 | Ericsson: Same comment as to Nokia CR R4-2212683. Ericsson’s CR in R4-2212900 also add the new subframe definitions into the 7.5.1 and 7.6.1 sections. We can merge CRs. |
|  | Nokia We have a CR covering similar changes for MRTD R4-2212683. We prefer that the clarification on the sync scenarios is not done in the tables, since it is more in line with the existing text like what is implemented in R4-2212683 and R4-2212900. |
|  | Moderator: It is suggested to merge this CR into 4078. |
| R4-2214078 (Qualcomm Incorporated) Draft CR for timing requirements for FR2-2 – MRTD-MTTD | Ericsson: This CR adds the definitions also to 7.5.1 and 7.6.1. That is good. For section 7.5.4 and subframe timing text, I prefer this CR, when we merge CRs. There are no values in this CR, just new definitions. |
|  | Moderator: It is suggested to revise this CR to capture changes in 2683/2900/2985. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: Timing Advance adjustment accuracy # companies comment in 1st round. 1 company support option 1 and 3 companies support option 2. Tentative agreements: NA Candidate options: - Option 1: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±12 Tc]; [±8 Tc]. - Option 2: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±8 Tc]; [±4 Tc]. Recommendations for 2nd round: Continue discussion in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 2-1  Timing Advance adjustment accuracy

Background:  Way forward in RAN4#103-e R4-2210590

Way forward: (To be handled during maintenance phase) The UE timing advance adjustment accuracy for 480/960kHz SCS is defined as shown in the table below: - Option 1: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±12 Tc]; [±8 Tc]. - Option 2: UL Sub Carrier Spacing(kHz); 15; 30; 60; 120; 480; 960. UE Timing Advance adjustment accuracy; ±256 Tc; ±256 Tc; ±128 Tc; ±32 Tc; [±8 Tc]; [±4 Tc].

Issue 2-1-1: Timing Advance adjustment accuracy

- Proposals

  - Option 1: Option 1 of the UE timing advance adjustment accuracy for 480/960kHz SCS is agreed. (Apple)

- Recommended WF:

  - Discuss the above issue, and try to reach consensus.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: |
| Apple | As said multiple times, Option 2 is a tentative requirement subject to further consideration of implementation. Option 1 is already a compromise and we hope companies can take it and we thus close this issue. |
| Ericsson | Issue 2-1-1: Issue 2-1-1: We prefer option 2, since this is a linear scale from SCS = 120 kHz to SCS = 480 kHz and SCS = 960 kHz. The UL uncertainty budget is already very tight with almost no room for RF channel delay spread. |
| Nokia | We still prefer Option 2. |
| CATT | The same view with Ericsson. |

Topic #3: LBT impact on RRM requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211877 | Apple | RAN4 to adopt Option 2 on how to specify the maximum separation between two consecutive measurements. |
| R4-2212911 | Nokia, Nokia Shanghai Bell | - Proposal 1: RAN4 requirements are defined under the assumption that the UE knows the channel access mechanism used by neighbor cells in RRC_CONNECTED. - Proposal 2: the requirements applies when - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured |
| R4-2212959 | Huawei, HiSilicon | Proposal 2: For connected mode measurement, the requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. Proposal 3: Wait for RAN1/RAN2 input on whether to introduce explicit signalling on channel access mode assumption for neighbour cell. |
| R4-2213024 | vivo | Observation 1: Based on RAN1’s conclusion, UE has no idea to clarify whether the LBT at serving cell side is used even the channel access mode is provided to the UE from the serving cell indicating LBT mode is used. Observation 2: When channel access mode is provided to the UE from serving cell indicating LBT mode is not used, for neighbour cell, it may be feasible that UE can follow the same conclusion with the serving cell, i.e, LBT mode is not used for neighbour cell. When channel access mode is provided to the UE from serving cell indicating LBT mode is used, it is up to gNB implementation whether LBT at gNB side is actually used, UE can assume LBT is always used for neighbour cell. |
| R4-2213350 | Ericsson | Proposal 1: Support Option1. The requirement applies provided any two measurements shall not be separated in time by more than the maximum time requirement for the cell to remain known. |

Open issues summary

Sub-topic 3-1 maximum separation between two consecutive measurements

Issue 3-1-1:

- Proposals

  - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. (Huawei, Ericsson)

  - Option 2: (Apple, Nokia)

The requirement only applies when

- Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and

- Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured.

- Recommended WF

  - Discuss above option.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: |
| Huawei: | Fine with option 1 or 2. |
| Ericsson | Issue 3-1-1: We slightly prefer Option 1. |
| Nokia | We can compromise to the view of the majority in this issue. |
| CATT | Prefer Option 1. |
| vivo | We prefer Option 1. We can follow the similar rule with NR-U. |

Sub-topic 3-2 LBT assumptions on neighbour cell measurement

Issue 3-2-1:

- Proposals

  - Option 1: RAN4 requirements are defined under the assumption that the UE knows the channel access mechanism used by neighbour cells in RRC_CONNECTED. (Nokia)

  - Option 2: Wait for RAN1/RAN2 input on whether to introduce explicit signalling on channel access mode assumption for neighbour cell (Huawei)

- Recommended WF

  - Discuss above options.

Comments collection

| Company | Comments |
| --- | --- |
| LGE | Issue 3-2-1: Prefer option 2. RAN1 will discuss this issue in August meeting. |
| MTK | Fine with Option 2. On Option 1, UE may not know the LBT status of neighboring cells if NW doesn't provide the information. So Option 1 may not the proper assumption. |
| Huawei | We would like to slightly modify option 1. “…under the assumptions that UE knows the channel access assumption of neighbour cell” As explained by companies, whether LBT is actually used may up to gNB implementation. UE only needs to the known the LBT assumption of neighbour cell. And the RRM requirements for carrier with CCA apply when the channel access assumption of the carrier is “LBT”. Regarding how UE can know the assumption, RAN4 shall wait more input from RAN1 and RAN2. |
| Ericsson | Issue 3-2-1: LS is sent out. We shall wait for feedback. |
| Nokia | We can agree to option 2. |
| Qualcomm | Option 2 |
| CATT | Option 2, as Huawei explained, how UE can know the the channel access mechanism used by neighbour cells should wait for RAN1/RAN2 input. |
| vivo | We agree with Option 2. Further wait for RAN1/RAN2’s outcome. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212390 (Nokia, Nokia Shanghai Bell) CR on Intra-frequency measurement requirements with CCA in FR2-2 | Company A |
|  | Company B |
|  | Huawei: For RSSI parts, it depends on the conclusion of pending issues. |
|  | Moderator: It is suggested to revise the CR and changes on RSSI can be merged into 2216. |
| R4-2212960 (Huawei, HiSilicon) Draft CR on RSSI requirements of FR2-2 | Nokia: this CR depends on open issues. |
|  | Company B |
|  | Moderator: It is suggested to revise the CR and changes on RSSI QCL assumption can be merged into 2216. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: maximum separation between two consecutive measurements 5 companies comment in 1st round. Option 1 seems acceptable to most companies. Apple still prefer option 2. Tentative agreements: NA Candidate options: - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known. - Option 2: The requirement only applies when - Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and - Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured. Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic#3-2 | Issue 3-2-1: LBT assumptions on neighbour cell measurement 6 companies comment in 1st round and most companies agree to wait for the feedback from RAN1/2. Tentative agreements: Wait for RAN1/RAN2 input on whether to introduce explicit signalling on channel access mode assumption for neighbour cell Candidate options: NA Recommendations for 2nd round: NA |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Sub-topic 3-1 maximum separation between two consecutive measurements

Issue 3-1-1:

- Proposals

  - Option 1: The requirement apply provided any two measurement shall not be separated in time by more than the maximum time requirement for the cell to remain known.

  - Option 2:

The requirement only applies when

- Within the set of measurements any two measurements shall not be separated in time by more than 2 seconds when no DRX is configured, and

- Within the set of measurements any two measurements shall not be separated in time by more than max(1 DRX occasion group duration, 2 seconds) when DRX is configured.

- Recommended WF

  - Discuss above option.

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: |
| Apple | As discussed in our contribution, “for FR2-2, even the cell remains known, if the UE RX beam is not aligned with the BS TX beam, the UE still cannot perform measurements properly. Therefore, the physical meaning of “the maximum time requirement for the cell to remain known” is not straightforward to understand and thus its applicability to FR2-2 is at least unclear.” Can proponents of Option 1 clarify? |
| Ericsson | we support Option 1, just a reminder is that we shall list the exact number or clear reference to the number. |
| CATT | Prefer Option 1 to follow the similar rule with NR-U. And we just propose a limit on the measurement time interval. |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR extension to 71 GHz RRM requirements (Part 1) | Huawei, HiSilicon | Capture agreement in this meeting. |
|  | LS reply on TCI assumption for RSSI measurement for FR2-2 | Apple | To: RAN1 |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2212216 |  | CR on QCL-ed assumption for RSSI measurement in FR2-2 | LG Electronics Inc. | Revised | Capture QCL assumptions for RSSI measurement |
| R4-2212403 |  | CR on TS38.133 RRM requirements in FR2-2 | MediaTek inc. | Revised | Changes on RSSI QCL assumption will be merged into R4-2212216 |
| R4-2212682 |  | Draft CR Measurement procedure updates for FR2-2 | Nokia, Nokia Shanghai Bell | Revised | Merge the changes on clause 9 in R4-2213025 |
| R4-2212957 |  | Draft CR on beam sweeping factor for RRM requirements of FR2-2 | Huawei, HiSilicon | Revised | Capture comments in 1st round |
| R4-2213025 |  | CR on introduction of cell Re-selection, inter-frequency and intra-frequency measurments requirements for FR2-2 | vivo | Revised | Capture changes in clause 4. Changes on clause 9 will be merged into R4-2212682 |
| R4-2214077 |  | Draft CR for RSSI measurements for FR2-2 | Qualcomm Incorporated | Merged | Be merged into R4-2212216 |
| R4-2214215 (revised from R4-2212214) |  | LS on Rx beam sweeping factor for RRM measurements | LG Electronics Inc. | Agreeable |  |
| R4-2211875 |  | Draft CR on UE timing advance adjustment accuracy for Rel-17 NR extension to 71GHz | Apple | Return to |  |
| R4-2212899 |  | Draft CR Timing RRM requirements for FR2-2 | Ericsson | Merged | Be merged into R4-2214078 |
| R4-2212958 |  | Draft CR on timing requirements of FR2-2 | Huawei, HiSilicon | Merged | Be merged into R4-2214078 |
| R4-2214078 |  | Draft CR for timing requirements for FR2-2 – MRTD-MTTD | Qualcomm Incorporated | Revised | Capture changes on MTTD and MRTD |
| R4-2212390 |  | CR on Intra-frequency measurement requirements with CCA in FR2-2 | Nokia, Nokia Shanghai Bell | Revised | Changes on RSSI QCL assumption will be merged into R4-2212216 |
| R4-2212960 |  | R4-2212960 | Huawei, HiSilicon | Revised | Changes on RSSI QCL assumption will be merged into R4-2212216 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214476 |  | WF on NR extension to 71 GHz RRM requirements (Part 1) | Huawei, HiSilicon | Return to |  |
| R4-2214477 |  | LS reply on TCI assumption for RSSI measurement for FR2-2on | Apple | Agreeable |  |
| R4-2211875 |  | Draft CR on UE timing advance adjustment accuracy for Rel-17 NR extension to 71GHz | Apple | Return to |  |
| R4-2212216 | R4-2214577 | CR on QCL-ed assumption for RSSI measurement in FR2-2 | LG Electronics Inc. | Agreeable |  |
| R4-2212390 | R4-2214584 | CR on Intra-frequency measurement requirements with CCA in FR2-2 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2212403 | R4-2214588 | CR on TS38.133 RRM requirements in FR2-2 | MediaTek inc. | Agreeable |  |
| R4-2212682 | R4-2214971 | Draft CR Measurement procedure updates for FR2-2 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2212957 | R4-2214982 | Draft CR on beam sweeping factor for RRM requirements of FR2-2 | Huawei, HiSilicon | Agreeable |  |
| R4-2212960 | R4-2214983 | Draft CR on RSSI requirements of FR2-2 | Huawei, HiSilicon | Agreeable |  |
| R4-2213025 | R4-2214612 | CR on introduction of cell Re-selection, inter-frequency and intra-frequency measurements requirements for FR2-2 | vivo | Agreeable |  |
| R4-2214078 | R4-2215104 | Draft CR for timing requirements for FR2-2 – MRTD-MTTD | Qualcomm Incorporated | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents