3GPP TSG-RAN WG4 Meeting # 104-e												        R4-2214273

Electronic Meeting, 15 - 26 August 2022

Agenda item:			9.17.5

Source:	Moderator (Samsung)

Title:	Email discussion summary for [104-e][222] NR_feMIMO_RRM_2

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Rel-17 NR feMIMO is a RAN1 leading WI, in which the following items are identified for having RAN4 RRM requirement impact, based on previous RAN4 discussion:

- Unified TCI state switching

- L1-RSRP measurement for inter-cell beam management

- TRP specific beam failure recovery

In RAN4 meeting #102-e, main tasks within the RRM core work scope have completed. In the last RAN4 meeting #103-e, RAN4 has started to discuss on RRM performance requirements and test cases. In the subsequent meetings, discussion will focus on the feMIMO RRM performance requirement of the above aspects for Release-17, following the WF R4-2210617.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

As the rapporteur company for Rel-17 MIMO further enhancement WI, we would like to suggest the following candidate target of 1st and 2nd round email discussion:

- 1st round: Collect more views on all topics and to get progress as much as possible:

- 2nd round: Based on results from 1st round, complete outstanding issues and reach the consensus on test case CRs and the WF.

This discussion summary includes the topics for FeMIMO RRM performance requirements in [104-e][222] NR_feMIMO_RRM_2, AI 9.17.3 of RAN4#104e meeting. R4-2214142 is the 1st round discussion summary. R4-2214273 is the 2nd round discussion summary.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung (Moderator) | Yiyan Zhang | yiyan.zhang@samsung.com |
| MediaTek | Chih-Kai Yang | ck.yang@mediatek.com |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Unified TCI State Switching

Main technical topic overview. The structure can be done based on sub-agenda basis.

Discussion on the test cases for unified TCI state switching

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2213175 | Samsung | Observation 1: The following test cases for Rel-17 unified TCI state switching can be introduced. Test Case; Scenario; Key Configurations. Unified TCI state switching; 1; EN-DC + FR2 + Serving Cell; Joint TCI + not in the active list. 2; EN-DC + FR2 + Serving Cell; UL TCI + PL-RS is not maintained. 3; EN-DC + FR2 + Cell with different PCI; DL TCI + not in the active list. 4; NR SA + FR2 + Serving Cell; Joint TCI + not in the active list. 5; NR SA + FR2 + Serving Cell; UL TCI + PL-RS is not maintained. 6; NR SA + FR2 + Cell with different PCI; DL TCI + not in the active list. |
| R4-2213487 | Huawei, HiSilicon | Proposal 1: For R17 unified TCI state switch testing, it is suggested only to test MAC-CE based beam indication. Proposal 2: The existing test method of MAC-CE based active TCI state switching in R15/R16 can be used as reference for R17 DL/joint TCI state switching testing. Proposal 3: The existing test method of MAC-CE based UL spatial relation switching in R15/R16 can be used as reference for R17 UL TCI state switching testing. Proposal 4: New TCI configurations for R17 unified TCI shall be introduced for defining R17 unified TCI switching delay tests. Proposal 5: The TCI configurations for R17 unified TCI switching delay tests can be defined as Table 1. Table 1: TCI configurations for unified TCI state switching delay test Test case; TCI state 0; TCI state 1. DL TCI state switching associated with different PCI; DL TCI state 0: referenceSignal is CSI-RS which is QCL’d to SSB0. SSB0 is associated with serving PCI; DL TCI state 1: referenceSignal is CSI-RS which is QCL’d to SSB1. SSB1 is associated with non-serving PCI.. Joint TCI state switching associated with serving PCI; Joint TCI state 0: referenceSignal is CSI-RS which is QCL’d to SSB0. SSB0 is associated with serving PCI. PL-RS is same CSI-RS for referenceSignal.; Joint TCI state 1: referenceSignal is CSI-RS which is QCL’d to SSB1. SSB0 is associated with serving PCI. PL-RS is same CSI-RS for referenceSignal.. UL TCI state switching associated with serving PCI; UL TCI state 0: referenceSignal is SSB0. SSB0 is associated with serving PCI PL-RS is CSI-RS which is QCL’d to SSB0.; UL TCI state 1: referenceSignal is SSB1. SSB1 is associated with serving PCI. PL-RS is CSI-RS which is QCL’d to SSB1.. Proposal 6: The parameter ‘unifiedtci-StateType’ shall be configured to serving cell in R17 unified TCI state switching tests, where unifiedtci-StateType is indicated as ‘SeparateULDL’ for DL TCI and UL TCI switching tests and indicated as ‘JointULDL’ for joint TCI switching tests. |
| R4-2213174 | Samsung | DraftCR, introduce RRM test configurations for FeMIMO test, including UL TCI and CSI-RS |
| R4-2213488 | Huawei, HiSilicon | DraftCR, introducing unified TCI configuration for the test cases |
| R4-2213885 | ZTE Corporation | DraftCR, introducing test case of unified TCI state switching for EN-DC joint TCI |
| R4-2213173 | Samsung | DraftCR, introducing test case of unified TCI state switching for EN-DC UL TCI |
| R4-2211861 | Apple | DraftCR, introducing test case of unified TCI state switching for EN-DC DL TCI from cell with different PCI |
| R4-2212518 | MediaTek Inc. | DraftCR, introducing test case of unified TCI state switching for NR-SA joint TCI |
| R4-2213946 | Ericsson | DraftCR, introducing test case of unified TCI state switching for NR-SA UL TCI |
| R4-2212669 | vivo | DraftCR, introducing test case of unified TCI state switching for NR-SA DL TCI from cell with different PCI |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1

Test procedure for unified TCI state switching

Sub-topic description: discussion on the test procedures

Open issues and candidate options before e-meeting:

Issue 1-1-1: Ending point for DL TCI state switching test case

- Proposals:

  - Option 1: Follow the same logic of R16 TCI state switching test case

    - continue to receive on TCI state 0 till xxx and start receiving on TCI state 1 after yyy

  - Option 2: other solution

- Recommended WF

  - If no better solution, Option 1 is preferred.

Issue 1-1-2: Ending point for UL TCI state switching test case

- Proposals

  - Option 1: Follow the same logic of R16 uplink spatial test case, i.e.

    - continue to transmit PUCCH on UL TCI state 0 before xxx and start transmitting PUCCH on UL TCI state 1 from yyy

  - Option 2: In the test only check the core requirement, i.e.

    - start transmitting PUCCH on UL TCI state 1 from yyy

  - Option 3: Other solution

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-1-3: Time T1 in UL and Joint TCI test case

- Proposals:

  - Option 1: Follow the same logic of R16 test case

    - T1 = 0.2s and T2 = 2s for L1-RSRP report

  - Option 2: Longer T1 is set in the test case for UE report L1-RSRP measurement results before T2

    - e.g.T1 = 2s

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-1-4: Test requirement in Joint TCI test case

- Proposals:

  - Option 1: Separate DL/UL requirements in the joint TCI test case

  - Option 2: unified DL/UL requirement in the joint TCI test case, through max(DL TCI state switch delay, UL TCI state switch delay)

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-1-5: When to start transmitting SSB1 in the UL and Joint TCI test case

- Proposals

  - Option 1: At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting.

  - Option 2: Before the beginning of the test case

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

  - Depending on issue 1-1-3

Sub-topic 1-2

Test configuration for unified TCI state switching

Sub-topic description: discussion on the test configurations

Open issues and candidate options before e-meeting:

Issue 1-2-1: Pathloss RS in joint TCI test case

- Proposals

  - Option 1: Configure Pathloss RS in joint TCI case

    - Option 1a: Pathloss RS is maintained

    - Option 1b: Pathloss RS is not maintained

  - Option 2: Do not configure Pathloss RS in joint TCI case

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-2: Pathloss RS in UL (and joint TCI) test case

- Proposals

  - Option 1: Use CSI-RS as the Pathloss RS in the test case, and the CSI-RS is QCLed Type-D with the corresponding SSB

    - Option 1a: Reuse existing CSI-RS configuration in 38.133

    - Option 1b: Introduce new configuration of CSI-RS for pathloss RS (Periodic CSI-RS with repetition on)

  - Option 2: Use the same SSB as Pathloss RS

  - Option 3: other solution

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-3: Introduce new unified TCI state configurations

- Proposals

  - Option 1: Introduce separate DL, UL and joint TCI state configuration in unified TCI configuration section

  - Option 2: Introduce unified TCI state configuration in existing TCI configuration section

  - Option 3: No need for new DL TCI state

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-4: Data transmission on cell with different PCI in the test case

- Proposals

  - Option 1: No DL/UL data transmission to cell with different PCI during the test, and measurement only

  - Option 2: DL/UL data transmission to both serving cell and cell with different PCI during the test

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 1-2-5: Introduce new RS configuration for cell with different PCI in the test case

- Proposals

  - Option 1: Introduce new TRS and DL TCI state for cell with different PCI in the test case

  - Option 2: No need to introduce more configuration, and reuse existing ones

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Sub-topic 1-3

Structure for drafting test case

Sub-topic description: test case section structure

Open issues and candidate options before e-meeting:

Issue 1-3-1: subsections for unified TCI state switching test case

- Proposals

  - Option 1: Two subsections, 1 Test Purpose and Environment and 2 Test Requirements

  - Option 2: Three subsections, 1 Test Purpose and Environment, 2  Test parameters and 3 Test Requirements

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

  - It is moderator suggestion that using the style as option 2.

Companies views’ collection for 1st round

Open issues

Sub-topic 1-1 Test procedure for unified TCI state switching

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1-1: Option 1 is fine for us Issue 1-1-2: Option 1 is fine for us Issue 1-1-4: Option 2 is fine for us. |
| Intel | Issue 1-1-1: Option 1. Issue 1-1-2: No obvious difference between option 1 and 2. Fine with both. Issue 1-1-3: Option 1. Issue 1-1-4: Option 2. |
| MediaTek | Issue 1-1-1: support Option 1 Issue 1-1-2: support Option 1 Issue 1-1-3: more discussion is needed. one question: the unit is (ms) or (s)? Issue 1-1-4: support option 2. The UL TCI state should be stable to transmit ACK/NACK for DL TCI state. Issue 1-1-5: either way is fine for us. |
| Apple | Issue 1-1-1: Option 1 Issue 1-1-2: Option 1 Issue 1-1-3: Need to further check. Assuming time is in s not ms. Issue 1-1-4: Option 2, with switching delay max(DL, UL) switching delay. Issue 1-1-5: Option 1, similar to DL TCI state switching. |
| Samsung | Issue 1-1-1: Ending point for DL TCI state switching test case Prefer Option 1 as the ending point and the requirement to be test for the test cases. Issue 1-1-2: Ending point for UL TCI state switching test case Prefer Option 1. Issue 1-1-3: Time T1 in UL and Joint TCI test case Both options are fine. The test procedure should be revised accordingly. Issue 1-1-4: Test requirement in Joint TCI test case It may be based on the issue in core part discussion. If “In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch” is kept, option 2 seems to be more reasonable. Issue 1-1-5: When to start transmitting SSB1 in the UL and Joint TCI test case Depending on issue 1-1-3. |
| Ericsson | Issue 1-1-1: Ending point for DL TCI state switching test case Option 1 is fine. Issue 1-1-2: Ending point for UL TCI state switching test case Fine with both option 1. Issue 1-1-3: Time T1 in UL and Joint TCI test case We are fine with Option 1. Of course legacy is in Sec. Issue 1-1-4: Test requirement in Joint TCI test case We are fine with option 2. Issue 1-1-5: When to start transmitting SSB1 in the UL and Joint TCI test case Option 1 similar to legacy UL spatial relation info switching. |
| vivo | Issue 1-1-1: Option 1 Issue 1-1-2: Option 1 Issue 1-1-4: Option 2 Issue 1-1-5: both are fine |
| Moderator | Issue 1-1-1: Ending point for DL TCI state switching test case - Proposals: - Option 1: Follow the same logic of R16 TCI state switching test case (Huawei, Intel, MediaTek, Apple, Samsung, Ericsson, vivo) - continue to receive on TCI state 0 till xxx and start receiving on TCI state 1 after yyy - Option 2: other solution Issue 1-1-2: Ending point for UL TCI state switching test case - Proposals - Option 1: Follow the same logic of R16 uplink spatial test case, i.e. (Huawei, MediaTek, Apple, Samsung, Ericsson, vivo) - continue to transmit PUCCH on UL TCI state 0 before xxx and start transmitting PUCCH on UL TCI state 1 from yyy - Option 2: In the test only check the core requirement, i.e. - start transmitting PUCCH on UL TCI state 1 from yyy - Option 3: Other solution Issue 1-1-3: Time T1 in UL and Joint TCI test case - Proposals: - Option 1: Follow the same logic of R16 test case (Intel, Ericsson) - T1 = 0.2s and T2 = 2s for L1-RSRP report - Option 2: Longer T1 is set in the test case for UE report L1-RSRP measurement results before T2 - e.g.T1 = 2s Issue 1-1-4: Test requirement in Joint TCI test case - Proposals: - Option 1: Separate DL/UL requirements in the joint TCI test case - Option 2: unified DL/UL requirement in the joint TCI test case, through max(DL TCI state switch delay, UL TCI state switch delay) (Huawei, Intel, MediaTek, Apple, Samsung, Ericsson, vivo) Issue 1-1-5: When to start transmitting SSB1 in the UL and Joint TCI test case - Proposals - Option 1: At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting.(Apple, Ericsson) - Option 2: Before the beginning of the test case |

Sub-topic 1-2 Test configuration for unified TCI state switching

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-2-1: Prefer option 1a. Issue 1-2-2: Option 1, following GTW agreements Issue 1-2-3: Option 1, following GTW agreements Issue 1-2-4: Following GTW agreements |
| Intel | Issue 1-2-1: Both option 1 or option 2 are fine. Similar with UL spatial relation switch, no PL-RS is configured. Issue 1-2-2: Option 1. For option 1a, the legacy CSI-RS configuration is used for known TCI state test case. While for option 1b, it can used for unknown TCI state test case where Rx beam sweeping is needed. Issue 1-2-3: Agreed in GTW discussion. Option 1. Issue 1-2-4: Agreed in GTW discussion. For DL TCI state switching test case(s), data transmission is based on TCI state which is QCLed with the SSB from cells. |
| MediaTek | Issue 1-2-1: prefer option 2. The reason is because the PL-RS has been tested in R16 eMIMO. Issue 1-2-2: follow GTW agreement Issue 1-2-3: follow GTW agreement Issue 1-2-4: follow GTW agreement Issue 1-2-5: prefer option 1 to make the requirement clear. |
| Apple | Issue 1-2-1: Support Option 1a. We should configure PL-RS. IN R16 UL SpRelInfo switch testcase PL-RS is configured for PUCCH. Issue 1-2-2: follow GTW agreement Issue 1-2-3: follow GTW agreement Issue 1-2-4: follow GTW agreement Issue 1-2-5: In our understanding only SSB from cell with diff PCI needs to be configured for additional cell. The other signals are QCLed to it. |
| Samsung | Issue 1-2-1: Pathloss RS in joint TCI test case Prefer option 2. First, for UL TCI state switching and Joint TCI switching, pathloss RS is an optional field in TCI signalling. Second, PL RS switching has already been configured in UL TCI state switching test case. Thus, PL-RS configured in Joint TCI test case is not necessary. Issue 1-2-2: Pathloss RS in UL (and joint TCI) test case For FR2 pathloss RS measurement, the not maintained PL-RS needs to be measured for L1-RSRP report. Due to the demand of FR2 beam sweeping, CSI-RS resource set with repetition on could guarantee its L1-RSRP measurement procedure can be done on time in any circumstance. In this context, repetition on CSI-RS resource set is preferred for PL-RS here. As we checked the existing CSI-RS configurations, no one is qualified. So one more configuration is desired. And Note that the purpose of the test case is to test whether TCI switching can be done accordance with the requirement, not to test whether PL-RS is switched. Therefore, the configuration could be made favourable for configuring/switching PL-RS, to the largest extent, but no need to verify it. And if CSI-RS is configured as PL-RS, when to start L1 measurement on the CSI-RS in the test procedure is another issue as currently no definition for “maintained” or “not maintained”. Issue 1-2-3: Introduce new unified TCI state configurations As agreed, introduce separate DL, UL and joint TCI state configuration in unified TCI configuration section. Issue 1-2-4: Data transmission on cell with different PCI in the test case  For DL TCI state switching test case(s), data transmission is based on TCI state which is QCLed with the SSB from cells with different PCIs. That is to say, for SSB0, data is transmitted between UE and SC; after TCI switching, (at least DL) data is transmitted from cell with different PCI to UE. The test case will be revised accordingly. |
| Ericsson | Issue 1-2-1: Pathloss RS in joint TCI test case We support option 1. For known TCI state, we could test the PL-RS maintained. Issue 1-2-5: Introduce new RS configuration for cell with different PCI in the test case Do not understand the need for new RS configuration. |
| vivo | Issue 1-2-1: FFS. Need more time to check Issue 1-2-5: Wording is not clear. Same view as Apple. Option 1 should be ‘Introduce new TRS and DL TCI state that are QCLed to the SSB from cell with different PCI in the test case’ |
| Moderator | Issue 1-2-1: Pathloss RS in joint TCI test case - Proposals - Option 1: Configure Pathloss RS in joint TCI case - Option 1a: Pathloss RS is maintained (Huawei, Apple, Ericsson) - Option 1b: Pathloss RS is not maintained - Option 2: Do not configure Pathloss RS in joint TCI case (MediaTek, Samsung) Issue 1-2-2: Pathloss RS in UL (and joint TCI) test case - Proposals - Option 1: Use CSI-RS as the Pathloss RS in the test case, and the CSI-RS is QCLed Type-D with the corresponding SSB (Huawei, Intel, MediaTek, Apple, ) - Option 1a: Reuse existing CSI-RS configuration in 38.133 - Option 1b: Introduce new configuration of CSI-RS for pathloss RS (Periodic CSI-RS with repetition on) (Samsung, ) - Option 2: Use the same SSB as Pathloss RS - Option 3: other solution Issue 1-2-3: Introduce new unified TCI state configurations - Proposals - Option 1: Introduce separate DL, UL and joint TCI state configuration in unified TCI configuration section - Option 2: Introduce unified TCI state configuration in existing TCI configuration section - Option 3: No need for new DL TCI state Issue 1-2-4: Data transmission on cell with different PCI in the test case - Proposals - Option 1: No DL/UL data transmission to cell with different PCI during the test, and measurement only - Option 2: DL/UL data transmission to both serving cell and cell with different PCI during the test Issue 1-2-5: Introduce new RS configuration for cell with different PCI in the test case - Proposals - Option 1: Introduce new TRS and DL TCI state for cell with different PCI in the test case (MediaTek, vivo) - Option 2: No need to introduce more configuration, and reuse existing ones |

Sub-topic 1-3 Structure for drafting test case

| Company | Comments |
| --- | --- |
| Intel | Issue 1-3-1: Prefer option 2. |
| MediaTek | Issue 1-3-1: prefer option 1 to follow legacy requirement structure. |
| Apple | We support option 1 to follow existing structure. |
| Samsung | Issue 1-3-1: subsections for unified TCI state switching test case Prefer option 2. 3 sub-sections for the test case drafting. Current structure cannot properly describe each part of the test case. |
| Ericsson | We do not have strong view. We are fine with both options. |
| vivo | No strong view. |
| Moderator | Issue 1-3-1: subsections for unified TCI state switching test case - Proposals - Option 1: Two subsections, 1 Test Purpose and Environment and 2 Test Requirements (MediaTek, Apple) - Option 2: Three subsections, 1 Test Purpose and Environment, 2 Test parameters and 3 Test Requirements (Intel, Samsung) |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213174 Samsung | Moderator: DraftCR, introduce RRM test configurations for FeMIMO test, including UL TCI and CSI-RS |
|  | Samsung: Based on the conclusion of open issues, can be merged into R4-2213488 |
| R4-2213488 Huawei, HiSilicon | Moderator: DraftCR, introducing unified TCI configuration for the test cases |
|  | Samsung: Based on the conclusion of open issues, new CSI-RS configuration may be needed. |
| R4-2213885 ZTE Corporation | Moderator: DraftCR, introducing test case of unified TCI state switching for EN-DC joint TCI |
|  | Samsung: For test requirement, whether separately test DL and UL, or test the longer one in the test case, denpends on the conclusion of open issue. |
| R4-2213173 Samsung | Moderator: DraftCR, introducing test case of unified TCI state switching for EN-DC UL TCI |
|  | Samsung: Revise the test procedure according to agreed procedure, including issues period of T1, T2, and when to transmit SSB1. |
| R4-2211861 Apple | Moderator: DraftCR, introducing test case of unified TCI state switching for EN-DC DL TCI from cell with different PCI |
|  | Samsung: Based on the GTW agreements, downlink data transmission could be configured for cell with different PCI after the beginning of T2. n+ THARQ +3 ms => “n+ THARQ + (3ms / NR slot length)” or “n+ THARQ +24” or “n+ THARQ + 3Nslotsubframe,µ” |
| R4-2212518 MediaTek Inc. | Moderator: DraftCR, introducing test case of unified TCI state switching for NR-SA joint TCI |
|  | Samsung: In the test, PL-RS is defined as SSB but it may cause unexpect longer delay for PL-RS switching. As discussed in GTW, CSI-RS serves as PL-RS in the test case. In our understanding “The rate of correct events observed during repeated tests shall be at least [90]%” is not perfered in TCI state switching test, here the delay time can be hardly statistical. So we prefer to remove the sentence. |
| R4-2213946 Ericsson | Moderator: DraftCR, introducing test case of unified TCI state switching for NR-SA UL TCI |
|  | Samsung: In core requirement, THARQ can be specified in slot, not ms, if the CR is agreed. For PL-RS, based on GTW discussion, CSI-RS is configured as the RS. And in the test, UE should also report L1-RSRP measurement for the CSI-RS. |
| R4-2212669 vivo | Moderator: DraftCR, introducing test case of unified TCI state switching for NR-SA DL TCI from cell with different PCI |
|  | Samsung: New configuration for unified TCI can be merged into Huawei’s CR. [vivo] Fine to merge CR. In our understanding, in this test, TCI state switching is from a serving cell to a cell with different PCI. In vivo’s CR, TRS in Cell1 is QCL type-D with SSB0 of Cell2, whether to use TRS depends on conclusion of related open issue. [vivo] As in RAN1 spec TS 38.214, For the DM-RS of PDCCH, the UE shall expect that a TCI-State or DLorJointTCIState except an indicated DLorJointTCIState indicates one of the following quasi co-location type(s): - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or - 'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource. In our understanding, using TRS would be the easist way. In vivo’s CR, we suggest using SSB0 of Cell 1 and SSB1 of Cell 2, instead of two SSB0 to aviod ambiguity. In OTA parameter TABLE, two SSBs are the same. [vivo] OK |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Ending point for DL TCI state switching test case Tentative agreements: Follow the same logic of R16 test case. Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 1-1-2: Ending point for UL TCI state switching test case Tentative agreements: Follow the same logic of R16 uplink spatial test case Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 1-1-3: Time T1 in UL and Joint TCI test case Tentative agreements: Follow the same logic of R16 test case, T1 = 0.2s and T2 = 2s Candidate options: - Option 1: T1 = 0.2s and T2 = 2s - Option 2: T1 = 2s and T2 = 2s Recommendations for 2nd round: If no further comments in 2nd round, go option 1. Issue 1-1-4: Test requirement in Joint TCI test case Tentative agreements: unified DL/UL requirement in the joint TCI test case, through max(DL TCI state switch delay, UL TCI state switch delay) Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 1-1-5: When to start transmitting SSB1 in the UL and Joint TCI test case Tentative agreements: Follow the same logic of R16 test case Candidate options: - Option 1: At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting - Option 2: Before the beginning of the test case Recommendations for 2nd round: If no further comments in 2nd round, go option 1. |
| Sub-topic #1-2 | Issue 1-2-1: Pathloss RS in joint TCI test case Tentative agreements: N/A Candidate options: - Option 1: Pathloss RS is maintained - Option 2: Do not configure Pathloss RS in joint TCI case Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. Issue 1-2-2: Pathloss RS in UL (and joint TCI) test case Tentative agreements: Follow the GTW agreement. Candidate options: - Option 1a: Reuse existing CSI-RS configuration in 38.133 - Option 1b: Introduce new configuration of CSI-RS for pathloss RS (Periodic CSI-RS with repetition on) Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. Issue 1-2-3: Introduce new unified TCI state configurations Tentative agreements: Follow the GTW agreement. Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 1-2-4: Data transmission on cell with different PCI in the test case Tentative agreements: Follow the GTW agreement. Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 1-2-5: Introduce new RS configuration for cell with different PCI in the test case Tentative agreements: N/A Candidate options: - Option 1: Introduce new TRS and DL TCI state that are QCLed to the SSB from cell with different PCI in the test case - Option 2: No need to configure a TRS for serving cell that is QCLed type-D with the SSB from cell with different PCI in the test case Recommendations for 2nd round: |
| Sub-topic #1-3 | Issue 1-3-1: subsections for unified TCI state switching test case Tentative agreements: Three subsections, 1 Test Purpose and Environment, 2 Test parameters and 3 Test Requirements Candidate options: - Option 1: Two subsections, 1 Test Purpose and Environment and 2 Test Requirements - Option 2: Three subsections, 1 Test Purpose and Environment, 2 Test parameters and 3 Test Requirements Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. From moderator’s view, Option 2 is clearer for test case presentation. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| --- | --- |
| R4-2213174 Samsung | To be merged. Merged into R4-2213488. |
| R4-2213488 Huawei, HiSilicon | To be revised. Based on 2nd round discussion, a new CSI-RS and a new TRS might be needed. For the unified TCI configurations, need to further check. |
| R4-2213885 ZTE Corporation | To be revised. According to 1st round summary, the test requirement for Joint TCI case should be joint max(UL,DL). |
| R4-2213173 Samsung | To be revised. According to 1st round opinions, following the same procedure as legacy test cases. |
| R4-2211861 Apple | To be revised. Considering how to configure data transmission for NSC case. |
| R4-2212518 MediaTek Inc. | To be revised. Further consider whether “The rate of correct events observed during repeated tests shall be at least [90]%” is needed. |
| R4-2213946 Ericsson | To be revised. CSI-RS is configured as the RS. Consider how to deal with not maintained PL-RS in test case. |
| R4-2212669 vivo | To be revised. Configuration part can be merged into R4-2213488. Further consider TRS is needed in the test case. |

Discussion on 2nd round (if applicable)

Open issues

Sub-topic 1-1

Issue 1-1-1: Test procedures for unified TCI test case

- Proposals:

  - Option 1: Follow the legacy R16 test case

    - T1 = 0.2s where only SSB0 is transmitted and UE is configured for data transmission which QCLed with SSB0;

    - T2 = 2s where UE is configured L1-RSRP measurement, UE reports measurement results and then TCI state switching MAC-CE is sent.

    - At the beginning of T2, the SSB1 corresponding to TCI state 1 starts transmitting

  - Option 2: other procedures

- Recommended WF

  - Companies’ views are collected in 2nd round discussion. If no other solution, Option 1 can be agreed.

| Company | Comments |
| --- | --- |
| Ericsson | We agree with the Option 1 |
| Apple | Option 1 looks okay to us. If the time periods are not sufficient for some case, we can update them later. |
| MediaTek | Ok to option 1. |
| Intel | Fine with option 1. |
| vivo | OK to option 1. |
| Samsung | Agree with Option 1. |

Sub-topic 1-2

Issue 1-2-1: Pathloss RS in joint TCI test case

- Proposals:

  - Option 1: Pathloss RS is maintained

  - Option 2: Do not configure Pathloss RS in joint TCI case

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

Issue 1-2-2: Configuration of pathloss RS in UL (and joint TCI) test case

- Proposals:

  - Option 1: Reuse existing CSI-RS configuration in 38.133

  - Option 2: Introduce new configuration of CSI-RS for pathloss RS (Periodic CSI-RS with repetition on)

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

  - Proponent of Option 1 please specify which CSI-RS configuration to be used.

Issue 1-2-3: Introduce TRS configuration for cell with different PCI in the test case

- Proposals:

  - Option 1: Introduce a new TRS with DL TCI state that are QCLed to the SSB from cell with different PCI in the test case

  - Option 2: No need to configure a TRS for serving cell that is QCLed type-D with the SSB from cell with different PCI in the test case

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-2-1: Option 1 Issue 1-2-2: Assuming PL-RS is maintained we are fine with option 1 |
| Apple | Issue 1-2-1: Pathloss RS in joint TCI test case Option 1, since we test with PL-RS not maintained in separate Ul TCI testcase. But not sure how to configure maintained PL-RS. Issue 1-2-2: Configuration of pathloss RS in UL (and joint TCI) test case Option 1 is fine. We assume CSI-RS is preferred instead of SSB because of longer delay with SSB based PL-RS measurement is PL-RS is not maintained? Issue 1-2-3: Introduce TRS configuration for cell with different PCI in the test case It is not clear which test case this is needed for. For Dl TCI state switch to cell with diff PLI, we have a target TCI state with additionalPCI configured. Could proponents please clarify? |
| MediaTek | Issue 1-2-1: Pathloss RS in joint TCI test case Slightly prefer option 2. But we are also fine with option 1 if companies think it is necessary. Issue 1-2-2: Configuration of pathloss RS in UL (and joint TCI) test case No strong view Issue 1-2-3: Introduce TRS configuration for cell with different PCI in the test case No strong view |
| Intel | Issue 1-2-1: Prefer Option 2. Similar as legacy uplink spatial info switch, PL-RS is not mandatory to be configured and we don’t know how to configure maintained PL-RS. Issue 1-2-2: for option 2, it can used for unknown TCI state test case where Rx beam sweeping is needed. For option 1, the legacy CSI-RS configuration is used for known TCI state test case. We need to clarify whether the testcase is for known and unknown case first. |
| vivo | Issue 1-2-1: Ok to option 1 Issue 1-2-2: Ok to option 1 Issue 1-2-3: Support option 1. Otherwise it contradicts with RAN1 spec as we cited in the 1st round. [Fix typo by vivo2] |
| Samsung | Issue 1-2-1: Pathloss RS in joint TCI test case Prefer option 2, but either is fine. Currently the problem is how to define a maintained or not maintained PL-RS in the test case. In RAN1 spec, “A UE does not expect to simultaneously maintain more than four pathloss”. We are not sure it is a UE capability or a UE behavior. Also it is not clear who specifies which 4 PL-RSs need to be maintained. Issue 1-2-2: Configuration of pathloss RS in UL (and joint TCI) test case One compromise solution is use repetition off CSI-RS as PL-RSs and the CSI-RS is QCLed type-D with the source SSB of corresponding TCI. In that case, existing CSI-RS may be accepted. Issue 1-2-3: Introduce TRS configuration for cell with different PCI in the test case It seems either way is fine for the test case. |
| Huawei | Issue 1-2-1: We are fine with option 1. Issue 1-2-2: Option 1. The existing CSI-RS for tracking configuration can be reused. Issue 1-2-3: Option 2. No need to introduce new TRS configuration, but to clarify that the source RS in TCI state of TRS is SSB associated to a different PCI. |

Sub-topic 1-3

Issue 1-3-1: subsections for unified TCI state switching test case

- Proposals:

  - Option 1: Two subsections, 1 Test Purpose and Environment and 2 Test Requirements

  - Option 2: Three subsections, 1 Test Purpose and Environment, 2  Test parameters and 3 Test Requirements

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

  - From moderator’s view, option 2 is more feasible for drafting the test case.

Issue 1-3-2: the rate of correct events for unified TCI state switching

- Proposals:

  - Option 1: Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the test case

  - Option 2: No need to specify “90%” threshold for the test case as it is not applicable to TCI state switching test.

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

  - From moderator’s view, option 2 is more feasible for drafting the test case.

Issue 1-3-3: How to configure Maintained PL-RS / NOT maintained PL-RS in the test case

- Proposals:

  - Option 1: Only state the PL-RS is maintained or not in the test case

  - Option 2: Other solutions, e.g. by configuring L1-RSRP measurement on the PL-RS

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-3-1: fine with both options Issue 1-3-2: To be consistent with legacy test cases, we are fine with option 2 Issue 1-3-3: We prefer option 2. We can say that, before sending the TCI switch command, UE is expected to measure on the active TCI list and maintain the PL-RS. |
| Apple | Issue 1-3-1: subsections for unified TCI state switching test case Option2 Issue 1-3-2: the rate of correct events for unified TCI state switching Option 1. It is already used in some (not sure why not all) UL spatial relation or Active DL state switching requirements. Issue 1-3-3: How to configure Maintained PL-RS / NOT maintained PL-RS in the test case For not maintained, the PL-RS in target TCI state is different from the PL-RS in first TCI state. For maintained, it can be configured as the same for both, but seems like artificial setting. Configuring PL-RS for L1-RSRP measurement doesn’t guarantee it as being maintained. |
| MediaTek | Issue 1-3-1: subsections for unified TCI state switching test case Either way is fine to us. Issue 1-3-2: the rate of correct events for unified TCI state switching Prefer option 1 to follow the logic as legacy requirement. |
| Intel | Issue 1-3-1: both are fine. Issue 1-3-2: Fine with option 2. Issue 1-3-3: Not sure how to configure maintain case. Suggest to only configure for non-maintain case. For non-maintain case, PL-RS is configured in TCI state and TCI state is unknown. |
| vivo | Issue 1-3-1: Both are OK, Issue 1-3-2: Option 1 Issue 1-3-3: Option 2. Other solution can be FFS. |
| Samsung | Issue 1-3-1: subsections for unified TCI state switching test case Option2 Issue 1-3-2: the rate of correct events for unified TCI state switching Prefer option 2. We are open to discuss on this issue. The reason why 90% is introduced in some of test case in 133 is that “The delay time is statistical in nature for several reasons, among others that several of the measurements are performed by the UE in a fading radio environment.” However, for this case, we could see it is a comparatively conservative requirements and enough time is left to UE for TCI switching. The variations make a strict limit unsuitable for a test, but for this requirement, there is few variation for switching TCI. Therefore, it is fine to not add “90% rate of correct events” for TCI state switching. Issue 1-3-3: How to configure Maintained PL-RS / NOT maintained PL-RS in the test case Maintained PL-RS is used in RAN1 without definition. In our understanding, we share similar view as Apple comments. If a new RS is configured as a PL-RS, then it is a “not maintained PL-RS”. In UL TCI switching test case, it could be set that “a CSI-RS is transmitted at the beginning of T2” to indicate it is a not maintained PL-RS. |
| Huawei | Issue 1-3-1: no strong view. Issue 1-3-2: prefer option 1. Issue 1-3-3: Whether to maintain PL-RS is up to UE implementation. The value of delay in test requirement can be calculated based on the worst case. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| (Revised from R4-2213488) Huawei, HiSilicon | For DL TCI switch: ‘DLorJoint TCI.State.0’ and ‘DLorJoint TCI.State.1’ are suggested to be used. Both TCI are without PL-RS conffiguration. Source RS in ‘DLorJoint TCI.State.0’ is QCL-TypeD to SSB0. Source RS in ‘DLorJoint TCI.State.1’ is QCL-TypeD to SSB1. For Joint TCI switch: ‘DLorJoint TCI.State.2’ and ‘DLorJoint TCI.State.3’ are suggested to be used. Both TCI are with PL-RS conffiguration. Source RS and PL-RS in ‘DLorJoint TCI.State.2’ are identical and QCL-TypeD to SSB0. Source RS and PL-RS in ‘DLorJoint TCI.State.3’ are identical and QCL-TypeD to SSB1. For UL TCI switch: Option 1: ‘UL TCI.State.0’ and ‘UL TCI.State.0’ are suggested to be used. Source RS in ‘UL TCI.State.0’ is SSB0 and PL-RS in ‘UL TCI.State.0’ is QCL-TypeD to SSB0. Source RS in ‘UL TCI.State.1’ is SSB1 and PL-RS in ‘UL TCI.State.1’ is QCL-TypeD to SSB1. Option 2: ‘UL TCI.State.2’ and ‘UL TCI.State.3’ are suggested to be used. Source RS and PL-RS in ‘UL TCI.State.2’ are identical and QCL-TypeD to SSB0. Source RS and PL-RS in ‘UL TCI.State.3’ are identical and QCL-TypeD to SSB1. |
| (Revised from R4-2213885) ZTE Corporation | Samsung: Test specifications 38.133 => 38.533 It is EN-DC, so no NR PCell (Cell 1), should be NR FR2 PSCell Based on agreement, 3 sub-section structure is used for TC drafting. Joint TCI configuration should follow HW’s latest CR. |
| (Revised from R4-2213173) Samsung |  |
| (Revised from R4-2211861) Apple | Samsung: In the cover page, Test specifications TS 38.521-4. It is not demode test.. Based on agreement, 3 sub-section structure is used for TC drafting. In the test, “UE is configured with 2 different TCI states for PSCell, PDCCH TCI state 0 (QCL’d to SSB0) on Cell 2 and TCIstate 1 (QCL’d to SSB1) on Cell 2 before starting the test.” TCIstate 1 should be on cell with additional PCI (Cell 3), not PSCell (Cell 2). Currently we have an agreement in draft WF: “Configure a TRS for serving cell that is QCLed type-D with the SSB from cell with different PCI in the test” |
| (Revised from R4-2212518) MediaTek Inc. | Samsung: CSI-RS (PLRS) Configuration can be TRS, referring to Huawei’s CR. Repetition on is not needed any more. PL-RS (CSI-RS#0) is maintained or not configured is have not decided. CSI-RS#1 is not needed here. For the requirement, Xms should be convert to slots. |
| (Revised from R4-2213946) Ericsson | Samsung: No change mark in cover page. PL-RS should be not maintained based on last meeting agreement. Based on agreement, 3 sub-section structure is used for TC drafting. “The rate of correct events observed during repeated tests shall be at least [90]%.” can be put in []. CSI-RS (PL-RS) Configuration should follow HW’s latest CR. |
| (Revised from R4-2212669) vivo | Samsung: In the CR, which cell is Cell 2 should be clarified. Based on agreement, 3 sub-section structure is used for TC drafting. |

Topic #2: L1-RSRP measurement for Inter-cell BM

Main technical topic overview. The structure can be done based on sub-agenda basis.

Discussion on the test cases for L1-RSRP measurement on cells with different PCI with serving cell

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2213489 | Huawei, HiSilicon | Proposal 1: The existing test configuration for L1-RSRP measurements on serving cell SSBs can be used as reference for testing L1-RSRP measurements on SSBs with different PCI. Proposal 2: For testing L1-RSRP measurements on SSBs with different PCI, both SSB with serving PCI and SSB with different PCI need to be configured for L1-RSRP measurements. Proposal 3: For testing L1-RSRP measurements on SSBs with different PCI, the SSB resource configured for L1-RSRP measurements shall be clarified which PCI it is associated to. Proposal 4: For testing L1-RSRP measurements on SSBs with different PCI, the expected L1-RSRP measurement period in FR2 is calculated based on sharing factor PSC/CDP=2. |
| R4-2213175 | Samsung | Observation 2: The following test cases for L1-RSRP measurement on cell with different PCI can be introduced. Test Case; Scenario; Key Configurations. L1-RSRP measurement on cells with different PCI; 1; NR SA + FR1; SSB-based + DRX mode. 2; NR FR1-FR2 CA where FR2 Cell is tested; SSB-based + Non-DRX mode. Observation 3: No separate measurement accuracy test case for L1-RSRP measurement on cells with different PCI is needed as the existing test cases are also applied to the case of cells with different PCI. |
| R4-2213884 | ZTE | DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for FR1 NR SA case |
| R4-2212670 | vivo | DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for FR1 NR SA case |
| R4-2213490 | Huawei, HiSilicon | DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for NR SA FR1+FR2 CA case |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1

Test configuration for L1-RSRP measurement on cell with different PCI

Sub-topic description: discussion on the test configurations

Open issues and candidate options before e-meeting:

Issue 2-1-1: Whether configure L1-RSRP on the serving cell in the test case

- Proposals

  - Option 1: No need to configure L1-RSRP on the serving cell

  - Option 2: Configure L1-RSRP measurement on both serving cell and cell with different PCI

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-1-2: SSB configuration for serving cell and cell with different PCI

- Proposals

  - Option 1: the same period for SSBs from serving cell and cell with different PCI, clarify which PCI the SSB is associated to in the test, and the sharing factor  PSC/CDP=2

  - Option 2:Other SSB configurations

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-1-3: CP length configuration in the test case

- Proposals

  - Option 1: Specify the CP length configuration in the test

  - Option 2: No need to specify the CP length configuration

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 2-1-4: AOA setup for tested FR2 cell in the test case

- Proposals

  - Option 1: AoA setup of FR2 cell for the test case is Setup 1

  - Option 2: AoA setup of FR2 cell for the test case is Setup 3, one AoA is for FR2 serving cell, and another is for cell with different PCI.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Companies views’ collection for 1st round

Open issues

Sub topic 2-1 Test configuration for L1-RSRP measurement on cell with different PCI

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-1-1: option 2, follow GTW agreements Issue 2-1-2: option 1 if same or adjacent SSB index are configured for SSB with serving PCI and SSB with additional PCI. Issue 2-1-3: configure the time offset between SSB with serving PCI and SSB with additional PCI. Issue 2-1-4: fine with option 2. |
| Intel | Issue 2-1-1: Agreed in GTW. Option 2. Issue 2-1-2: Option 1. Issue 2-1-3: Option 2. Issue 2-1-4: Option 2. |
| MediaTek | Issue 2-1-1: follow GTW agreements Issue 2-1-2: fine with option 1 Issue 2-1-3: no strong view on this issue. However, we do not see how it impact to performance requirement. Issue: 2-1-4: support option 2. |
| Apple | Issue 2-1-1: Follow GTW agreements Issue 2-1-2: Okay with Option 1. Might need to further check on test setup in FR2. Issue 2-1-3: We don’t understand why we need to specify this. CP length will depend on the SCS used for the test case. We specify multiple configurations in FR1 typically. Issue 2-1-4: Support option 2. |
| Samsung | Issue 2-1-1: Whether configure L1-RSRP on the serving cell in the test case L1-RSRP is configured at both serving cell and non-serving cell, while only requirement on non-serving cell is test. Issue 2-1-2: SSB configuration for serving cell and cell with different PCI Support Option 1. In the test, two L1-RSRP measurement is configured for UE, sharing SSBs for measurements fifty-fifty. Issue 2-1-4: AOA setup for tested FR2 cell in the test case Support option 2. For FR2 test case, the test is to verify measurement from different QCL type-D. This is the most possible case for FR2 inter-cell beam management. |
| Ericsson | Issue 2-1-2: SSB configuration for serving cell and cell with different PCI Fine with option 1. Issue 2-1-3: CP length configuration in the test case Fine with Option 2 Issue 2-1-4: AOA setup for tested FR2 cell in the test case Fine with Option 2 |
| vivo | Issue 2-1-2: Option 1. Sharing factor should be only for FR2. Issue 2-1-3: Option 1. CP length is specified in many other test cases, e.g. RLM and BFR test cases, e.g. in Table A.7.5.1.1.1-2. Since this would impact core part, we slightly prefer to clarify that only normal CP length is tested. Issue 2-1-4: Option 2. |
| Moderator | Issue 2-1-1: Whether configure L1-RSRP on the serving cell in the test case - Proposals - Option 1: No need to configure L1-RSRP on the serving cell - Option 2: Configure L1-RSRP measurement on both serving cell and cell with different PCI Issue 2-1-2: SSB configuration for serving cell and cell with different PCI - Proposals - Option 1: the same period for SSBs from serving cell and cell with different PCI, clarify which PCI the SSB is associated to in the test, and the sharing factor PSC/CDP=2 (Huawei, Intel, MediaTek, Apple, Samsung, Ericsson, vivo) - Option 2:Other SSB configurations Issue 2-1-3: CP length configuration in the test case - Proposals - Option 1: Specify the CP length configuration in the test (vivo) - Option 2: No need to specify the CP length configuration (Intel, Apple, Ericsson, ) Issue 2-1-4: AOA setup for tested FR2 cell in the test case - Proposals - Option 1: AoA setup of FR2 cell for the test case is Setup 1 - Option 2: AoA setup of FR2 cell for the test case is Setup 3, one AoA is for FR2 serving cell, and another is for cell with different PCI. (Huawei, Intel, MediaTek, Apple, Samsung, Ericsson, vivo) |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212670 vivo | Moderator: DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for FR1 NR SA case |
|  | Samsung: In FR1 test, it seems no need to specify the measurement within CP. In FR2 OTA test, it seems to be necessary to define. [vivo] This is the conclusion from core part in previous meetings. RRM requirements are only specified for the known case, and one of the conditions is that timing difference is <= CP. Our understanding is that it also applies to FR1. Samsung: Agree with “also apply to FR1”. Our point is no need to specify in FR1 test. But anyway it is a presentation issue, not a technical one. So we are fine with either case. |
| R4-2213490 Huawei, HiSilicon | Moderator: DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for NR SA FR1+FR2 CA case |
|  | Samsung: In the CR, the AoA setup 1 is configured for different SSB. But in our understanding, for FR2 L1-RSRP measurement on different PCI, two cells are associated with different QCL type-D SSB and UE will measure them in TDM mannar. |
| R4-2213884 ZTE | Moderator: DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for FR1 NR SA case. |
|  | Samsung: the same test case as R4-2212670 and can be merged. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: Whether configure L1-RSRP on the serving cell in the test case Tentative agreements: Follow the GTW agreement. Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 2-1-2: SSB configuration for serving cell and cell with different PCI Tentative agreements: the same period for SSBs from serving cell and cell with different PCI, clarify which PCI the SSB is associated to in the test. Sharing factor = 2 is applied for FR2 test case. Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 2-1-3: CP length configuration in the test case Tentative agreements: N/A Candidate options: - Option 1: Specify the CP length configuration in the test: Normal CP - Option 2: No need to specify the CP length configuration Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. Issue 2-1-4: AOA setup for tested FR2 cell in the test case Tentative agreements: AoA setup of FR2 cell for the test case is Setup 3, one AoA is for FR2 serving cell, and another is for cell with different PCI. Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Revise the CR accordingly. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| --- | --- |
| R4-2212670 vivo | To be revised. |
| R4-2213490 Huawei, HiSilicon | To be revised. AoA setup 3 is configured for FR2. |
| R4-2213884 ZTE | To be merged. Merged into R4-2212670. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Open issues

Sub topic 2-1

Issue 2-1-1: Specify CP length configuration in the test case

- Proposals

  - Option 1: Specify the CP length configuration in the test: Normal CP is used.

  - Option 2: No need to specify the CP length configuration

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-1-1: We do not see big different if normal CP is used. Our understanding is unless otherwise stated assumption is normal CP |
| Apple | Option 1, not sure how it is relevant and why we need to specify. |
| MediaTek | Either way is fine to us. Same question as Apple. |
| Intel | both are fine. |
| vivo | Option 1. To follow some other legacy test cases. |
| Samsung | Either way is fine to us. |
| Huawei | We are fine with not to specify CP length. Our suggestion is to clarify that timing offset between SSB of SC and SSB of CDP is within CP. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| (Revised from R4-2212670) vivo |  |
| (Revised from R4-2213490) Huawei, HiSilicon | Samsung: Text proposal for scenario, procedure and configuration can be further refined, in order to better describing the test case. |

Topic #3: TRP-specific Beam Failure Recovery

Main technical topic overview. The structure can be done based on sub-agenda basis.

Discussion on the test cases for L1-RSRP measurement on cells with different PCI with serving cell

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2213175 | Samsung | Observation 4: The following test cases for TRP-specific beam failure recovery can be introduced. Test Case; Scenario; Key Configurations. TRP-specific beam failure recovery; 1; EN-DC + FR1 + PSCell; SSB based + Non-DRX. 2; EN-DC + FR2 + PSCell; CSI-RS based + DRX. 3; EN-DC + FR1 + SCell; CSI-RS-based BFD and SSB-based LR + Non-DRX. 4; NR SA + FR1 + PCell; CSI-RS based + DRX. 5; NR SA + FR2 + PCell; SSB based + Non-DRX. 6; NR SA + FR2 + SCell; CSI-RS based + DRX. Proposal 1: Introduce the test cases listed in above tables and take the analysis into consideration for designing the test cases for FeMIMO RRM. |
| R4-2212127 | Intel | In this contribution, we provide our views regarding inter-cell beam management in FeMIMO: Observation 1: if BFD-RS (or CBD-RS) resource in one TRP is different from BFD-RS (or CBD-RS) resource in another TRP, there is no difference from legacy from the test perspective. Proposal 1: For TRP specific beam failure detection testcase, suggest to consider the configuration that BFD-RS of two TRPs are overlapped. Proposal 2: For TRP specific beam failure detection testcase, only one TRP will detect beam failure and perform link recovery. SNR level of TRP 2 will always keep at high level. Proposal 3: For TRP specific beam failure detection and link recovery testcase, T2~T5 duration time will be extended due to scaling factor. |
| R4-2213491 | Huawei, HiSilicon | Proposal 1: The existing test configuration for link recovery tests can be used as reference for R17 TRP-specific link recovery tests, where one more pair of BFD-RS and CBD-RS resource sets need to be configured. Proposal 2: For R17 TRP-specific link recovery tests, it is suggested that beam failure occurs on one BFD-RS resource set and the SNR levels of two BFD-RS resource sets can be defined as Table 4. Table 4: SNR levels of BFD-RS for legacy link recovery tests RS; T1; T2; T3; T4; T5. BFD-RS set (q0,0); SNR1; SNR2; SNR3; SNR3; SNR3. BFD-RS set (q0,1); SNR1; SNR1; SNR1; SNR1; SNR1. Proposal 3: The SNR levels of CBD-RS resource in legacy link recovery tests can be reused for both CBD-RS resources in TRP-specific link recovery tests. |
| R4-2213492 | Huawei, HiSilicon | DraftCR, introducing new SSB configuration for TRP specific BFR test case |
| R4-2212918 | Nokia | DraftCR, introducing test case of TRP specific BFR for FR1 EN-DC PSCell case |
| R4-2212919 | Nokia | DraftCR, introducing test case of TRP specific BFR for FR2 EN-DC PSCell case |
| R4-2213947 | Ericsson | DraftCR, introducing test case of TRP specific BFR for FR1 EN-DC SCell case |
| R4-2211862 | Apple | DraftCR, introducing test case of TRP specific BFR for FR1 PCell NR SA case |
| R4-2212123 | Intel | DraftCR, introducing test case of TRP specific BFR for FR2 PCell NR SA case |
| R4-2212124 | Intel | DraftCR, introducing test case of TRP specific BFR for FR1 SCell NR SA case |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1

Test Procedure for TRP specific BFR

Sub-topic description: discussion on the test configurations

Open issues and candidate options before e-meeting:

Issue 3-1-1: The time period of T1-T5 in the test

- Proposals

  - Option 1: For TRP specific beam failure detection and link recovery test case, T2~T5 duration time will be extended due to scaling factor.

| FR | RS type | DRX Config | T1(s) | T2(s) (BFD time +CBD time +margin) | T3(s) (BFD time +margin) | T4(s) | T5(s) (CBD time +margin) | D1(s) (CBD time +margin) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 2.61 = 2*1.6+2*0.96+0.05 | 3.24 = 2*1.6+0.04 | 0 | 1.97 =2*0.96+0.05 | 1.93 =2*0.96+0.01 |
| FR2 | CSI-RS | DRX | 1 | 10.81 =2*5.12+2*0.26+0.05 | 10.28 = 2*5.12+0.04 | 0 | 0.57 =2*0.26+0.05 | 0.53 =2*0.26+0.01 |

  - Option 2: Other duration setting

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-1-2: The SNR level of T1-T5 for beam failure TRP in the test

- Proposals

  - Option 1: the SNR levels of two BFD-RS resource sets can be defined as Table 4. Reuse the legacy SNR level for CBD-RS.

- Table 4: SNR levels of BFD-RS for legacy link recovery tests

| RS | T1 | T2 | T3 | T4 | T5 |
| --- | --- | --- | --- | --- | --- |
| BFD-RS set (q0,0) | SNR1 | SN2 | SNR3 | SNR3 | SNR3 |
| BFD-RS set (q0,1) | SNR1 | SNR1 | SNR1 | SNR1 | SNR1 |

  - Option 2:Other SNR level setting

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-1-3: Beam candidate reporting procedure for different cells

- Proposals

  - Option 1:

    - For SPcell CFRA, BFR is achieved by RA, the same procedure as Rel-15.

    - For SPcell CBRA, UE transmits RACH first for applying UL-SCH resources and then MAC CE on the UL resource for BFR.

    - For Scell BFR, LRR is transmitted on PUCCH first for UL resource application, then followed by MAC CE containing candidate beams.

    - For Scell BFR without dedicated resource for BFR SR on PUCCH, UE will transmit RACH first for UL resource application, then followed by MAC CE on the UL-SCH containing candidate beams.

  - Option 2: Other procedures

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Sub-topic 3-2

Test Configurations for TRP specific BFR

Sub-topic description: discussion on the test configurations

Open issues and candidate options before e-meeting:

Issue 3-2-1: BFD/CBD RS configuration for TRP specific link recovery tests

- Proposals

  - Option 1: Following the table below and introduce new SSB configurations

- Table 2: BFD/CBD RS configuration for TRP specific link recovery tests

| Resource set | SSB based link recovery test | CSI-RS based link recovery test |
| --- | --- | --- |
| BFD-RS set (q0,0) | SSB#0 | CSI-RS#0 |
| CBD-RS set (q1,0) | SSB#1 | CSI-RS#1 |
| BFD-RS set (q0,1) | SSB#2 | CSI-RS#2 |
| CBD-RS set (q1,1) | SSB#3 | CSI-RS#3 |
| RLM-RS set | SSB#0, SSB#1, SSB#2, SSB#3 | CSI-RS#0, CSI-RS#1, CSI-RS#2, CSI-RS#3 |

  - Option 2: Other RS settings

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-2: AOA setup in FR2 test

- Proposals

  - Option 1: Two AoAs are configured in the test, each of which is for one of two TRPs

  - Option 2: other AoA setup

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-3: Beam failure occurred during the test case

- Proposals

  - Option 1: For R17 TRP-specific link recovery tests, it is suggested that beam failure occurs on one BFD-RS resource set. SNR level of TRP 2 will always keep at high level.

  - Option 2: Beam failure occurs on both TRPs in the test

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Issue 3-2-4: BFR-RS configured for 2 TRPs in the test

- Proposals

  - Option 1: BFD-RS of two TRPs are overlapped

  - Option 2: Other configuration

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

Companies views’ collection for 1st round

Open issues

Sub topic 3-1 Test procedure for TRP specific BFR

| Company | Comments |
| --- | --- |
| Huawei | Issue 3-1-1: we can agree with the principles in option 1. Issue 3-1-2: option 1 |
| Intel | Issue 3-1-1: Option 1. Issue 3-1-2: Option 1. Issue 3-1-3: For SCell BFR, prefer to re-use the legacy test procedure. E.g. , the UE shall transmit preamble for UL-SCH resource application, followed by MAC-CE on the assigned uplink resources containing a beam associated with the candidate beam set q1. |
| MediaTek | Issue 3-1-1: fine with option 1 Issue 3-1-2: fine with option 1 Issue 3-1-3: fine with option 1 |
| Apple | Issue 3-1-1: We can use option 1 as baseline and capture in “[ ]” and revise if needed. Is this for both FR1 and FR2?. Issue 3-1-2: Fine with option 1. Issue 3-1-3: Are we introducing 2 tests for SpCell with CFRA and CBRA? We prefer to only configure CFRA. For SCell,we. Already have test in Rel16 eMIMO with both configurations. We propose to reduce to only define test case with dedicated resource configured . |
| Samsung | Issue 3-1-1: The time period of T1-T5 in the test Support Option 1. Issue 3-1-2: The SNR level of T1-T5 for beam failure TRP in the test Support Option 1. Issue 3-1-3: Beam candidate reporting procedure for different cells Support option 1. The finalization procedure should be aligned with option 1. |
| Ericsson | Issue 3-1-1: The time period of T1-T5 in the test Do not understand the current calculation. Even if we take scaling factor of 2, legacy tests values should be multiplied by 2. May be clarification is needed. Issue 3-1-2: The SNR level of T1-T5 for beam failure TRP in the test We think both can have same level to test the BFD, BFR on both TRP. RS; T1; T2; T3; T4; T5. BFD-RS set (q0,0); SNR1; SN2; SNR3; SNR3; SNR3. BFD-RS set (q0,1); SNR1; SNR2; SNR3; SNR3; SNR3. Issue 3-1-3: Beam candidate reporting procedure for different cells One TRP can be with CBRA/CFRA RACH other can be SR based transmission |
|  | Issue 3-1-1: The time period of T1-T5 in the test - Proposals - Option 1: For TRP specific beam failure detection and link recovery test case, T2~T5 duration time will be extended due to scaling factor. (Huawei, Intel, MediaTek, Apple, Samsung) - Option 2: Other duration setting Issue 3-1-2: The SNR level of T1-T5 for beam failure TRP in the test - Proposals - Option 1: the SNR levels of two BFD-RS resource sets can be defined as Table 4. Reuse the legacy SNR level for CBD-RS. (Huawei, Intel, MediaTek, Apple, Samsung) - Option 2:Other SNR level setting - Option 2a: the same SNR level for both TRP(Ericsson) Issue 3-1-3: Beam candidate reporting procedure for different cells - Proposals - Option 1: (Huawei, Intel, MediaTek, Samsung) - For SPcell CFRA, BFR is achieved by RA, the same procedure as Rel-15. - For SPcell CBRA, UE transmits RACH first for applying UL-SCH resources and then MAC CE on the UL resource for BFR. - For Scell BFR, LRR is transmitted on PUCCH first for UL resource application, then followed by MAC CE containing candidate beams. - For Scell BFR without dedicated resource for BFR SR on PUCCH, UE will transmit RACH first for UL resource application, then followed by MAC CE on the UL-SCH containing candidate beams. - Option 2: Other procedures - Proposal 2: One TRP can be with CBRA/CFRA RACH other can be SR based transmission (Ericsson) - Proposal 3: only configure CFRA for PSCell and with dedicated resource for SCell (Apple) |

Sub topic 3-2 Test configuration for TRP specific BFR

| Company | Comments |
| --- | --- |
| Huawei | Issue 3-2-1: agree with option 1. Issue 3-3-2: agree with option 1 |
| Intel | Issue 3-2-1: in order to consider sharing fator. We suggest to use the same SSB configuration. Resource set; SSB based link recovery test; CSI-RS based link recovery test. BFD-RS set (q0,0); SSB#0; CSI-RS#0. CBD-RS set (q1,0); SSB#1; CSI-RS#1. BFD-RS set (q0,1); SSB#0; CSI-RS#0. CBD-RS set (q1,1); SSB#1; CSI-RS#1. RLM-RS set; SSB#0, SSB#1; CSI-RS#0, CSI-RS#1. Issue 3-2-2: Option 1. Issue 3-2-3: Option 1. Issue 3-2-4: Option 1. |
| MediaTek | Issue 3-2-1: fine with option 1 Issue 3-2-2: fine with option 1 Issue 3-2-3: fine with option 1 Issue 3-2-4: prefer option 1. |
| Apple | Issue 3-2-1: We cannot configure SSB as resource in BFD RS set based on RAN1 design. We suggest revisiting the test cases we are defining. Also, we propose in [221] to delete core requirements for SSB based BFD for TRP specific link recovery requirements. Issue 3-2-2: Fine with option 1 Issue 3-2-3: Support option 1 Issue 3-2-4: Fine with option 1 |
| Samsung | Issue 3-2-1: BFD/CBD RS configuration for TRP specific link recovery tests Option 1 is fine for us. In this case, some new configurations are needed. Issue 3-2-2: AOA setup in FR2 test Support option 1. The same as inter-cell beam management, different QCL type D is test for 2 TRPs in the test. Issue 3-2-3: Beam failure occurred during the test case Option 1 is preferred. Beam failure occurs on only one TRP while two TRPs are monitored simultaneously. It is the most basic scenario to verify. Issue 3-2-4: BFR-RS configured for 2 TRPs in the test Option 1 is preferred where two BFD-RS are overlapped and shared by respective TRP measurements (sharing factor = 2). |
| Ericsson | Issue 3-2-1: BFD/CBD RS configuration for TRP specific link recovery tests Agree with option 1 Issue 3-2-2: AOA setup in FR2 test Fine with option 1 Issue 3-2-3: Beam failure occurred during the test case We support Option 2. We think since UE can perform both TRP BFD simulataneously, it is important to test it. Issue 3-2-4: BFR-RS configured for 2 TRPs in the test We are fine with Option 1 |
| Moderator | Issue 3-2-1: BFD/CBD RS configuration for TRP specific link recovery tests - Proposals - Option 1: Following the table below and introduce new SSB configurations (Huawei, MediaTek, Samsung, Ericsson ) - Option 2: Other RS settings - Option 2a: use the same SSB configuration. (Intel) - Option 2b: SSB is not allowed for BFD RS.(Apple) Issue 3-2-2: AOA setup in FR2 test - Proposals - Option 1: Two AoAs are configured in the test, each of which is for one of two TRPs (Huawei, Intel, MediaTek, Samsung, Ericsson) - Option 2: other AoA setup Issue 3-2-3: Beam failure occurred during the test case - Proposals - Option 1: For R17 TRP-specific link recovery tests, it is suggested that beam failure occurs on one BFD-RS resource set. SNR level of TRP 2 will always keep at high level. (Intel, MediaTek, Samsung,) - Option 2: Beam failure occurs on both TRPs in the test (Ericsson) Issue 3-2-4: BFR-RS configured for 2 TRPs in the test - Proposals - Option 1: BFD-RS of two TRPs are overlapped (Intel, MediaTek, Samsung, Ericsson) - Option 2: Other configuration |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213492 Huawei, HiSilicon | Moderator: DraftCR, introducing new SSB configuration for TRP specific BFR test case |
| R4-2212918 Nokia | Moderator: DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for NR SA FR1+FR2 CA case |
| R4-2212919 Nokia | Moderator: DraftCR, introducing test case of L1-RSRP measurement on cell with different PCI for FR1 NR SA case. |
| R4-2213947 Ericsson | Moderator: DraftCR, introducing test case of TRP specific BFR for FR1 EN-DC SCell case |
| R4-2211862 Apple | Moderator: DraftCR, introducing test case of TRP specific BFR for FR1 PCell NR SA case |
| R4-2212123 Intel | Moderator: DraftCR, introducing test case of TRP specific BFR for FR2 PCell NR SA case |
| R4-2212124 Intel | Moderator: DraftCR, introducing test case of TRP specific BFR for FR1 SCell NR SA case |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: The time period of T1-T5 in the test Tentative agreements: For TRP specific beam failure detection and link recovery test case, T2~T5 duration time will be extended due to scaling factor. Further consider the value. Candidate options: N/A Recommendations for 2nd round: If no further comments in 2nd round, go tentative agreements. Issue 3-1-2: The SNR level of T1-T5 for beam failure TRP in the test Tentative agreements:N/A Candidate options: - Option 1: different SNR levels for two links - Option 2: the same SNR levels for two links Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. Related to issue 3-2-3. Issue 3-1-3: Beam candidate reporting procedure for different cells Tentative agreements: N/A Candidate options: - Option 1: Follow the legacy test cases - Option 2: One TRP can be with CBRA/CFRA RACH other can be SR based transmission - Option 3: Only configure CFRA based BFR for PSCell and SR based BFR for SCell Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. Related to issue 3-2-3. |
| Sub-topic#3-2 | Issue 3-2-1: BFD/CBD RS configuration for TRP specific link recovery tests Tentative agreements:N/A Candidate options: - Option 1: Use 4 different SSB/CSI-RS configuration - Option 2: Use 2 different SSB/CSI-RS configuration, same SSB configuration for different TRP. - Option 3: SSB cannot be used as BFD RS Recommendations for 2nd round: Continue discuss in 2nd round. RAN4 needs to check if SSB is qualified for BFD-RS. Issue 3-2-2: AOA setup in FR2 test - Proposals - Option 1: Two AoAs are configured in the test, each of which is for one of two TRP (Huawei, Intel, MediaTek, Samsung, Ericsson) - Option 2: other AoA setup Tentative agreements: Two AoAs are configured in the test, each of which is for one of two TRP Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. Issue 3-2-3: Beam failure occurred during the test case Tentative agreements:N/A Candidate options: - Option 1: Beam failure occurs on one BFD-RS resource set. SNR level of TRP 2 will always keep at high level. - Option 2: Beam failure occurs on both TRPs Recommendations for 2nd round: Continue discuss in 2nd round and discuss on candidate options. Issue 3-2-4: BFR-RS configured for 2 TRPs in the test Tentative agreements: BFD-RS of two TRPs are overlapped Candidate options: N/A Recommendations for 2nd round: No further discussion in 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| --- | --- |
| R4-2213492 Huawei, HiSilicon | To be revised. |
| R4-2212918 Nokia | To be revised. |
| R4-2212919 Nokia | To be revised. |
| R4-2213947 Ericsson | To be revised. |
| R4-2211862 Apple | To be revised. |
| R4-2212123 Intel | To be revised. |
| R4-2212124 Intel | To be revised. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Open issues

Sub topic 3-1

Issue 3-1-1: Use SSB as BFD-RS in test cases

- Proposals

  - Option 1: SSB can be used as BFD RS in the test case

  - Option 2: SSB cannot be used as BFD RS in the test case, revisiting the test cases.

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

  - From moderator’s understanding, SSB can be configured as BFD-RS implicitly based on RAN1 design. Proponent of Option 2 please clarify the proposal and show concrete evidence to support the proposal.

Issue 3-1-2: The time period of T1-T5 in the test

- Proposals

  - Option 1: For TRP specific beam failure detection and link recovery test case, T2~T5 duration time will be extended due to scaling factor.

| FR | RS type | DRX Config | T1(s) | T2(s) (BFD time +CBD time +margin) | T3(s) (BFD time +margin) | T4(s) | T5(s) (CBD time +margin) | D1(s) (CBD time +margin) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 2.61 = 2*1.6+2*0.96+0.05 | 3.24 = 2*1.6+0.04 | 0 | 1.97 =2*0.96+0.05 | 1.93 =2*0.96+0.01 |
| FR2 | CSI-RS | DRX | 1 | 10.81 =2*5.12+2*0.26+0.05 | 10.28 = 2*5.12+0.04 | 0 | 0.57 =2*0.26+0.05 | 0.53 =2*0.26+0.01 |

    - Option 1a: further consider the values of each period

  - Option 2: Other duration setting

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

Issue 3-1-3: BFD/CBD RS configuration for TRP specific link recovery tests

- Proposals

  - Option 1: Use 4 different SSB/CSI-RS configuration

| Resource set | SSB based link recovery test | CSI-RS based link recovery test |
| --- | --- | --- |
| BFD-RS set (q0,0) | SSB#0 | CSI-RS#0 |
| CBD-RS set (q1,0) | SSB#1 | CSI-RS#1 |
| BFD-RS set (q0,1) | SSB#2 | CSI-RS#2 |
| CBD-RS set (q1,1) | SSB#3 | CSI-RS#3 |
| RLM-RS set | SSB#0, SSB#1, SSB#2, SSB#3 | CSI-RS#0, CSI-RS#1, CSI-RS#2, CSI-RS#3 |

  - Option 2: Use 2 different SSB/CSI-RS configuration, same SSB configuration for different TRP.

| Resource set | SSB based link recovery test | CSI-RS based link recovery test |
| --- | --- | --- |
| BFD-RS set (q0,0) | SSB#0 | CSI-RS#0 |
| CBD-RS set (q1,0) | SSB#1 | CSI-RS#1 |
| BFD-RS set (q0,1) | SSB#0 | CSI-RS#0 |
| CBD-RS set (q1,1) | SSB#1 | CSI-RS#1 |
| RLM-RS set | SSB#0, SSB#1 | CSI-RS#0, CSI-RS#1 |

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

Issue 3-1-4: Test procedures for TRP specific BFR test case

- Proposals

  - Option 1: Beam failure occurs on one BFD-RS resource set. SNR level of TRP 2 will always keep at high level.

  - Option 2: Beam failure occurs on both TRPs.

    - Option 2a: SNR level of 2 TRPs are keeping at the same level. UE perform BFD on 2 TRPs simultaneously.

- Recommended WF

  - Companies’ views are collected in 2nd round discussion.

Issue 3-1-5: Beam recovery procedure defined in the test case

- Proposals

  - Option 1: Beam failure occurs on one TRP. For BFR on SPCells, CFRA based BFR is configured for all test cases. For BFR on SCells, dedicated BFR resource is configured for all test cases.

  - Option 2: Beam failure occurs on both TRPs. One TRP can be with CBRA/CFRA RACH, another can be SR based transmission

  - Option 3: Other configurations.

- Recommended WF

  - Companies’ views are collected in 2nd round discussion. Related to 3-1-4.

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1-1: Option 1. We think RAN2 supports SSB based BFD and it should be supported in Ran4 too. Sometimes RAN1 spec may be it high level and may not state explicitly everything. If signalling is supported, NW can configure it. Issue 3-1-2: Option 2. We think the values should be scaled by 2 times for equivalent test case defined in Rel-16. We do not understand the calculation as they are quite high compared to 2times of legacy values. Issue 3-1-3: Option 1 is preferred. Or modified option 2 (where BFD-RS is different across sets) Issue 3-1-4: We prefer option 2a. That is the config we assumed in our CR. At least option 2a should be tested for SCell case where two TRP are configured for SCell. Some test cases at least should be with option 2/2a. Issue 3-1-5: We think some tests can be with option 1 and some tests can be with option 2. We provided test case with option 2. |
| Apple | Issue 3-1-1: Use SSB as BFD-RS in test cases Based on companies’ clarification in 1st round that its supported by RAN2 design, we are fine to include it. Issue 3-1-2: The time period of T1-T5 in the test Option 1 is fine as baseline. We can capture it in “[ ]” and revise later if needed. Issue 3-1-4: Test procedures for TRP specific BFR test case Option 1. It would be more meaningful to test with 1 TRP with lower SNR level to validate that UE is correctly processing TRP specific BFR. We don’t see the purpose of configuring both to fail, seems like an artificial setup. Issue 3-1-5: Beam recovery procedure defined in the test case Option 1. |
| Intel | Issue 3-1-1: Option 1. Issue 3-1-2: Option 1. We agree to add bracket for the values. To Ericsson, these values are scaled by 2 from legacy values. the margin time don’t need to be doubled. Issue 3-1-3: prefer option 2. If SSB are not overlapped, we don’t see any difference between legacy test. Issue 3-1-4: Option 2. Test two different behavior make more sense. Issue 3-1-5: Support one TRP is beam failure and the procedure is the same as legacy SCell BFR test. |
| Samsung | Issue 3-1-1: Use SSB as BFD-RS in test cases In RAN1 spec, clearly there is “For the set q0, the UE assesses the radio link quality only according to SS/PBCH blocks on the PCell or the PSCell or periodic CSI-RS resource configurations that are quasi co-located, as described in [6, TS 38.214], with the DM-RS of PDCCH receptions by the UE.” So we think it is no problem for SSB serving as BFD-RS. Issue 3-1-2: The time period of T1-T5 in the test Option 1 and 1a. If companies have concern, we could further consider the value. Issue 3-1-3: BFD/CBD RS configuration for TRP specific link recovery tests Option 1 is preferred. In our understanding, SSB#0 and SSB#2 have different id, but they are overlapped. Issue 3-1-4: Test procedures for TRP specific BFR test case Option 1 is preferred. It would be a common case if BFD occurred in mTRP scenario. Issue 3-1-5: Beam recovery procedure defined in the test case Option 1 is preferred. |
| Huawei | Issue 3-1-1: from RAN2 perspective, SSB based BFD is supported for TRP specific BFR. Issue 3-1-2: we are fine with option 1 Issue 3-1-3: either option is fine for us. Issue 3-1-4: support option 1. Issue 3-1-5: support option 1. |

CRs/TPs comments collection

| CR/TP number |  |  | Comments collection |
| --- | --- | --- | --- |
| Tdoc Number | Revised from | Source Company |  |
| R4-2215065 | R4-2213492 | Huawei, HiSilicon | Samsung: According to Huawei’s CR, the SSB and CSI-RS configurations for TRP specific BFR tests can be summarized as Table below, FR; SCS; SSB based link recovery test; CSI-RS based link recovery test. FR1; FDD 15kHz; SSB.3 FR1 for TRP1 SSB.7 FR1 for TRP2; CSI-RS.1.2 FDD for TRP1 CSI-RS.1.7 FDD for TRP2. TDD 15kHz; SSB.3 FR1 for TRP1 SSB.7 FR1 for TRP2; CSI-RS.1.2 TDD for TRP1 CSI-RS.1.6 TDD for TRP2. TDD 30kHz; SSB.4 FR1 for TRP1 SSB.8 FR1 for TRP2; CSI-RS.2.2 TDD for TRP1 CSI-RS.2.7 TDD for TRP2. FR2; TDD 120kHz; SSB.1 FR2 for TRP1 SSB.9 FR2 for TRP2; CSI-RS.3.2 TDD for TRP1 CSI-RS.3.6 TDD for TRP2. TDD 240kHz; SSB.2 FR2 for TRP1 SSB.10 FR2 for TRP2; N/A. According to agreement, BFD RS are overlapped for TRP1 and TRP2. Hence, SSB#0 and SSB#2 shall overlap in symbols, with different “SS/PBCH block index”. But as I checked, CSI-RS.3.2 and CSI-RS.3.6 have different “offset”, so they seems to be not overlapped. Resource set; SSB based link recovery test; CSI-RS based link recovery test. BFD-RS set (q0,0); SSB#0; CSI-RS#0. CBD-RS set (q1,0); SSB#1; CSI-RS#1. BFD-RS set (q0,1); SSB#2; CSI-RS#2. CBD-RS set (q1,1); SSB#3; CSI-RS#3. RLM-RS set; SSB#0, SSB#1, SSB#2, SSB#3; CSI-RS#0, CSI-RS#1, CSI-RS#2, CSI-RS#3. Please correct me if I am wrong. |
| R4-2214980 | R4-2212918 | Nokia | Samsung: CSI-RS configuration for TRP2 is not aligned with above Table. |
| R4-2214981 | R4-2212919 | Nokia | Samsung: AoA setup 1 is used in the CR, which is not aligned with our agreements that AoA setup 3 should be used and each AoA is for one TRP. CSI-RS configuration for TRP2 is not aligned with above Table. In Nokia’s FR2 CR, inter-cell 2 TRPs scenario are used, instead of intra-cell 2 TRPs. Maybe we need to align the test setup for all FR2 tests. |
| R4-2215100 | R4-2213947 | Ericsson | Samsung: CSI-RS configuration is not aligned with Huawei’s Table. BFD RS and CBD RS configuration should be specify for each TRP, respectively, in the table “General test parameters”. |
| R4-2214910 | R4-2211862 | Apple | Samsung: CSI-RS configuration for TRP1 is not aligned with above Table. |
| R4-2214938 | R4-2212123 | Intel | Samsung: SSB configuration for TRP2 is not aligned with above Table. |
| R4-2214939 | R4-2212124 | Intel | Samsung: CSI-RS configuration for TRP2 is not aligned with above Table. This is a new test case, but some changes do not have change mark. |

Samsung: A general comments for all BFR test cases: We have not reached agreement on candidate beam reporting procedure for each test. Revision may be needed for the test cases after we have agreement on the issue.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2214483 | WF on FeMIMO RRM Performance Requirement and Test Case | Samsung | Way forward on FeMIMO RRM performance part |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2213174 |  | Draft CR to TS38.133 RRM test configurations for FeMIMO test | Samsung | To be merged. | Merged into R4-2213488. |
| R4-2213488 |  | DraftCR on introducing unified TCI configurations | Huawei, HiSilicon | To be revised. | Based on 2nd round discussion, a new CSI-RS and a new TRS might be needed. For the unified TCI configurations, need to further check. |
| R4-2213885 |  | Draft CR on test case for MAC CE based TCI state switch for a known joint TCI state in FR2 in TS38.133 A7.5.x.y | ZTE Corporation | To be revised. | According to 1st round summary, the test requirement for Joint TCI case should be joint max(UL,DL). |
| R4-2213173 |  | Draft CR to TS38.133 Test case for R17 UL TCI state switching | Samsung | To be revised. | According to 1st round opinions, following the same procedure as legacy test cases. |
| R4-2211861 |  | Draft CR for test case for FeMIMO - Unified TCI state switching for EN-DC in FR2 | Apple | To be revised. | Considering how to configure data transmission for NSC case. |
| R4-2212518 |  | Draft CR on TC for joint unified TCI state switching in FR2 NR SA_MediaTek Inc | MediaTek Inc. | To be revised. | Further consider whether “The rate of correct events observed during repeated tests shall be at least [90]%” is needed. |
| R4-2213946 |  | Test case on unified TCI state switching | Ericsson | To be revised. | CSI-RS is configured as the RS. Consider how to deal with not maintained PL-RS in test case. |
| R4-2212669 |  | Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA | vivo | To be revised. | Configuration part can be merged into R4-2213488. Further consider TRS is needed in the test case. |
| R4-2212670 |  | Draft CR on test case for L1-RSRP measurement procedure in FR1 NR-SA | vivo | To be revised |  |
| R4-2213490 |  | DraftCR on L1-RSRP measurement test for R17 inter-cell beam managements | Huawei, HiSilicon | To be revised | AoA setup 3 is configured for FR2. |
| R4-2213884 |  | Draft CR on test case for L1-RSRP measurement on a cell with different PCI from serving cell for NR SA in FR1 in TS38.133 A.6.6.x.y | ZTE | To be merged. | Merged into R4-2212670. |
| R4-2213492 |  | DraftCR on SSB and CSI-RS configurations for TRP specific BFR tests | Huawei, HiSilicon | To be revised. |  |
| R4-2212918 |  | Draft CR to TS38.133 TRP BFR performance EN-DC + FR1 + PSCell FR1 | Nokia | To be revised. |  |
| R4-2212919 |  | Draft CR to TS38.133 TRP BFR performance EN-DC + FR1 + PSCell FR2 | Nokia | To be revised. |  |
| R4-2213947 |  | Test case on link recovery procedure per TRP | Ericsson | To be revised. |  |
| R4-2211862 |  | Draft CR for test case for FeMIMO – TRP specific BFR for NR-SA in FR1 (PCell) | Apple | To be revised. |  |
| R4-2212123 |  | Draft CR about TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Intel | To be revised. |  |
| R4-2212124 |  | Draft CR about TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-based BFD and LR in DRX mode | Intel | To be revised. |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214483 | N/A | WF on FeMIMO RRM Performance Requirement and Test Case | Samsung | Agreeable | Way forward on FeMIMO RRM performance part |
| R4-2213488 | R4-2215063 | DraftCR on introducing unified TCI configurations | Huawei, HiSilicon | Agreeable |  |
| R4-2213885 | R4-2215097 | Draft CR on test case for MAC CE based TCI state switch for a known joint TCI state in FR2 in TS38.133 A7.5.x.y | ZTE Corporation | Agreeable |  |
| R4-2213173 | R4-2215030 | Draft CR to TS38.133 Test case for R17 UL TCI state switching | Samsung | Agreeable |  |
| R4-2211861 | R4-2214909 | Draft CR for test case for FeMIMO - Unified TCI state switching for EN-DC in FR2 | Apple | To be revised |  |
| R4-2212518 | R4-2214965 | Draft CR on TC for joint unified TCI state switching in FR2 NR SA_MediaTek Inc | MediaTek Inc. | Agreeable |  |
| R4-2213946 | R4-2215099 | Test case on unified TCI state switching | Ericsson | Agreeable |  |
| R4-2212669 | R4-2214969 | Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA | vivo | Agreeable |  |
| R4-2212670 | R4-2214970 | Draft CR on test case for L1-RSRP measurement procedure in FR1 NR-SA | vivo | Agreeable |  |
| R4-2213490 | R4-2215064 | DraftCR on L1-RSRP measurement test for R17 inter-cell beam managements | Huawei, HiSilicon | Agreeable |  |
| R4-2213492 | R4-2215065 | DraftCR on SSB and CSI-RS configurations for TRP specific BFR tests | Huawei, HiSilicon | Agreeable |  |
| R4-2212918 | R4-2214980 | Draft CR to TS38.133 TRP BFR performance EN-DC + FR1 + PSCell FR1 | Nokia | Agreeable |  |
| R4-2212919 | R4-2214981 | Draft CR to TS38.133 TRP BFR performance EN-DC + FR1 + PSCell FR2 | Nokia | Agreeable |  |
| R4-2213947 | R4-2215100 | Test case on link recovery procedure per TRP | Ericsson | Agreeable |  |
| R4-2211862 | R4-2214910 | Draft CR for test case for FeMIMO – TRP specific BFR for NR-SA in FR1 (PCell) | Apple | To be revised |  |
| R4-2212123 | R4-2214938 | Draft CR about TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Intel | Agreeable |  |
| R4-2212124 | R4-2214939 | Draft CR about TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-based BFD and LR in DRX mode | Intel | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents