3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214308

Electronic Meeting, 15– 26 August 2022

Agenda item:			5.3

Source:	Moderator (Apple)

Title:	Email discussion summary for [104-e][319] Demod_Maintenance_UE

Document for:	Information

# Introduction

In this email thread for UE Demod Maintenance, the following topics will be covered:

- Up to Rel-16 Demod maintenance: UE part (4.6.1, 4.6.2)

- Rel-17 UE Demod Maintenance (5.2.2, 5.2.4.4, 9.6.3, 9.12.3)

Target of email discussion for 1st round and 2nd round

- 1st round: Companies provide comments on draft CRs and responses to comments

- 2nd round: Revised draft CRs based on comments are shared and agreed

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Qualcomm | Pier Vallese | pvallese@qti.qualcomm.com |
| ZTE | Wenhao Liu | liu.wenhao@zte.com.cn |
| Apple | Manasa Raghavan | Manasa.raghavan@apple.com |
| Qualcomm | Gaurav Nigam | gnigam@qti.qualcomm.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Huawei | Zehan Zhao | zhaozehan@hisilicon.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Up to Rel-16 UE Demod Maintenance

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211571 | Rohde & Schwarz | Addition of clarification for SIB1 scheduling in RMCs Change: Add statement on SIB1 scheduling to the Annex. |
| R4-2211572 | Rohde & Schwarz | Addition of clarification for SIB1 scheduling in RMCs Cat-A CR |
| R4-2211573 | Rohde & Schwarz | Addition of clarification for SIB1 scheduling in RMCs Cat-A CR |
| R4-2211582 | Rohde & Schwarz | Correction of aperiodicTriggeringOffset aperiodicTriggeringOffset has been set to “Not Configured” in Table 6.2.3.2.2.2.3-1. Editorial correction of incorrect table number. |
| R4-2211583 | Rohde & Schwarz | Correction of aperiodicTriggeringOffset Cat-A CR |
| R4-2211584 | Rohde & Schwarz | Correction of aperiodicTriggeringOffset Cat-A CR |
| R4-2212335 | MediaTek Inc. | Corrections on Rel-16 NR V2X PSCCH decoding capability 1. Modify the PSSCH RMC parameter of NR V2X PSCCH decoding capability test in Table 11.1.8.1.1-1 of Cluase 11.1.8 since the current PSSCH RMC is not correct. |
| R4-2212336 | MediaTek Inc. | Corrections on Rel-16 NR V2X PSCCH decoding capability Cat-A CR |
| R4-2212636 | ZTE Corporation | draft CR to TS 38.101-4[R16]_Corrections on the Doppler shift equation for kth RRH in Annex B 1. Modify the equation of B.3.2.5 and B.3.3. 5, i.e. replace v*fc/c with fixed fd and delete the sentences stating v, fc, c . 2. Correct the error reference/typos for some figures or equations Moderator: Cat-A CR reservation is missing ZTE: Thank you for reminding, I am applying for Cat-A tdoc number. |
| R4-2213077 | Nokia, Nokia Shanghai Bell | Correction to table headers for PBCH TDD demodulation requirements The table headers for PBCH TDD demodulation requirements are corrected for 2 Rx (subclause 5.4.2.2) and for 4 Rx (subclause 5.4.3.2) to refer to SS/PBCH block index. |
| R4-2213078 | Nokia, Nokia Shanghai Bell | Correction to table headers for PBCH TDD demodulation requirements Cat-A CR |
| R4-2213079 | Nokia, Nokia Shanghai Bell | Correction to table headers for PBCH TDD demodulation requirements Cat-A CR |
| R4-2213080 | Nokia, Nokia Shanghai Bell | Corrections to Tables A.4-1 and A.4-2 1) The reference to CSI reporting requirements for FR1 and CSI reporting requirements for FR2 is added. 2) For Table A.4-1, available RE-s are corrected for TBS.1-1 and TBS.1-2. Available REs = [(12 SCs per PRB * 12 symbols) – 24 DMRS REs per PRB – 6 overhead REs per PRB] * 66 PRBs = 7524. |
| R4-2213081 | Nokia, Nokia Shanghai Bell | Corrections to Tables A.4-1 and A.4-2 1) For Table A.4-1, available RE-s are corrected for TBS.1-1 and TBS.1-2. Available REs = [(12 SCs per PRB * 12 symbols) – 24 DMRS REs per PRB – 6 overhead REs per PRB] * 66 PRBs = 7524. 2) For Table A.4-2, available RE-s are corrected for TBS.2-7 introduced in the WI NR_DL256QAM_FR2 (value 3680 was introduced in R4-2103838). Available REs = [(12 SCs per PRB * 12 symbols) – 24 DMRS REs per PRB – 6 overhead REs per PRB] * 32 PRBs = 3648. 3) For Table A.4-2, the Information bit payload per slot is corrected for TBS.2-8, CQI index 1 and rank 2. It is calculated as 2856 according to TS 38.214, clause 5.1.3.2. |
| R4-2213082 | Nokia, Nokia Shanghai Bell | Corrections to Tables A.4-1 and A.4-2 Cat-A CR |
| R4-2213640 | Qualcomm Incorporated | Draft CR on Correction in PDSCH CA Tests PUCCH format 1 is only applied when number of ACK/NACK to be transmitted on single PUCCH is <=2. |
| R4-2213743 | Qualcomm Incorporated | Draft CR on Correction in PDSCH CA Tests Cat-A CR |
| R4-2213910 | MediaTek inc. | Draft CR to TS38.101-4, correction to CQI requirements for Table 3 (Rel-16) Change “First OFDM symbol in the PRB used for CSI-RS” from 1 to 13. |
| R4-2213908 | MediaTek inc. | Draft CR to TS38.101-4, correction to CQI requirements for Table 3 (Rel-17) Cat-A CR |
| R4-2213911 | MediaTek inc. | Draft CR to TS38.101-4, TDD UL-DL pattern for HST-DPS (Rel-16) Add TDD UL-DL pattern defined in HST-DPS requirement |
| R4-2213909 | MediaTek inc. | Draft CR to TS38.101-4, TDD UL-DL pattern for HST-DPS (Rel-17) Cat-A CR |
| R4-2211547 | Anritsu Corporation | Draft CR to CQI-PMI-RI Delay in 6.2A.3.1.1 Separated CQI/PMI/RI Delay with condition based on CA band combinations stated for the tests. |
| R4-2211548 | Anritsu Corporation | Draft CR to CQI-PMI-RI Delay in 6.2A.3.1.1 Cat-A CR |

## Open issues summary

None

## Companies views’ collection for 1st round

### Open issues

N/A

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211571 (R&S) | Anritsu: Agree. Assuming the change is aligned with the previous agreement in topic group #316 at the #103-e meeting. |
|  | Company B |
| R4-2211582 (R&S) | Anritsu: Agree. |
|  | Company B |
| R4-2212335 (MediaTek) | Company A |
|  | Company B |
| R4-2212636 (ZTE) | Ericsson: We are fine to update the formula. This formula comes from LTE and it works for LTE because LTE usually assumes fc=2GHz. On the other hand, NR can assume several fc like 2GHz for FDD 15kHz and 3.6GHz for TDD 30kHz. If it is agreed, Cat-A CR to Rel-17 is needed as the moderator commented. |
|  | Apple: We prefer not to change it and keep the original formula as Doppler would be derived based on velocity and fc. We define the requirements for max Fd, but we can have different Fd depending on the operating band. |
|  | ZTE: First of all, we want to reach a unified understanding: the requirement in the specification corresponds to two Doppler trajectories which can be seen from figure B.3.2-3(shown below) and Figure B.3.2-4(omitted) that fd is related to different SCS (870Hz for SCS 15kHz, 1667Hz for SCS 30kHz). If the above is the common understanding, The Doppler trajectories figures means no explicit specific fc is considered. Figure B.3.2-3 Doppler shift trajectories (= 870 Hz) The specification says that ”The trajectories of ralative power, Doppler shifts and absolute delays presented in Figures B.3.2-2, B.3.2-3, B.3.2-4 and B.3.2-5 are derived from the equations B.3.2.4 ~ B.3.2.6 respectively” if the current formula is used then different fc will corresponds to different fd. While from Table B.3.2-1 we can see only 2 fd are defined. Table B.3.2-1: HST-SFN scenario Parameter; Value. 700 m. 150 m. 500 km/h. 870 Hz for 15 kHz SCS test; 1667 Hz for 30 kHz SCS test. Therefore, there is confusion in the specification: If the specification defines two typical values of fd, fc should not appear in the formula. If fc appears in the formula then the Doppler trajectories should be multiple cases instead of only 2 value (870Hz and 1667Hz). The proposed modification is only a candidate for disambiguation of understanding, other solutions that can eliminate confusion can also be considered. |
|  | Qualcomm: We don’t have a strong preference for this change. However, if we do make this change, we still need to define the symbols f_d, v and C since they are being used in Equation B.3.2.6 and Tables B.3.2-1, B.3.3-1. |
|  | Huawei: We slightly prefer to keep current wording. The formula is reused from LTE and is generic formal for Doppler shift calculation.. |
|  | ZTE: @Qualcomm: We can see symbol fd has already appeared in Table B.3.2-1 which is highlighted in yellow above. For symbol v and symbol C, we think these symbols can be kept in the specification. @Huawei: The motivation for modifying the specification is that there is inconsistency between the text, graphics and formulas. If there is such inconsistency in the LTE, we can also consider modifying LTE specification. |
| R4-2213077 (Nokia) | Company A |
|  | Company B |
| R4-2213080 (Nokia) | Ericsson: We should keep 7590 bits for available REs, as it is derived from: (12 SCs per PRB * 12 symbols - 24 DMRS symbols per PRB) * 66PRB - 10 PT-RS symbols per PRB * 33 PRB, that is, (12*12-24)*66-10*33=7590. Note PT-RS is transmitted every other RBs. Overhead is considered only when TBS is derived; we should not consider for available RE calculation. |
|  | Qualcomm: Same comment as Ericsson |
| R4-2213081 (Nokia) | Apple: Based on comments from Ericsson above, changes in TBS.1-1 and TBS.1-2 should be removed. For TBS.2-7 3680 is correct based on clarification from Ericsson above. Correction for TBS.2-8 is fine. |
|  | Qualcomm: Same comment as Apple |
| R4-2213640 (Qualcomm) | Anritsu: Agree. |
|  | Ericsson: We are ok with such a clarification. We suggest the following wordings: - PUCCH format 1 for cases the number of ACK/NACK to be transmitted on single PUCCH is 2 or less. - PUCCH format 3 for cases the number of ACK/NACK to be transmitted on single PUCCH is more than 2. |
|  | Qualcomm: @Ericsson: We are ok with such wording with small editorial change: - PUCCH format 1 for cases where the number of ACK/NACK to be transmitted on single PUCCH is 2 or less. - PUCCH format 3 for cases where the number of ACK/NACK to be transmitted on single PUCCH is more than 2. We hope this is ok with you. |
|  | Huawei: The rewording from Qualcomm is fine for us. |
|  | Ericsson2: The rewording from Qualcomm is fine with us. |
| R4-2213910 (MediaTek) | Company A |
|  | Company B |
| R4-2213911 (MediaTek) | Anritsu: Agree. |
|  | Huawei: Ok with the addition of TDD UL-DL pattern for HST-DPS cases. |
| R4-2211547 (Anritsu) | Ericsson: The CR is ok. But the attached calculation Excel sheet may confuse. In '1-1. CQI DLCA FDD15kHz+TDD30kHz' the CQI delay for SCELL (SCS=30kHz) should be 9.5ms, instead of 6ms. No need to revise the CR, because it does not affect to the CR. |
|  | Qualcomm: We have a question for clarification for TDD SCell case when PCell is also TDD. If we assume gNB delay of 4ms, then CSI report should be applied on Slot 27 which is an S slot, where PDSCH is not scheduled. So, won’t gNB have to wait until slot 30, similar to PCell to schedule the grant? If that’s the case, delay should be same as PCell, i.e., 14.5ms. |

## Summary for 1st round

### Open issues

N/A

### CRs/TPs

Provided in Section 3.1

## Discussion on 2nd round

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212636 (ZTE) | Moderator: 2 companies don’t have strong opinion on the update, 2 companies prefer to keep the existing formula. Continue discussion in 2nd round to see if change is agreeable. @ZTE-Please trigger separate email thread for discussion |
|  | Apple: Thanks for the detailed description. For the test cases the fd will be configured independent of the operating band/ carrier freq. So, we are fine with updating the equation to have fd instead of fc*v/c and keep the required parameters used in equations in the table. |
|  | Huawei: The fd in Table B.3.2-1 is clearly set for different SCS as captured in the table: Table B.3.2-1: HST-SFN scenario Parameter; Value. <image001.png>; 700 m. <image002.png>; 150 m. <image003.png>; 500 km/h. <image004.png>; 870 Hz for 15 kHz SCS test; 1667 Hz for 30 kHz SCS test. If you have strong concern about the general formula for the fd calculation, we suggest to add one note for this formula and not sure if it is acceptable for you? |
|  | ZTE: @Huawei If a note to the formula is considered, how do you think about the note? Maybe the following can be a candidate for the note: Formula B.3.2.5 is a general formula for Doppler calculation. When defining the requirement for FR1 HST, two fixed maximum Doppler shifts are selected according to SCS as defined in table B.3.2-1. Considering the consistency of the specification, we prefer to directly change the formula, otherwise the formula in the specification is not actually applied and the accuracy of adding a note may not be as accurate as changing the formula. |
|  | R&S: We understand the reasoning behind the CR, but we would like to check the impact on the implementation when changing the formula, but we will not be able to during this meeting. We can go with a Note for now, but we need to check on the implementation impact before changing the calculation. |
|  | Huawei: Note: Formula B.3.2.5 is a general formula for Doppler shift calculation. When defining the requirement for FR1 HST, fixed Doppler shifts are selected as defined in Table B.3.2-1 for HST-SFN scenario. |
|  | ZTE: The revised version has been uploaded to RevisedCRs folder: REV_R4-2212636 draft CR to TS 38.101-4[R16]_Corrections on the Doppler shift equation for kth RRH in Annex B.docx The revised version added a note to the Doppler calculation formulas of HST-SFN and HST-DPS, and the word of 'formula' was replaced by 'equation' for consistency with the specification. |
|  | Huawei: The updates are fine for us. Other minor comments: - Please correct the “Proposed change affects” field in the coversheet from “Radio Access Network” to “ME” in your final version. - Maybe the mirror CR for Rel-17 is also needed. |
|  | Qualcomm: Thank you for the revised CR. We would like to correct the note. Current wording of “When defining the requirement for FR1 HST, fixed Doppler shifts are selected” may suggest that Doppler shift is fixed regardless of UE location because referenced equation also has UE location as one of the parameters. So, we would like to emphasize that “fd = fc*v/C is fixed” rather than “fixed Doppler shift”. |
|  | ZTE: @Qualcomm-Thanks for pointing the problem in the revised draft CR. Does the following modification resolve your concerns? Equation B.3.2.5 is a general formula for Doppler shift calculation. When defining the requirement for FR1 HST, max Doppler shifts which corresponds to fc*v/C in equation B.3.2.5 are selected as defined in Table B.3.2-1 for HST-SFN scenario. The updated version(v3) is uploaded to the RevisedCRs folder for review. Thank you! |
|  | Qualcomm: max Doppler shifts fd which corresponds to fc*v/C |
|  | Qualcomm: The change looks ok. However, it seems that the latest version of the spec is not used to prepare this CR because equation B.3.3.3 was recently fixed in past meetings (it was changed to k*Ds-Ds <= a < k*Ds + Ds) and that change is not present in this CR while it is available in June 2022 version of the spec. Can you please base you changes based on the latest version of the spec? |
|  | ZTE: Based on the email discussion, we agreed to add a note to the formula(B.3.2.5 and B.3.3. 5) instead of modifying the formula itself. The updated version(v2) is uploaded to the RevisedCRs folder, please final check, thanks! |
| R4-2213081 (Nokia) | Moderator: To be revised. @Nokia - Please inform companies when revision is available. |
| R4-2213640 (Qualcomm) | Moderator: To be revised. @Qualcomm - Please inform companies when revision is available. |
|  | Qualcom: We have uploaded the revised version of R4-2213640 at: revised_R4-2213640_draftCR_PDSCH_CA_Rel16_v1.docx |
| R4-2211547 (Anritsu) | Moderator: One company requested additional clarification. @Anritsu - Please respond to Qualcomm’s comment and check if revision is needed. If CR is going to be revised, please also update the attached Excel for completeness. Please trigger separate email thread for discussion. |
|  | Anritsu: Anritsu: Our understanding of the CSI RMC is that the number of PDSCH symbol is 12 as shown below. Thus based on our reference timing diagram, assuming 4ms delay from the reception of CSI-Report in Slot #18, PDSCH will fall in Slot #26 which is a DL slot. |
|  | Qualcomm: Thank you for the clarification. We are ok with this explanation. |

# Topic #2: Rel-17 UE Demod Maintenance

## Companies’ contributions summary

| TDoc Number | Company | Proposals / Observations |
| --- | --- | --- |
| 1024QAM |  |  |
| R4-2212887 | Ericsson | Summary of PDSCH simulation results for DL 1024QAM in FR1 |
| R4-2212888 | Ericsson | CR: Correction of 1024QAM UE demodulation and CQI reporting requirements Removal of [] from SNR test points for CQI reporting test with 1024QAM CQI table. |
| R4-2213830 | Huawei,HiSilicon | Draft CR on FDD CQI reporting cases for 1024QAM (TS38.101-4, Rel-17) Remove square brackets for 1024QAM CSI requirements. |
| R4-2213913 | MediaTek inc. | Draft CR to TS38.101-4, PDSCH requirements for 1024QAM 1. Remove bracket for the PDSCH requirements for FDD 2Rx/4Rx and TDD 2Rx/4Rx 2. In Table 5.2.2.1.1-3, correct the propagation condition from TDLD30 to TDLD30-5 3. In Table 5.2.2.1.1-3 and Table A.3.2.1.1-17, modify the reference channel number from “R.PDSCH.1-13.1 FDD” to “R.PDSCH.1-17.1 FDD” |
| R4-2213963 | Qualcomm Incorporated | Correction CR for 1024QAM UE Demod Requirements Correct 1024 QAM requirements line in Table 5.2.2.1.1-3 (PDSCH FDD Rank 1), missing doppler spread value in the propagation condition column; Remove square brackets where necessary |
| FR1 HST |  |  |
| R4-2211931 | CMCC | draftCR on FR1 HST-SFN CA for 2RX •Specify PDSCH requirements for BW of {35, 45} MHz for HST-SFN CA with 2RX. •Remove square brackets |
| R4-2212886 | Ericsson | draft CR: Correction of PDSCH demodulation requirements with HST Configure the correct active PDSCH TCI states. |
| R4-2213843 | Huawei,HiSilicon | Draft CR on HST FR1 CA requirements (TS38.101-4, Rel-17) Remove square brackets for HST FR1 CA requirements, correct the wrong calculated SNR requirements. |

## Open issues summary

None

## Companies views’ collection for 1st round

### Open issues

N/A

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| 1024QAM |  |
| R4-2212888 (Ericsson) | Qualcomm: We are ok with the correction |
|  | Moderator: Recommend to merge with formal CR from Qualcomm R4-2213963 to save workload. |
| R4-2213830 (Huawei) | Ericsson: Correction is OK, but it should be the formal CR. |
|  | Qualcomm: We are ok with the correction |
|  | Moderator: Recommend to merge with formal CR from Qualcomm R4-2213963 to save workload. |
| R4-2213913 (MediaTek) | Ericsson: Correction is OK, but it should be the formal CR. |
|  | Qualcomm: We are ok with the correction; |
|  | Apple: SNR values to be updated based on latest simulation results summary. |
|  | Moderator: Recommend to merge with formal CR from Qualcomm R4-2213963 to save workload. |
| R4-2213963 (Qualcomm) | Moderator: Same changes as in R4-2212888, R4-2213830, R4-2213913. Recommend to merge. |
|  | Qualcomm: changes are covered already in other CRs, we are ok to merge; |
|  | Moderator: After offline check and confirmation on meeting guidelines, recommend revising this CR with changes from R4-2212888, R4-2213830, R4-2213913 and comments received to save workload. |
| FR1 HST |  |
| R4-2211931 (CMCC) | Moderator: Overlapped with some updates in R4-2213843. Recommend to merge with R4-2213843. |
|  | CMCC: we are OK to merge to R4-2213843 |
| R4-2212886 (Ericsson) | Company A |
|  | Company B |
| R4-2213843 (Huawei) | Qualcomm: We think that the correction in the requirements is not needed. If we look at the simulation results summary from the last meeting (R4-2209069) and remove the farthest results until we get the span within 2.5dB (as agreed in the past), then existing requirements are correct. Replacing TBA and removing [] is fine. |
|  | Huawei: Thanks for your comments. After double check, we show our calculation step as following. We are not sure if there is somewhere the calculation is different. For SFN 2Rx FDD 15kHz with 40MHz case, the original span is 3.00. For calculating the farthest results, if we remove 12.60 then the span is 1.84 and if we remove 9.6 then the span is 1.70, so 9.6 should be removed. After that, the finally result is 14.6 rather than 14.3. 11; 11.44; 10.94; 12.60; 9.6; 10.9. For SFN 4Rx TDD 30kHz with 5MHz case, the original span is 2.78. For calculating the farthest results, if we remove 9.33 then the span is 2.45 and if we remove 6.55 then the span is 2.63, so 9.33 should be removed. After that, the finally result is 11.0 rather than 12.0. 9; 6.55; 9.33; 6.7; 8.8; 7.9. For SFN 4Rx TDD 30kHz with 15MHz case, the original span is 2.61. For calculating the farthest results, if we remove 9.7 then the span is 2.54 and if we remove 7.09 then the span is 2.60, so 9.7 should be removed. Then, if we remove 9.63 then the span is 1.81 and if we remove 7.09 then the span is 2.53, so 9.63 should be removed. After that, the finally result is 10.9 rather than 12.3. 9.7; 7.09; 9.63; 7.1; 8.90; 7.9. |

## Summary for 1st round

### Open issues

N/A

### CRs/TPs

Provided in Section 3.1

| R4-2213843 (Huawei) | Moderator: Please refer to updated version of R4-2209069 in Round 1 folder added for information showing calculation of final SNR reqt. Companies - please check if calculation is correct. @Huawei please update the requirements accordingly. |
| --- | --- |

## Discussion on 2nd round

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| 1024QAM |  |
| R4-2213963 (Qualcomm) | Moderator: To be revised. @Qualcomm - Please inform companies when revision is available. |
|  | Qualcomm: We have uploaded a revised version of R4-2213963 in the revised CR folder, merging other endorsed draftCR according to relative comments received. Companies are encouraged to review and comment here below. |
| FR1 HST |  |
| R4-2213843 (Huawei) | Moderator: To be revised. @Huawei - Please inform companies when revision is available. |
|  | Huawei: We have upload our revised R4-2213843 Draft CR on HST FR1 CA requirements (TS38.101-4, Rel-17).docx into the draft inbox. The mainly change is to modify the SNR for requirements based on the latest simulation results summary. |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | Summary for FR1 HST demodulation results | Apple | Updated requirement SNR derivation. |

Existing tdocs

| TDoc Number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| Rel-15/16 Maintenance |  |  |  |  |  |
| R4-2211571 |  | Addition of clarification for SIB1 scheduling in RMCs | Rohde & Schwarz | Agreeable |  |
| R4-2211582 |  | Correction of aperiodicTriggeringOffset | Rohde & Schwarz | Agreeable |  |
| R4-2212335 |  | Corrections on Rel-16 NR V2X PSCCH decoding capability | MediaTek Inc. | Agreeable |  |
| R4-2212636 |  | draft CR to TS 38.104[R16]_Corrections on the Doppler shift equation for kth RRH in Annex B | ZTE Corporation | Return to | Address concerns from companies |
| R4-2213077 |  | Correction to table headers for PBCH TDD demodulation requirements | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2213080 |  | Corrections to Tables A.4-1 and A.4-2 Rel-15 | Nokia, Nokia Shanghai Bell | Not pursued |  |
| R4-2213081 |  | Corrections to Tables A.4-1 and A.4-2 Rel-16 | Nokia, Nokia Shanghai Bell | Revised | Address comments from companies |
| R4-2213640 |  | Draft CR on Correction in PDSCH CA Tests | Qualcomm Incorporated | Revised | Address comments from companies |
| R4-2213743 |  | Draft CR on Correction in PDSCH CA Tests Cat-A CR | Qualcomm Incorporated |  |  |
| R4-2213910 |  | Draft CR to TS38.101-4, correction to CQI requirements for Table 3 (Rel-16) | MediaTek inc. | Agreeable |  |
| R4-2213911 |  | Draft CR to TS38.101-4, TDD UL-DL pattern for HST-DPS (Rel-16) | MediaTek inc. | Agreeable |  |
| R4-2211547 |  | Draft CR to CQI-PMI-RI Delay in 6.2A.3.1.1 | Anritsu Corporation | Return to | Address concern from Qualcomm |
| 1024QAM |  |  |  |  |  |
| R4-2212887 |  | Summary of PDSCH simulation results for DL 1024QAM in FR1 | Ericsson | To be noted when available |  |
| R4-2212888 |  | CR: Correction of 1024QAM UE demodulation and CQI reporting requirements | Ericsson | Merged | Merged to R4-2213963 |
| R4-2213830 |  | Draft CR on FDD CQI reporting cases for 1024QAM (TS38.101-4, Rel-17) | Huawei,HiSilicon | Merged | Merged to R4-2213963 |
| R4-2213913 |  | Draft CR to TS38.101-4, PDSCH requirements for 1024QAM | MediaTek inc. | Merged | Merged to R4-2213963 |
| R4-2213963 |  | Correction CR for 1024QAM UE Demod Requirements | Qualcomm Incorporated | Revised | Update requirements based on latest summary and merge changes from other CRs |
| FR1 HST |  |  |  |  |  |
| R4-2211931 |  | draftCR on FR1 HST-SFN CA for 2RX | CMCC | Merged | Merged to R4-2213843 |
| R4-2212886 |  | draft CR: Correction of PDSCH demodulation requirements with HST | Ericsson | Agreeable |  |
| R4-2213843 |  | Draft CR on HST FR1 CA requirements (TS38.101-4, Rel-17) | Huawei,HiSilicon | Revised | Update the SNR requirements based on updated calculations. |

## 2nd round

| TDoc Number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| Rel-15/16 Maintenance |  |  |  |  |  |
| R4-2212636 | R4-2215128 | draft CR to TS 38.104[R16]_Corrections on the Doppler shift equation for kth RRH in Annex B | ZTE Corporation | Agreeable |  |
| R4-2213081 | R4-2214813 | Corrections to Tables A.4-1 and A.4-2 Rel-16 | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2213640 | R4-2214835 | Draft CR on Correction in PDSCH CA Tests | Qualcomm Incorporated | Agreeable |  |
| R4-2211547 |  | Draft CR to CQI-PMI-RI Delay in 6.2A.3.1.1 | Anritsu Corporation | Agreeable |  |
| 1024QAM |  |  |  |  |  |
| R4-2213963 | R4-2214552 | Correction CR for 1024QAM UE Demod Requirements | Qualcomm Incorporated | Agreeable |  |
| FR1 HST |  |  |  |  |  |
| R4-2213843 | R4-2214852 | Draft CR on HST FR1 CA requirements (TS38.101-4, Rel-17) | Huawei,HiSilicon | Agreeable |  |
| R4-2214356 |  | Summary for FR1 HST demodulation results | Apple | Noted |  |