3GPP TSG-RAN4 Meeting #104-e	R4-2214892

# Electronic meeting, Aug. 15 – Aug. 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test set 3-5: PRS-RSRP reporting delay test case with reduced number of samples in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - PRS-RSRP measurement requirements for reduced number of samples are defined in R17 ePOS WI and the corresponding test case needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce test set 3-5, PRS-RSRP measurement reporting delay test case for reduced number of samples in FR1 in TS 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRS-RSRP measurement reporting delay test case for reduced number of samples in FR1 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.6.13.X1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.6.13.X1	PRS-RSRP reporting delay test case for reduced number of samples

A.6.6.13.X1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements for reduced number of samples specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.13.X1.1-1.

Table A.6.6.13.X1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.X1.1-2, and cell specific test parameters are listed in Table A.6.6.13.X1.1-3.

Table A.6.6.13.X1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.X1.1-3: Cell specific test parameters

| Parameter | Unit |  | Test configuration |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | 1 |  | N/A |  | N/A |  |
|  |  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  |  | 1 |  | SR.1.1 FDD |  | N/A |  |
|  |  |  | 2 |  | SR.1.1 TDD |  |  |  |
|  |  |  | 3 |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 |  | CR.1.1 FDD |  | N/A |  |
|  |  |  | 2 |  | CR.1.1 TDD |  |  |  |
|  |  |  | 3 |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | 1 |  | CCR.1.1 FDD |  | N/A |  |
|  |  |  | 2 |  | CCR.1.1 TDD |  |  |  |
|  |  |  | 3 |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  | 1, 2, 3 |  | OP.1 |  | OP.1 |  |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  |  | 1 |  | TRS.1.1 FDD |  | N/A |  |
|  |  |  | 2 |  | TRS.1.1 TDD |  |  |  |
|  |  |  | 3 |  | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  |  | 1, 2, 3 |  | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  |  | 1, 2, 3 |  | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  |  | 1, 2, 3 |  | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  |  | 1 |  | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  |  | 2 |  | PRS.1.1 FR1 |  | PRS.1.1 FR1 |  |
|  |  |  | 3 |  | PRS.2.1 FR1 |  | PRS.2.1 FR1 |  |
| PRS BW |  | 1 | 48 PRBs |  | 48 PRBs |  |
|  |  | 2 | 48 PRBs |  | 48 PRBs |  |
|  |  | 3 | 48 PRBs |  | 48 PRBs |  |
| PRS muting info |  |  | 1, 2, 3 |  | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS |  | 1 |  | -98 |  |  |  |
|  |  |  | 2 |  | -98 |  |  |  |
|  |  |  | 3 |  | -95 |  |  |  |
| Note 2 | dBm/15 kHz |  | 1 |  | -98 |  |  |  |
|  |  |  | 2 |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |
| PRS | dB |  | 1 |  | -Infinity | -1.76 | -Infinity | -6.01 |
|  |  |  | 2 |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |
| PRS | dB |  | 1 |  | -Infinity | 0 | -Infinity | -3 |
|  |  |  | 2 |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |
| PRP Note 3 | dBm/SCS kHz |  | 1 |  | -Infinity | -98 | -Infinity | -101 |
|  |  |  | 2 |  | -Infinity | -98 | -Infinity | -101 |
|  |  |  | 3 |  | -Infinity | -95 | -Infinity | -98 |
| Io | dBm/9.36 MHz |  | 1 |  | N/A | -66.07 | N/A | -66.07 |
|  | dBm/9.36 MHz |  | 2 |  |  | -66.07 |  | -66.07 |
|  | dBm/38.16 MHz |  | 3 |  |  | -59.97 |  | -59.97 |
| Propagation Condition |  |  | 1, 2, 3 |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

A.6.6.13.X1.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit for reduced number of samples specified in clause 9.9.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change 1>