3GPP TSG-RAN WG4 Meeting #104bis-e                              	                          R4-2215363

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	9.1

Source:	Intel Corporation

Title:	Analysis and summary of specification impacts of RAN4 options for FG 6-1a support

Document for:	Discussion

- Introduction

In the latest RAN Plenary meeting #96-e, clarification on the solution of FG 6-1a support was discussed and RAN reached the agreement with the following meeting minutes [1].

Conclusion in RAN Plenary #96-e • No new solution for FG 6-1a shall be added to Rel-17 • If CSI-RS based RLM/BM/BFD are supported by a UE, FG 6-1a can work without any issue. FG1-7 (CSI-RS based RLM) and FG 2-24 (SSB/CSI-RS for beam measurement) are mandatory with capability signalling features. • No change to TU allocation for current RAN4 works in Q4 2022. • RAN asks RAN4 to do a high-level analysis of the options (copied below) in RAN4’s answer to Q2 in RP-221911 and report it to RAN#98 for RAN decision. Options from RP-221911: a) Perform BM/RLM/BFD based on CSI-RS within active BWP b) Perform BM/RLM/BFD based on SSB outside active BWP i) UE’s capability to operate using larger BW covering SSB outside active BWP, or a UE that is equipped with a separate RF chain ii) BM/RLM/BFD on SSB outside BWP are performed with shared MG or NCSG for L3 measurement, or dedicated MG or NCSG for RLM/BFD/BM measurements. c) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

- This paper discusses the specification impacts of each option and provides our views.

- Discussion

A fundamental motivation of BWP is UE power saving while still allowing flexibility to fully utilize the entire available RF channel.  As we consider the options discussed in the previous RAN4 #104 meeting and summarized in the conclusion from RAN above we see trade-offs that affect this utility of the BWP methodology.

The first method,  a) using CSI-RS within the active BWP, is ideal from a BWP perspective.  For this case, the CSI-RS can be located within any active BWP and full flexibility is preserved.

For the methods under b) using the SSB outside the active BWP, there are some trade-offs.  FG 6-1 UEs with small BWP are currently limited to being centered on (CD) SSB. This results in inefficiency in resource utilization from gNB point of view as only BWP co-located with the CD-SSB can be used.  On the other hand, FG 6-1a devices can offer full use of the spectrum, monitoring both the active BWP and the reference signal, but often require an additional LO chain or LO retuning with a MG which may consume additional UE power.  Alternatively, FG 6-1a devices can simply utilize a wide bandwidth, so as to cover the target BWP an SSB located outside the active BWP.  This solution also consumes considerable additional UE power.

The third method discussed, c) NCD-SSB approach would allow the active BWP to be placed away from the CD-SSB allowing full utilization of the available RF channel at the cost of NCD-SSB resource overhead at gNB side. The additional benefit would be that no additional UE power consumption or complexity would be required for an additional LO or LO re-tuning or operating on the full RF channel bandwidth.  Finally, a third benefit to the CD-SSB approach, is that it works with both FG 6-1 and FG 6-1a devices.

FG 6-1 is mandatory feature w/o capability signalling while FG 6-1a is optional capability with field name “bwp-WithoutRestriction “and parent IE “BandNR” [2] and thus they can be signalled independently per supported RF band. While FG 6-1a capability provides additional flexibility, many UEs in the field do not support this additional capability.  The additional requirement of FG 6-1a is the ability to track of reference signals outside of the active BWP for BM/RLM/BFD as well as time-freq. tracking.

| Index | Feature group | Components |
| --- | --- | --- |
| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell |

## BM/RLM/BFD based on CSI-RS within active BWP

### Related UE capability

As pointed out in RAN meeting summary, FG6-1a with CSI CSI-RS based RLM/BM/BFD can work today without issue, and FG1-7 (CSI-RS based RLM) and FG 2-24 (SSB/CSI-RS for beam measurement) are mandatory with capability signalling. However, it is also necessary to check component details in FG 2-24 with RAN1/RAN2. It seems that either component-1, component-2 or component-3 should have non-zero value to enable correct CSI-RS based BM operation as shown in  Table 1. For FG 2-31, there is no issue since component-1 of Max # of CSI-RS over all CCs for BFD is non-zero value if BFR supported for FR1. FG 2-24, 2-31 and 2-50 have parent IE of “MIMO-ParametersPerBand”[2].

Observation 1: Some components of FG 2-24 can have zero value while FG 2-24 is mandatory with capability signalling. If UE reports a zero value for one of these components, then CSI-RS based BM cannot work.

Proposal 1: Report to RAN#98 that UEs supporting FG 6-1a with CSI-RS based operation need to report FG 2-24 capability with non-zero values for the component 1, 2 and 3.

One more capability to check for CSI-RS based operation is  CSI-RS for time-freq. tracking i.e TRS. From the capability point of view, it is defined as mandatory, and no further discussion is needed.

### Related RAN4 requirements

Considering the situation that CSI-RS for tracking (TRS) is the only RS within active BWP for timing tracking under FG 6-1a setting, RAN4 should consider whether there is a need to define the conditions for TRS in terms of BW and periodicity to meet the existing timing error requirements based on SSB.

| Feature | Related Requirements in TS 38.133 [3] |
| --- | --- |
| BM | 9.5.4.1 SSB based L1-RSRP Reporting 9.5.4.2 CSI-RS based L1-RSRP Reporting 9.5.5.1 Measurement restriction for SSB based L1-RSRP 9.5.5.2 Measurement restriction for CSI-RS based L1-RSRP |
| RLM | 8.1.2 Requirements for SSB based radio link monitoring 8.1.3 Requirements for CSI-RS based radio link monitoring |
| BFD | 8.5.2 Requirements for SSB based beam failure detection 8.5.3 Requirements for CSI-RS based beam failure detection 8.5.5 Requirements for SSB based candidate beam detection 8.5.6 Requirements for CSI-RS based candidate beam detection |
| UE transmit timing | 7.1.2 (UE transmit timing) Requirements .Meet Te requirements under the condition that at least one SSB available during the last 160 ms |

Proposal 2: RAN4 to discuss the necessity of “Introduction of timing requirements based on CSI-RS” to meet the existing UE transmit error requirements for FG 6-1a support with CSI-RS based solution. If agreed that new requirements are needed, then report the conclusion to RAN #98.

Table 1. Feature group related with FG 6-1a

| Index | Feature group | Components | Field name in TS 38.331 | Mandatory / Optional |
| --- | --- | --- | --- | --- |
| 1-7 | CSI-RS based RLM | CSI-RS based RLM | csi-RS-RLM | Mandatory with capability signalling |
| 2-24 | SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | beamManagementSSB-CSI-RS { 1. maxNumberSSB-CSI-RS-ResourceOneTx 2. maxNumberCSI-RS-Resource 3. maxNumberCSI-RS-ResourceTwoTx 4. supportedCSI-RS-Density 5. maxNumberAperiodicCSI-RS-Resource } | Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-4: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} On FR2, UE is mandated to signal either "3 only" or "both 1 and 3" On FR1, either "3 only" or "both 1 and 3" is mandatory with UE capability signalling. Component-5, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4 |
| 2-31 | Beam failure recovery | 1) Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2) Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3) Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. | 1. maxNumberCSI-RS-BFD 2. maxNumberSSB-BFD 3. maxNumberCSI-RS-SSB-CBD | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} UE is mandated to support at least 32 for FR2 |
| 2-50 | Basic TRS | 1) Support of TRS (mandatory) 2) All the periodicity are supported. 3) Support TRS bandwidth configuration as both "BWP" and "min(52, BWP)" |  | Mandatory without capability signalling |
| 2-51 | TRS (CSI-RS for tracking) | 1) TRS burst length (X), 2) Max # of TRS resource sets (per CC) UE is able to track simultaneously 3) Max # of TRS resource sets configured to UE per CC 4) Max # of TRS resource sets configured to UE across CCs | csi-RS-ForTracking { 1. maxBurstLength 2. maxSimultaneousResourceSetsPerCC 3. maxConfiguredResourceSetsPerCC 4. maxConfiguredResourceSetsAllCC } | Mandatory with capability signalling Component-1: candidate values {1, "both 1 and 2"}. UE is mandated to report "both 1 and 2" Component-2: Candidate value set: {1 to 8} Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. Component-4: Candidate value set: {1 to 256} UE is mandated to report at least 16 for FR1 and 32 for FR2. |

## BM/RLM/BFD based on SSB outside of active BWP

### 2.2.1 UE with wider RF bandwidth capability

The wider RF bandwidth capability for FG 6-1a would work with some changes in specification as below.

  - RAN1 is expected to relax active BWP confinement of BM/RLM/BFD SSB signals for FG 6-1a UEs under wider RF BW in their specifications. It is expected that wider RF is only for BM/RLM/BFD purpose and does not give impact on PRB size of PDSCH/PUSCH of the configured BWP.

  - RAN2 is expected to define the capability signalling on the supported RF BW for FG 6-1a purpose. In addition, it is expected that wider RF bandwidth is always activated as a baseline assumption.

For the given band, UE may implement single RF bandwidth to cover the whole band for testability and stability of RF filter characteristics. This RF capability can be used for BWP operation with smaller size than maximum supported BWP in the band. In these scenarios, UE can access SSB outside of active BWP at the discretion of UE intention under its own power consumption budget. The trade-off of this approach is that it degrades one of the fundamental motivations for using BWP – reduced power consumption. For situations where only a narrow BWP is needed, if the UE must monitor a wide RF bandwidth, then there is a considerable UE power increase.

Proposal 3: Report to RAN #8 that allowing a UE to utilize a wider RF bandwidth for 6-1a operation and only utilizing a single LO chain in band is feasible at the cost of higher UE power consumption.  It is possible for the UE to be willing to access SSB outside of active BWP under its own power consumption budget. The expected specification changes are as follows.

RAN1: Relax requirement that active BWP contains SSB for BM/RLM/BFD

RAN2: New UE capability for FG 6-1a with a component of wider RF BW signaling

RAN4: No specification changes expected

### 2.2.2 UE with additional RF chain capability

The increased RF chain capability for FG 6-1a would work with some changes in specification as    			below.

  - RAN4 may need to define how far active BWP is from the reference SSB without causing performance loss depending on RF architecture for additional RF chain utlization.

  - RAN1 is expected to relax active BWP confinement of BM/RLM/BFD SSB signals for FG 6-1a UEs under additional RF chain in their specifications. It is expected that additional RF chain is only for BM/RLM/BFD purpose and does not give impact on PRB size of PDSCH/PUSCH of the configured BWP.

  - RAN2 is expected to define the capability signalling on additional RF chain with BW for FG 6-1a purpose. It is also expected that this RF chain is to be always activated as a baseline assumption.

However, the additional RF chain would be used very small part of time and thus it is not desirable from RF complexity point of view. Depending on the RF acrhictecture the additional RF chain might be available for SSB monitoring in some band combinations but not available for others. The UE capability signalling could be defined in a way these dependencies are made available to the network, but it would add complexity to UE capability signalling and could impact the network's flexibility to use BWPs without SSB.

Proposal 4: Report to RAN #98 that from UE complexity perspective, it may not be desiable for UEs to support the “Additional RF chain capability” for control channel monitoring purpose since the additional RF would be used very small part of time. The expected specification changes are as follows.

RAN1: Relax requirement that active BWP contains SSB for BM/RLM/BFD

RAN2: New UE capability for FG 6-1a with a component of additional RF chain BW

RAN4: Potential requirements on separation between the active BWP and reference SSB depending on RF architecture for additional RF chain utilization

### 2.2.3 Measurement Gap

Introduction of measurement gap for BM/RLM/BFD would require changes in specification as below.

  - RAN4 need to define measurement gap related requirements for BM/RLM/BFD purpose as well as time-freq. tracking.

    - Shared MG or NCSG for L3 measurement, or dedicated MG or NCSG for RLM/BFD/BM measurements

    - For transmit timing requirements, at least 160 ms availability of SSB is needed.

  - RAN1 is expected to relax active BWP confinement of BM/RLM/BFD SSB signals for FG 6-1a UEs with configure gap in their specifications.

  - RAN2 is expected to define new capability signalling of  BM/RLM/BFD with gap for FG 6-1a purpose.

Gap-related requirements and specifically the gap sharing design is expected to require additional TU allocation in RAN4 work and different UE handling between UEs in BWP with SSB and UEs with gap may be a burden for gNB scheduler optimization. Moreover, it is expected to take much of time to reach the agreement on the design of gap configuration for BM/RLM/BFD whether reusing existing gap framework or introducing new dedicated framework.

Proposal 5: Report to RAN #98 the RAN4 concern on TU impact especially from the controversial topics regarding the design of gap configuration for BM/RLM/BFD whether reusing existing gap framework or introducing new dedicated framework.

## NCD-SSB approach

NCD-SSB is originally introduced in Rel-17 for RedCap UEs. The main purpose of NCD-SSB is for gNB’s scheduling flexibility from load balancing perspective. The same benefit can be obtained if NCD-SSB is applied to non-RedCap UE as well. Rigorously speaking, this approach is not directly related with BWP without restriction (i.e FG 6-1a) since NCD-SSB itself would be within the active BWP. Also, it is worthy of noting that SSB signals is much more popular than CSI-RS signals in RRM implementation.

### Related RAN4 requirements

Existing requirements with 2 Rx. RedCap UEs can be applied for non-RedCap UEs with NCD-SSB capability UEs including BWP change related timing requirements. Other requirements on BM/RLM/BFD as well as transmit timing error based on SSB are fully reused.

### Related RAN1/RAN2 specifications

  - Minor text changes on RAN1 specifications are required to extend BM/RLM/BFD operations with CD-SSB to NCD-SSB as well for non-RedCap UE with NCD-SSB capability.

  - Also, UE capability signalling on NCD-SSB support should be introduced for non-RedCap UEs

Proposal 6: Report to RAN #98 that NCD-SSB approach is an independent and separate capability from FG 6-1a although it can also achieve the same goal of FG 6-1a to give gNB’s scheduling flexibility from load balancing perspective. The expected specification changes are as follows.

RAN1: Relax requirement s.t active BWP contains both CD-SSB and NCD-SSB for BM/RLM/BFD

RAN2: New UE capability for NCD-SSB for non-RedCap UEs

RAN4: Minor text change to apply existing 2 Rx. RedCap UE requirements with NCD-SSB

# Conclusion

The following proposals are recommended for RAN4 to capture in the report to RAN. All options need minor text changes in RAN1 and RAN2 if required.

Observation 1: Some components of FG 2-24 can have zero value while FG 2-24 is mandatory with capability signalling. If UE reports a zero value for one of these components, then CSI-RS based BM cannot work.

Proposal 1: Report to RAN#98 that UEs supporting FG 6-1a with CSI-RS based operation need to report FG 2-24 capability with non-zero values for the component 1, 2 and 3.

Proposal 2: RAN4 to discuss the necessity of “Introduction of timing requirements based on CSI-RS” to meet the existing UE transmit error requirements for FG 6-1a support with CSI-RS based solution. If agreed that new requirements are needed, then report the conclusion to RAN #98.

Proposal 3: Report to RAN #8 that allowing a UE to utilize a wider RF bandwidth for 6-1a operation and only utilizing a single LO chain in band is feasible at the cost of higher UE power consumption.  It is possible for the UE to be willing to access SSB outside of active BWP under its own power consumption budget. The expected specification changes are as follows.

RAN1: Relax requirement that active BWP contains SSB for BM/RLM/BFD

RAN2: New UE capability for FG 6-1a supported with wider RF BW

RAN4: No specification changes expected

Proposal 4: Report to RAN #98 that from UE complexity perspective, it may not be desiable for UEs to support the “Additional RF chain capability” for control channel monitoring purpose since the additional RF would be used very small part of time. The expected specification changes are as follows.

RAN1: Relax requirement that active BWP contains SSB for BM/RLM/BFD

RAN2: New UE capability for FG 6-1a supported with wider RF BW

RAN4: Potential requirements on separation between the active BWP and reference SSB depending on RF architecture for additional RF chain utilization

Proposal 5:  Report to RAN #98 the RAN4 concern on TU impact especially from the controversial topics regarding the design of gap configuration for BM/RLM/BFD whether reusing existing gap framework or introducing new dedicated framework.

Proposal 6: Report to RAN #98 that NCD-SSB approach is an independent and separate capability from FG 6-1a although it can also achieve the same goal of FG 6-1a to give gNB’s scheduling flexibility from load balancing perspective. The expected specification changes are as follows.

RAN1: Relax requirement s.t active BWP contains both CD-SSB and NCD-SSB for BM/RLM/BFD

RAN2: New UE capability for NCD-SSB for non-RedCap UEs

RAN4: Minor text change to apply existing 2 Rx. RedCap UE requirements with NCD-SSB

- References

- RP-222630,  “Summary for [97e-05-BWP-WithoutRestriction]”

- 4.1 Layer-1 UE features, 3GPP TR 38.822 V16.3.0: “NR; User Equipment (UE) feature list “

- 3GPP TS 38.133 V17.5.0: “NR; Requirements for support of radio resource management “