3GPP TSG RAN WG4 #105			R4-2218051

Meeting, Nov 14th – 18th, 2022

Source:	Kumu Networks, INC.

Title:	SBFD feasibility study and RF impact on BS aspects

Agenda Item:	8.18.2

Document for: 	Discussion

# Introduction

At RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss on the feasibility and RF impact of the subband non-overlapping full duplex feature for NR duplex evolution.

# SBFD feasibility study and RF impact on BS aspects

# Overview

During meeting #104 and #104-bis-e discussions, some companies have expressed desire for more realistic and concrete RSIC values for self-cancellation performance with realistic transmit and receive link model.

In this contribution, we will discuss the benefits of using RF cancellation in handling 3rd order non-linearity in the low noise amplifier, and discuss the RSIC values achievable with frequency domain beam-forming, beam-nulling and RF cancellation with a set of randomly placed  uplink and downlink UEs.

Our simulation model is as follows:

Figure 1 Simulation model

The simulation model shows signal flow in black and beamforming / RF SIC algorithm optimization targets in red. More details on the optimization procedure can be found in [3].

# Self-interference modelling and Simulation Results

# Benefits of RF cancellation in the presence of LNA 3rd order non-linearity

Based on Qualcomm(R4-2215346)[5], Intel(R4-2216409)[6] and Ericsson(R4-2216404)[7], we could expect that spatial isolation would be between 65 dB to 80 dB depending on antenna panel type and configuration. We simulated the effect of worst case antenna isolation (65 dB) on the receiver front end LNA and the result is summarized as follows. We started with 1 DL user and 1 UL user and extend the simulation to clusters of users with frequency domain beamforming per 4 RB span. For detail of our setup, please reference our contribution (R1-2205815) [3] shared on RAN 1 meeting #110. Further hardware details can be found [4].

Using an antenna configuration of 32 TX and 32 RX, and a total 64 cancellers, 32 cancellers connect the i-th transmitter to the i-th receiver, and the other 32 cancellers connect i-th transmitter to (i+1)-th receiver, we first simulated the effect of residue self-interference on the receiver LNA with 1 fixed DL user and 1 fixed UL user.

| - Basic beamforming | - Beamforming + beamnulling without canceller | - Beamforming + beamnulling with canceller |
| --- | --- | --- |

Figure 2: 32+32 SBFD configuration: Combined self-interference blocker channels per RX (pre Rx LNA). Each line represents the TX beamformed channel at one of the 32 receivers. RF cancellation before Rx LNA mitigates saturation of the LNA.

Figure 2 compares the blocker rejection performance at each receiver, with transmit beamforming coefficients optimizing for a fixed downlink UE position, at the relevant TX frequency 3.66 GHz – 3.74 GHz. While the basic beamforming optimization without beamnulling performs the worst at -74.7dB (Figure 2(a)), adding beamnulling into the optimization objective without a canceller only marginally improves the isolation to -78.4dB (Figure 2(b)). With 55dBm transmit power, -78.4dB isolation results in a -23.4dBm blocker at the receiver, which would cause saturation on a LNA with IIP3 of -15 dBm as shown in Figure 3(a).

Adding RF cancellers and jointly optimizing with beamnulling, as shown in Figure 2(c), the isolation improves to -105.7dB, which results in -50.7dBm blocker power at the receiver in the worst case. Receivers can accommodate this blocker without affecting the signal integrity at the receiver frequency as shown in Figure 3(b).

No LNA Saturation

No LNA Saturation

LNA Saturation

LNA Saturation

| (a) Beamforming + beamnulling without canceller | (b)Beamforming + beamnulling with canceller |
| --- | --- |

Figure 3 RX LNA spectral plot showing the worst case and best case residue for 32+32 Beamforming + beamnulling with canceller for combined self-interference ACL channels per RX. The worst case residue is low enough that no saturation is observed at the output of the LNA with IIP3 of -15 dBm when beamnulling and RF cancellation is jointly performed. The purple line represent the worst case signal passing through an ideal LNA while the red signal represent the worst case signal passing through a LNA with IIP3 of -15 dBm.

| - Basic beamforming | - Beamforming + beamnulling without canceller | - Beamforming + beamnulling with canceller |
| --- | --- | --- |

Figure 4: 32+32 SBFD configuration: Combined self-interference ACL channels per TX. Each line represents the RX beamformed channel from one of the 32 transmitters.

Figure 4 shows the ACL rejection performance at the RX frequency, 3.74 – 3.76 GHz, from each transmitter with receiver beamforming optimizing for a fixed UL UE location. Like the blocker channel results, beamnulling alone cannot provide the needed isolation, as shown in Figure 4(b), because -89dB isolation would result in 55dBm TX Pwr – 45dB ACLR – 89dB isolation = -79dBm on-channel noise at the receiver which degrades the RX sensitivity significantly.

However, the joint optimization with cancellers achieves -107.3dB isolation (Figure 4(c)), which results in -97.3dBm on-channel noise. Given the noise floor for 100MHz is typically around -89 dBm (-94dBm thermal noise floor plus 5 dB receiver noise figure), the ACL residual impact on receiver sensitivity would be minimal.

Observation 1:

Having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability.

Proposal 1:

Given that RF SIC is capable of avoiding LNA saturation, we should include RF SIC as a separate line item in the RSI table.

# RF Cancellation and Residue Self-Interference

In RAN 4 meeting #104-bis-e, some companies expressed interest in more real world estimate of the RSI value. We further simulated with having multiple UL and DL UEs each with a BS-UE frequency selective channel and uses frequency domain beamforming. Each UE is allocated 4 RBs. A total of 53 DL UEs and 13 UL UEs are active at the same time.

| - Basic beamforming | - Beamforming + beamnulling without canceller | - Beamforming + beamnulling with canceller |
| --- | --- | --- |

Figure 5: 32+32 SBFD configuration: Combined self-interference blocker channels per RX (pre Rx LNA). Each line represents the TX beamformed channel at one of the 32 receivers. RF cancellation before Rx LNA mitigates saturation of the LNA.

Figure 5 compares the blocker rejection performance at each receiver, with transmit beamforming coefficients optimizing for 53 downlink UE position arranged in 10 random clusters, at the relevant TX frequency 3.66 GHz – 3.74 GHz. While the basic beamforming optimization without beamnulling performs the worst at -75.79dB (Figure 5 (a)), adding beamnulling into the optimization objective without a canceller only marginally improves the isolation to -77.62dB (Figure 5 (b)). With 55dBm transmit power, -77.62dB isolation results in a -22.62dBm blocker at the receiver, which would cause saturation on a LNA with IIP3 of -15 dBm similar to the situation as shown in Figure 3(a).

Adding RF cancellers and jointly optimizing with beamnulling, as shown in Figure 5(c), the isolation improves to -108.7dB, which results in -53.7dBm blocker power at the receiver in the worst case. This is well within the target of -43 dBm discussed in RAN4 meeting #104e[8]. Receivers can accommodate this blocker without affecting the signal integrity at the receiver frequency, similar to the situation as shown in Figure 3(b).

| - Basic beamforming | - Beamforming + beamnulling without canceller | - Beamforming + beamnulling with canceller |
| --- | --- | --- |

Figure 6: 32+32 SBFD configuration: Combined self-interference ACL channels per TX for 13 UL UEs, each occupying 4RB. Each line represents the RX beamformed channel from one of the 32 transmitters.

Figure 6 shows the ACL rejection performance at the RX frequency, 3.74 – 3.76 GHz, from each transmitter with receiver beamforming optimizing for 13 UL UE location, each occupying 4 RB. Like the blocker channel results, beamnulling alone cannot provide the needed isolation, as shown in Figure 6(b), because -78.98dB isolation would result in 55dBm TX Pwr – 45dB ACLR – 78.98dB isolation = -68.98dBm on-channel noise at the receiver which degrades the RX sensitivity significantly.

However, the joint optimization with cancellers achieves -105.6dB isolation (Figure 6 (c)), which results in -95.6dBm on-channel noise. Given the noise floor for 100MHz is typically around -89 dBm (-94dBm thermal noise floor plus 5 dB receiver noise figure), the ACL residual impact on receiver sensitivity would be minimal and the degradation in sensitivity due to RSI is less than 1 dB (0.7 dB).

Based on the above-described simulation results, we report the following table for RSIC capability for SBFD gNB operation:

| FR1 Wide-Area BS |  | Kumu Networks |
| --- | --- | --- |
| Component capability | Spatial isolation | 65 dB |
|  | Frequency isolation | 45 dB |
|  | Beam nulling /isolation | 14 dB |
|  | RF Cancellation | 27 dB |
|  | Digital IC | (may give further gain) |
| Overall RSIC capability |  | 151 dB |
| additional implementation details | SBFD configuration | DU |
|  | Guardband assumption (if exist) | None |
|  | Sub-band filtering assumption (if exist) | None |
|  | Bandwidth over which suppression is achieved | 100 MHz |
|  | RF SIC Complexity | 192 RF taps for 32 Tx and 32 Rx antennas |
|  | Others | 55 dBm Total Radiated Power |

Observation 2: With more complex frequency domain beamforming scenarios, RF cancellation gives a 151 dB overall RSIC capability and the resulting loss in sensitivity is less than 1 dB.

Proposal 2:

We proposed that 1 dB sensitivity degradation for feasibility evaluation simulation should be a good starting point as 150 dB or lower RSIC and -43 dBm or lower (-50 dBm is feasible when using RF SIC) pre LNA input power is achievable.

.

# RF SIC Complexity

Figure 7(b) shows a typical implementation of our proposed RF SIC scheme compared to a standard beamforming antenna panel in Figure 7(a). Each antenna element is connected to a T/R switch which chooses whether the antenna is connected to a PA or an LNA.  Figure 5(b) shows an example architecture for a simple 4+4 SBFD case with 8 cancellers. The 8 cancellers are arranged such that Tx1 is connected to Rx1 and Rx2, Tx2 to Rx2 and Rx3, and so on. Each RF cancellation tap is made up of a phase shifter and an attenuator.

In our 32x32 simulation discussed in 2.2.2, each Tx antenna is connected to 6 Rx antenna, using a total of 192 RF cancellation taps.

| Figure 7(a): Typical mMIMO frontend architecture | Figure 7(b): mMIMO frontend architecture with canceller |
| --- | --- |

# Conclusion

In this contribution, we discuss on the feasibility and RF impact of the subband non-overlapping full duplex feature for NR duplex evolution. In particular, our simulation shows the following RSI values.

| FR1 Wide-Area BS |  | Kumu Networks |
| --- | --- | --- |
| Component capability | Spatial isolation | 65 dB |
|  | Frequency isolation | 45 dB |
|  | Beam nulling /isolation | 14 dB |
|  | RF Cancellation | 27 dB |
|  | Digital IC | (may give further gain) |
| Overall RSIC capability |  | 151 dB |
| additional implementation details | SBFD configuration | DU |
|  | Guardband assumption (if exist) | None |
|  | Sub-band filtering assumption (if exist) | None |
|  | Bandwidth over which suppression is achieved | 100 MHz |
|  | RF SIC Complexity | 192 RF taps for 32 Tx and 32 Rx antennas |
|  | Others | 55 dBm Total Radiated Power |

Based on the discussion we made the following observations and proposals.

Observation 1:

Having RF cancellation before the receiver LNA have the benefits of achieving the desired self-interference cancellation residue floor as well as preventing saturation of the Rx LNAs. When considering the viability of SBFD, RF cancellation plays a critical part and should be considered in the evaluation of overall RSIC capability.

Observation 2: With more complex frequency domain beamforming scenarios, RF cancellation gives a 151 dB overall RSIC capability and the resulting loss in sensitivity is less than 1 dB.

Proposal 1:

Given that RF SIC is capable of avoiding LNA saturation, we should include RF SIC as a separate line item in the RSI table.

Proposal 2:

We proposed that 1 dB sensitivity degradation for feasibility evaluation simulation should be a good starting point as 150 dB or lower RSIC and -43 dBm or lower (-50 dBm is feasible when using RF SIC) pre LNA input power is achievable.

# References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- 3GPP, TS38.830 v17.0.0, Dec. 2020.

- 3GPP R1-2205815, Kumu Networks, “RF cancellation techniques for Subband non-overlapping full duplex”, RAN1 #110, Aug 2022

- “K4051 Canceller Reference Board”. https://kumunetworks.com/wp-content/uploads/2020/04/K4051-Canceller-Product-Brief.pdf

- 3GPP R4-2215346, Qualcomm, “SBFD feasibility and impact on RF requirements: BS aspects”, Oct 2022.

- 3GPP R4-2216409, Intel, “Discussion on Total Achievable Self-Interference Cancellation”, Oct 2022.

- 3GPP R4-2216404, Ericsson, “SBFD gNB Radio and antenna considerations”, Oct 2022.

- 3GPP R4-2217464, Samsung, “WF on SBFD feasibility study and RF impact: BS aspects”, Oct 2022.

Appendix

# Simulation Parameters

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Carrier Frequency | 4 GHz |
|  | Duplex Type | SBFD |
|  | SBFD pattern | 80:20 SBFD slot:UL |
|  | Channel bandwidth | 80:20 MHz (DU) for SBFD |
|  | Guard Band | 0 MHz |
|  | Available resource blocks | 223:55 (DU) for SBFD |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 66 |
|  | Channel model | gNB-UE: LOS + ground bounce |
| BS | Array Configuration | 32x32 |
|  | Max gNB Tx Power | 55 dBm (same antenna gain) |
|  | (dv, dh) | (0.5λ, 0.5λ) |
|  | Antenna element gain | 7 dBi |
|  | Beamforming method | Frequency domain |
|  | Antenna Isolation | 65 dB |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 10 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
|  | UE antenna | 1TX 1RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
| UE | SNR target | 16 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution | Random cluster |
|  | Traffic split | 80:20 DL:UL |