3GPP TSG-RAN WG4 Meeting # 105		R4-2218175

Toulouse, France, November 14 – November 18, 2022

Agenda item:	5.4

Source:	Apple Inc.

Title:	Simulation results for PDSCH demod requirements in ICI

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e PDSCH demodulation and CQI reporting requirements with MMSE-IRC for inter-cell interference was discussed and big CR [1] was approved to introduce PDSCH demodulation and CQI reporting requirements in ICI. In this contribution we provide updated simulation results for PDSCH demod requirements in ICI.

# 3. Simulation Results

PDSCH demod in ICI

For the simulation assumptions agreed for PDSCH demodulation requirements in ICI, we present simulation results below.

Table 2: Performance results for PDSCH demod in ICI

| Duplex | Ant. Config | Num. Inf Cells | Deployment | Prop. Channel | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| FDD | 2x2 | 1 | HetNet | TDLA30-10 | 10.6 | 12.6 |
|  |  | 2 | HomoNet | TDLC300-100 | 14 | 16 |
|  | 2x4 | 1 | HetNet | TDLA30-10 | 5.6 | 7.6 |
|  |  | 2 | HomoNet | TDLC300-100 | 8.9 | 10.9 |
| TDD | 2x2 | 1 | HetNet | TDLA30-10 | 10.8 | 12.8 |
|  |  | 2 | HomoNet | TDLC300-100 | 14.4 | 16.4 |
|  | 2x4 | 1 | HetNet | TDLA30-10 | 6 | 8 |
|  |  | 2 | HomoNet | TDLC300-100 | 9.2 | 11.2 |

# 4. Conclusion

In this paper, we provide simulation results for PDSCH demod requirements definition.

# Reference

- R4-2214363, “Big CR for inter-cell MMSE-IRC”, Apple.