3GPP TSG-RAN WG4 Meeting #105	R4-2218198

Toulouse, France, 14th November – 18th November 2022

Source:	Qualcomm Incorporated

Title:	TP updates for TR37.878

Agenda item:	7.14.2

Work Item:	          NR_LTE_V2X_PC5_combos_R18-Core

Document for:	Approval

## --Start of changes

***********************************Start of the TP*************************************

# 1	Scope

The present document is the Technical Report on TR on band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X band in Release 18.

The purpose of the present document is to study the extension of the band combinations for V2X service to grow the NR V2X ecosystem. Operators propose new band combinations for con-current operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X band. Whether to specify con-current operation of LTE/NR CA/DC band combinations and V2X band depends on requests in Rel-18. Specifically, the self-desensitization problem of con-current operation band combinations will be analysed including harmonics, IMD problem, etc.. Also the candidate solutions will be studied to solve the self-desensitization problem.

Table 1-1: Release 18 NR V2X band combinations

| V2X Band combination | REL independent from |
| --- | --- |
| V2X_n3-n47 | Rel-16 |
| V2X_n34-n47 | Rel-16 |
| V2X_n3_47 | Rel-16 |
| V2X_n34_47 | Rel-16 |
| V2X_34_n47 | Rel-16 |

Note: All band combinations in table 1-1 that are release independent from Rel-16 are optional

--End of changes

# 6	Con-current operation with one Uu band and one PC5 band

## 6.2	Con-current operation between one NR Uu band and one NR PC5 band

<<Unchanged section omitted>>

### 6.2.2	V2X_n3A-n47A

## --Start of changes

#### 6.2.2.4	MSD, ∆TIB and ∆RIB values

Table 6.2.2.4-1: ΔTIB,c for inter-band con-current V2X operation (two bands)

| V2X con-current band Combination | NR Operating Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_n3A-n47A | n3 | 0.0 |

Table 6.2.2.4-2: ΔRIB,c for inter-band con-current V2X operation (two bands)

| V2X inter-band con-current band Combination | NR Operating Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n3A-n47A | n3 | 0.0 |

--End of changes

## 6.3	Con-current operation between one NR Uu band and one LTE PC5 band

<<Unchanged section omitted>>

### 6.3.2	V2X_n3A_47A

## --Start of changes

#### 6.3.2.4	MSD, ∆TIB and ∆RIB values

Table 6.3.2.4-1: ΔTIB,c for inter-band con-current V2X operation (two bands)

| V2X con-current band Combination | NR Operating Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_n3A_47A | n3 | 0.0 |

Table 6.3.2.4-2: ΔRIB,c for inter-band con-current V2X operation (two bands)

| V2X inter-band con-current band Combination | NR Operating Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_n3A_47A | n3 | 0.0 |

--End of changes