3GPP TSG-RAN WG4 Meeting #105	R4-2218308

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	6.3.6.2.1

Source:	Ericsson

Title:	Simulation results for PDSCH demodulation in 52.6 – 71 GHz band.

Document for:	Information

# 1	Introduction

In this paper and based on the progress summarized in WF [1], we update our simulation results on the PDSCH performance in the frequency range 52.6 GHz to 71 GHz. We carried out our simulations using the agreed channel models and parameters.

Agreement on PDSCH Requirement Table SCS (KHz)/ CBW (MHz); MCS; Propagation Channel; Antenna Conf.; Throughput; Num PRB. 120kHz/100MHz; MCS4; TDLA30-650; 2x2 Low; 70%; 66. MCS13; TDLA30-200; 2x2 Low; 70%; 66. MCS13; TDLA30-650; 2x2 Low; 30%; 66. MCS17; TDLD30-200; 2x2 Low; 70%; [66, 32]. 480kHz/400MHz; MCS4; TDLA10-200; 2x2 Low; 70%; 66. MCS13; TDLD10-200; 2x2 Low; 70%; [20,32]. Agreeable Extended maximum Testable SNR for reduced allocation CBW (MHz); Num RBs; Test method. IFF. Single band UE; 100; 66; [9.8]. [32]; [13.2]. 400; 66; [2.6]. [32]; [6.6]. [20]; [8.9].

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

As per WF [1], we carried out simulations using the agreed propagation environments. Furthermore, RAN4 was not able to agree on a single representative phase noise (PN) model during the study item. Therefore, and for the sake of fairness, we performed our simulations considering both PN models in TR 38.808 [2], referring to as Set 1 and Set 2, where Set 1 has been proposed in [3] and Set 2 also known as TR 38.803 Example 2.

We will use as general parameters.

- Numerologies:

- SCS = 120 KHz, CBW = 100 MHz (66 PRBs and [32] PRBs).

- SCS = 480 KHz, CBW = 400 MHz (66 PRB and [20, 32] PRB).

- MCS 4, 13, and 17

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 200 Hz), DMRS 1+1 ((2,11) symbol indexes)

- UE speed at 10 Km/h (f_D = 650 Hz), DMRS 1+1 ((2,11) symbol indexes)

- Two PN models in TR 83.808 [2]:

    - Set 1 in TR 38.808: PN model in [3], referred to as R4_2010176

The design margin (DM) should be set to 0 dB for BS while to be set to 5 dB for UE

    - Set 2 in TR 38.808: PN model TR38.803 Ex2, Set 2 in TR 38.808

- Carrier frequency: 70 GHz

- Antenna configuration: 2x2 ULA Low

- Rank 1

- Rel-15 PTRS configuration, K = 2, L = 1

- Phase noise at the UE receiver only

- Benchmark: No PN and No compensation

- CPE compensation only

We summarize the parameters above in the following table

70% Peak Throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

30% Peak throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |
| NOH | 6 |
| Number of HARQ Processes | 16 |

## 2.2 Simulation results

In this section, we will present the normalized PDSCH throughput as function of the signal to noise ratio (SNR). As per RAN4 PDSCH metric, we will refer to SNR1 and SNR2 as the corresponding SNR value to 70% (or 30 %) of the peak throughput value when using PN models Set 1 and Set 2, respectively.

Furthermore, it is worth mentioning that considering the PRB number allocation, we only apply CPE compensation. In the sequel, we present the simulation setup tables that we have used.

Test cases:

- CBW (MHz)/SCS (KHz) = 100/120, TDLA30-200/650 and TDLD30-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 70 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| Test 1-1 | 70 | 100/120 (66) | 4 | 1 | TDLA30-650 | CPE | 2x2 ULA Low | TBD | TBD |
| Test 1-2 | 70 | 100/ 120 (66) | 13 | 1 | TDLA30-200 | CPE | 2x2 ULA Low |  |  |
| Test 1-3 | 70 | 100/ 120 (66) | 17 | 1 | TDLD30-200 | CPE | 2x2 ULA Low |  |  |
| Test 1-4 | 70 | 100/ 120 (32) | 17 | 1 | TDLD30-200 | CPE | 2x2 ULA Low |  |  |

- CBW (MHz)/SCS (KHz) = 400/480, TDLA10-200 and TDLD10-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 70 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| Test 2-1 | 70 | 400/480 (66) | 4 | 1 | TDLA10-200 | CPE | 2x2 ULA Low | TBD | TBD |
| Test 2-2 | 70 | 400/480 (20) | 13 | 1 | TDLD10-200 | CPE | 2x2 ULA Low |  |  |
| Test 2-3 | 70 | 400/480 (32) | 13 | 1 | TDLD10-200 | CPE | 2x2 ULA Low |  |  |

- Metric 30% Peak Throughput, HARQ processes: 16

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 30 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| Test 3-1 | 70 | 100/120 (66) | 13 | 1 | TDLA30-650 | CPE | 2x2 ULA Low | TBD | TBD |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models and MCSs but limited to 1 MIMO layer (Rank 1). | 1-1, 1-2, 1-3, 1-4, 2-1, 2-2, 2-3. |
| 30% of peak Throughput | Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 3-1 |

## 2.2.1 SCS/CBW = 120 KHz/100 MHz results

Considering our benchmark, when no PN applied at both sides (ideal case), the achieved SNRs at 70% of the peak throughput have been referred to as SNR0 and gathered along with SNR1 and SNR2 in Tables 1.1 and 1.2. Furthermore, in the sequel, we will refer to D1 = SNR1 – SNR0 and D2 = SNR2 – SNR0 as the SNR gap (dB) between the PN CPE compensation output and the NO PN case.

For 120 KHz SCS and 100 MHz CBW, Table 1.1 gathers the results where 66 PRBs have been allocated and MCS (4, 13 and 17) have been considered where the max DL testable SNR level is at [9.8] dB as well as considering MCS 17 with partial PRB allocation [32], where the max DL testable SNR level is at [13.2] dB.

Table 1.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for MCS (4, 13, and 17) and using 66 PRBs.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-1 | 4 (66) | TDLA30-650 | -3.1 | -2.9 | 0.2 | -2.8 | 0.3 |
| Test 1-2 | 13 (66) | TDLA30-200 | 4.5 | 4.8 | 0.3 | 5.1 | 0.6 |
| Test -3 | 17 (66) | TDLD30-200 | 4.2 | 4.7 | 0.5 | 5.1 | 0.9 |
| Test 1-4 | 17 (32) | TDLD30-200 | 4.0 | 4.6 | 0.6 | 5.0 | 1.0 |

Observation 1: Referring to Table 1.1, and considering SCS/CBW = 120 KHz/100 MHz with full PRB allocation for MCS (4, 13 and 17) and CPE for PN compensation, we note that

- MCS 4, MCS 13 and MCS 17 are testable using full RB allocation under the agreed propagation environments and considering a margin of 3 dB for RF impairments.

## 2.2.2 SCS/CBW = 480 KHz/400 MHz results

Following same arguments as in Section 2.2.1, the results have been gathered in Tables 2.1 where both the full and partial PBR allocation have been considered. It is worth mentioning here the max DL testable SNR is at [2.6] dB when 66 RBs have been considered, while the max DL testable SNRs are at [8.9] dB and [6.6] dB, when considering 20 RBs and 32 RBs, respectively.

Table 2.1 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for low MCS (4 and 13).

|  | CBW (MHz) / SCS (KHz) = 400/480 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 2-1 | 4 (66) | TDLA10-200 | -3.1 | -2.9 | 0.2 | -2.8 | 0.3 |
| Test 2-2 | 13 (20) | TDLD10-200 | 0.4 | 0.7 | 0.3 | 0.8 | 0.4 |
| Test 2-3 | 13 (32) | TDLD10-200 | 0.6 | 1.0 | 0.4 | 1.0 | 0.4 |

Observation 2: Referring to Table 2.1, and considering SCS/CBW = 480 KHz/400 MHz with full and partial PRB allocation for MCS 4 and 13, respectively, with CPE for PN compensation, we note that

- MCS 4 is testable using full RB allocation under TDLA10-200 and considering a margin of 3 dB for RF impairments.

- MCS 13 is testable using partial RB allocation of 32 under TDLD10-200 and considering a margin of 3 dB for RF impairments.

Observation 3: As a general conclusion, the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation and considering an additional margin of 3 dB for RF impairments are

- 120 KHz/100 MHz: MCS 17 with full RB allocation under TDLD30-200

- 480 KHz/400 MHz: MCS 13 with partial RB allocation of 32 under TDLD10-200

## 2.2.3 Reference SNRs at 30% of the peak Throughput

In this section, we evaluate the reference SNRs at 30% of the peak throughput for 120 kHz SCS considering MCS 13 and full PRB allocation (66). The results have been gathered in Tables 3.1 where CPE compensation only was used.

Table 3.1 - Reference SNR values whether PN has been considered or not for SCS 120 kHz/ CBW 100 MHz for MCS 13.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 30% Max Throughput | SNR1 (dB) @ 30% Max Throughput | D1 (dB) | SNR2 (dB) @ 30% Max Throughput | D2 (dB) |
| Test 3-1 | 13 (66) | TDLA30-650 | -1.4 | -1.3 | 0.1 | -1.2 | 0.2 |

Observation 4: Referring to Table 3.1, when considering performance metric at 30% of the peak throughput, for 120 KHz/100 MHz and 480 KHz/400 MHz using MCS 13, it has been noted that

- 120 kHz/100 MHz: MCS 13 is testable under TDLA30-650 at 30% of peak throughput using full RB allocation and considering a margin of 3 dB for RF impairments.

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Referring to Table 1.1, and considering SCS/CBW = 120 KHz/100 MHz with full PRB allocation for MCS (4, 13 and 17) and CPE for PN compensation, we note that

MCS 4, MCS 13 and MCS 17 are testable using full RB allocation under the agreed propagation environments and considering a margin of 3 dB for RF impairments.

Observation 2: Referring to Table 2.1, and considering SCS/CBW = 480 KHz/400 MHz with full and partial PRB allocation for MCS 4 and 13, respectively, with CPE for PN compensation, we note that

- MCS 4 is testable using full RB allocation under TDLA10-200 and considering a margin of 3 dB for RF impairments.

- MCS 13 is testable using partial RB allocation of 32 under TDLD10-200 and considering a margin of 3 dB for RF impairments.

Observation 3: As a general conclusion, the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation are

- 120 KHz/100 MHz: MCS 17 with full RB allocation under TDLD30-200

- 480 KHz/400 MHz: MCS 13 with partial RB allocation of 32 under TDLD10-200

Observation 4: Referring to Table 3.1, when considering performance metric at 30% of the peak throughput, for 120 KHz/100 MHz and 480 KHz/400 MHz using MCS 13, it has been noted that

- 120 kHz/100 MHz: MCS 13 is testable under TDLA30-650 at 30% of peak throughput using full RB allocation and considering a margin of 3 dB for RF impairments.

# References

- R4-2217391, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

- R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson.

# Appendix	Simulation assumption

## A.1	Propagation environments

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |