3GPP TSG-RAN WG4 Meeting # 105			                                  R4-2218347

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.10.3.1

Source:	Intel Corporation

Title:	Discussion on measurements without gaps when UE reporting NFG

Document for:		Discussion

# Introduction

In the last RAN4 meeting, Rel18 NR measurement gap enhance WI was initially discussed. In this paper, the objective 2 below in [1] was discussed.

The following objectives are considered in this WI: - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc. - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

# Discussion

# Interruption

In the last RAN4 meeting, several open issues on whether the interruption requirements are allowed when UE reporting NeedofrGapInfoNR was captured in [2]

### Issue 1-1-1: Whether interruption is expected when UE reports ’no-gap’ in ‘NeedForGapsInfoNR' < Way forward >: - Option 1: - Yes - Option 2 - No - Option 3: - Introduce additional UE capability or the new indication of the existing UE capability (e.g. as part of needForGap) to differentiate whether interruption is expected

Firstly, in Rel17 NCSG was introduced for the measurement enhancements. There were some similar discussions on interruption requirements when UE can support NCSG. It was eventually agreed that the necessary requirements on UE can be dependent on the UE’s capability and NW configuration.

And the following IE was used to indicate whether UE’s capability to support NCSG.

NeedForNCSG-InfoNR information element[TS38.331 v17.1.0]

-- ASN1START

-- TAG-NeedForNCSG-InfoNR-START

NeedForNCSG-InfoNR-r17 ::=        SEQUENCE {

intraFreq-needForNCSG-r17         NeedForNCSG-IntraFreqList-r17,

interFreq-needForNCSG-r17         NeedForNCSG-BandListNR-r17

}

NeedForNCSG-IntraFreqList-r17 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17

NeedForNCSG-BandListNR-r17 ::=    SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17

NeedForNCSG-IntraFreq-r17  ::=    SEQUENCE {

servCellId-r17                    ServCellIndex,

gapIndicationIntra-r17            ENUMERATED {gap, ncsg, nogap-noncsg}

}

NeedForNCSG-NR-r17  ::=           SEQUENCE {

bandNR-r17                        FreqBandIndicatorNR,

gapIndication-r17                 ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NeedForNCSGInfoNR-STOP

-- ASN1STOP

NeedForNCSG-NR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that a NCSG is needed, and value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed.

Therefore, from RAN4 perspective, the interruption requirements when NW configures the different types of gaps can be implicitly applied up to these indication (e.g. “gap, ncsg, nogap-noncsg”). For an example, when UE can support “nogap-noncsg”, there is not any interruption allowed.

Observation 1: When UE indicates NeedForNCSG-NR the exact interruption requirements can be conducted.

Similarly, if UE need to support the “NeedForGapsInfoNR” for the measurements without gaps, the necessary interruption requirements for UE can be also defined. However, in the last meeting, companies have some different understanding on UE reporting with “no-gap”. One of ambiguity is whether UE reporting “no-gap” means neither NCSG or legacy measurement gap beyond Rel17 needed as given below.

NeedForGapsInfoNR information element[TS38.331 v17.1.0] -- ASN1START -- TAG-NeedForGapsInfoNR-START NeedForGapsInfoNR-r16 ::= SEQUENCE { intraFreq-needForGap-r16 NeedForGapsIntraFreqList-r16, interFreq-needForGap-r16 NeedForGapsBandListNR-r16 } NeedForGapsIntraFreqList-r16 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 NeedForGapsBandListNR-r16 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 NeedForGapsIntraFreq-r16 ::= SEQUENCE { servCellId-r16 ServCellIndex, gapIndicationIntra-r16 ENUMERATED {gap, no-gap} } NeedForGapsNR-r16 ::= SEQUENCE { bandNR-r16 FreqBandIndicatorNR, gapIndication-r16 ENUMERATED {gap, no-gap} } -- TAG-NeedForGapsInfoNR-STOP -- ASN1STOP

NeedForGapsNR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed.

Observation 2: When UE indicates “no-gap” in NeedForGapInfoNR , it is ambiguous that whether neither NCSGs nor legacy measurement gaps are be configured by NW.

In our views, if the indication “no-gap” in NeedForGapInfoNR means no either NCSG or legacy measurement gaps needed from UE perspective, the interruption can be NOT allowed. But in Rel17, the “no-gap” indication in “NeedForNCSG-NR” seems have variant interpretation in TS38.331.

Observation 3:  How to define the interruption requirements need RAN2 further clarification on the indication of “no-gap” in NeedForGapsNR message (e.g. whether it is consistent with that in “NeedForNCSG-NR”).

From RAN4 perspective, in order to more clearly define UE behaviors on the interruption requirements, the similar mechanism in Rel17 NCSG can be adopted also. For an instance, if the additional indication (e.g. “nogap-notinerruption”) reported in NeedForGapsInfoNR, UE behaviors can be defined explicitly:

| NW config UE capability | No MG |
| --- | --- |
| Gap | No requirements |
| no-gap | Measurements out of gap, interruption allowed and interruption requirements defined in TS38.133 9.1.9[3] |
| nogap-nointerruption | Measurements out of gap, No interruption allowed |

In other words, we support the Option 1 or 3 for issue 1-1-1 in [2].

Proposal 1: The interruption requirements when UE performing SSB measurements without gap by reporting ‘NeedForGapsInfoNR' can be defined with the additional indication (e.g. “nogap-notinerruption”).

# Measurement reporting delay requirements

In the last meeting, RAN4 has some discussion on the principles to define the necessary measurement reporting requirements for the different cases below[WF][Summary].

- Case 1: without gap and no interruption (e.g. ’nogap’ or ’nogap-nointerruption[TBD]’ indicated in NeedForGapInfoNR)

- Case 2: without gap but interruption allowed (e.g. ’nogap’ indicated in NeedForGapInfoNR)

Thus, the following options were agreed.

### Issue 1-2-2&Issue1-2-4 Requirement for intra-freq/inter-freq measurement without gap when interruption allowed (case 2) < Way forward >: - Option 1: - Take requirements NCSG requirements as a starting point - The other aspects can be FFS. e.g. - The time slot alignment among the measurement objects and interruption location - Option 2: - The deactivated SCell measurement requirement can be the start point. - The other aspects can be FFS, e.g. - The frequency layers in the band for which UE reports ‘no gap’ should be counted in CSSF outside gap - Option 3: - Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point

In principle, the measurement reporting delay requirements depends on multiple factors (e.g. CCSF and gap occasions). However, we believe the framework of NCSG requirements can be taken as our start point. Some impacts (e.g. CCSF) can be discussed separately. Thus we can support Option 1 for issue 1-2-2 &1-2-4 above.

Proposal 2: Take requirements NCSG requirements (9.3.10 in TS38.133[3]) as a starting point to define the measurement reporting delay requirements for the case 2.

| ### Issue 1-2-1: Requirement for intra-freq measurement without gap when no interruption (intra-f case 1) < Agreement >: - Reuse requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) for the reporting delay requirements for intra-frequency measurement without gap and no interruption allowed ### Issue 1-2-3: Requirement for inter-freq measurement without gap when no interruption (Inter-f case 1) - Take requirements in Section 9.3.9 of TS38.133 (inter-freq w/o gap) as a starting point - FFS on: - to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap - the sample number should be 8 - updated CSSFoutside_gap is needed. |  |
| --- | --- |

For issue 1-2-3 above, in our view there is no clear definitions on CSSFoutside_gap for the measurement without when UE reports ‘no-gap’ via ‘needForGapInfoNR’ in current spec [3]. Thus we propose to clarify the CSSFoutside_gap in 9.1.5.1 of TS38.133.

Proposal 3: Updates/Clarification on CSSFoutside_gap when defining the requirements for case 1 is needed.

# UE behaviours

In the last meeting, RAN4 has some discussion on UE behaviours when UE support both NeedForGap and NCSG as below.

### Issue 1-3-1: UE behaviours when UE supports both NeedForGap and NCSG capabilities < Way forward >: - FFS on: - Proposal 1: The gap status indication in NeedForGaps should have 1-to-1 mapping with the gap status in NCSG if UE supports both NeedForGaps and NCSG(or other[TBD] ) capabilities. - Proposal 2: Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2 - Other proposals are not precluded.

Generally, we also thought there was some overlap or ambiguity when UE support both NeedForGap and NCSG for the possible gap-less measurements. For an example, when there is a vacant RF chain UE can indicate either of the following options to perform the measurement outside of measurement gap.

- Alt 1. ‘no-gap’ via “needForGapsInfoNR”

- Alt 2. ‘ncsg or nogap-noncsg’ via “needForNCSG-InforNR’

Thus, whether UE need to distinguish them for the same purpose can be FFS. More importantly, for the possible enhancements, RAN4 shall be investigate the potential impacts on legacy UE.

Proposal 4: Whether UE need to distinguish NeedForGap and NCSG both of which can indicate the gap-less measurements can be FFS. And for the possible enhancements, RAN4 shall investigate the potential impacts on legacy UE.

# Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI are discussed. The proposals can be summarized as:

Observation 1: When UE indicates NeedForNCSG-NR the exact interruption requirements can be conducted.

Observation 2: When UE indicates “no-gap” in NeedForGapInfoNR , it is ambiguous that whether neither NCSGs nor legacy measurement gaps are be configured by NW.

Observation 3:  How to define the interruption requirements need RAN2 further clarification on the indication of “no-gap” in NeedForGapsNR message (e.g. whether it is consistent with that in “NeedForNCSG-NR”).

Proposal 1: The interruption requirements when UE performing SSB measurements without gap by reporting ‘NeedForGapsInfoNR' can be defined with the additional indication (e.g. “nogap-notinerruption”).

Proposal 2: Take requirements NCSG requirements (9.3.10 in TS38.133[3]) as a starting point to define the measurement reporting delay requirements for the case 2.

Proposal 3: Updates/Clarification on CSSFoutside_gap when defining the requirements for case 1 is needed.

Proposal 4: Whether UE need to distinguish NeedForGap and NCSG both of which can indicate the gap-less measurements can be FFS. And for the possible enhancements, RAN4 shall investigate the potential impacts on legacy UE.

# References

- RP-222337, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps

- R4-2217252, WF on Measurement without gaps for UEs reporting NeedForGapsInfoNR

- TS38.133 v17.7.0

- TS38.331 v17.1.0