3GPP TSG-RAN WG4 Meeting #105	R4-2218405

Toulouse, France, November 14 - November 18, 2022

Source:	China Telecom

Title:	DL interruption for Tx switching across 3/4 bands

Agenda Item:	8.20.3.1

Document for:	Discussion

- Introduction

The RRM WF on R18 multi-carrier enhancement was approved in [1]. This contribution discusses the open issues on DL interruption for Tx switching across 3/4 bands.

- Discussion

The follow agreements on DL interruption length were captured in the WF:

Issue 1-2: DL interruption length

<Agreement>:

For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption.

< Way forward >:

For Rel-18 Tx switching across 3/4 bands with 2 TAGs,

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption.

- Option 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as:

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Where

- Switching period is being discussed in RF session and the outcome can be reused in interruption design.

- TA adjustment uncertainty remains same as legacy.

- MRTD=3us which is used to derive R16/R17 DL interruption length is not valid for non-collocated case, further discuss the concrete value of MTRD

For the 2-TAG case with non-collocated scenario, we suggest to consider a large distance for inter-band BSs that is possible for the typical deployment scenarios, e.g., distance of 1000m.

- Firstly we assume a large distance between the two BSs of the two bands, i.e., 1000 m.

- Then assume an extreme situation that UE distances to the two BSs are 0m and 1000m respectively.

- With these, the propagation delay difference for the UE to the two BSs is 1000m/(3*10^8m/s)=3.33us.

It is seen that the maximum propagation delay difference is only 3.33us for a large inter-site distance of 1000m.

Observation 1: The maximum propagation delay difference is only 3.33us for a large inter-site distance of 1000m.

Then, looking at the formula for deriving the DL interruption length used in Rel-16/17, it is seen that very sufficient margin has been considered , and the additional propagation delay difference of 3.33us can be accommodated in the Rel-16/17 values for length of DL interruption. So, we prefer option 1 to reuse Rel-16/Rel-17 values for length of DL interruption.

Observation 2: Very sufficient margin has been considered for deriving the DL interruption length in Rel-16/17.

Proposal 1: For Rel-18 Tx switching across 3/4 bands with 2 TAGs, reuse Rel-16/Rel-17 values for length of DL interruption.

- Conclusion

In this contribution, we had the following observations and proposal on DL interruption for Tx switching across 3/4 bands with 2-TAG case:

Observation 1: The maximum propagation delay difference is only 3.33us for a large inter-site distance of 1000m.

Observation 2: Very sufficient margin has been considered for deriving the DL interruption length in Rel-16/17.

Proposal 1: For Rel-18 Tx switching across 3/4 bands with 2 TAGs, reuse Rel-16/Rel-17 values for length of DL interruption.

- References

R4-2217258, WF on R18 multi-carrier enhancements (RRM), Huawei, HiSilicon, RAN4 #104e-bis, Oct 2022.