3GPP TSG-RAN4 Meeting #105	                                                             R4-2218438

Toulouse, France, November 14 – November 18, 2022

Title:	Discussion on RRM requirements for Inter-RAT measurement without gap

Source:	CATT

Agenda Item:	8.10.3.2

Document for:	Discussion

1 Introduction

In the previous RAN4 meeting, RRM requirements for Inter-RAT measurement without gap were discussed and the conclusions are captured in the approved WF [1]. This contribution further discusses the topic and presents our views.

The following case differentiation in last meeting is used in this paper for discussion:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

2 Discussion

2.1 Inter-RAT NR measurements

It was discussed in last meeting whether to consider MR-DC case for case a-2 and case b-2 but no conclusions. For defining the requirements for measurement without gap, we think all the four cases should be considered. And both a-2 and b-2 can consider MR-DC as the requirements are the similar as that for SA.

Proposal 1: Consider MR-DC for case a-2 and b-2.

Proposal 2: All the four cases (case a-1, a-2, b-1 and b-2) can be considered to define the inter-RAT measurement requirements without gap.

For the UE capability for case a-1, it has been agreed to define the requirements based on UE reporting on interRAT-NeedForGapsNR-r16. We understand there is no need to further consider other capability e.g. NCSG reporting which seems not existed in LTE.

For the capability for case a-2, we understand the new UE capability to indicate whether UE support inter-RAT measurement without gap may be needed which is same as that for inter-frequency measurement without gap.

Proposal 3: There is no need to extend the other capability to inter-RAT NR measurement for case a-1.

For case b-2, we think the capability of the existing signalling for gap information report (e.g. NeedForGap, NeedForNCSG etc.) is not needed and a new capability indicating whether UE support measurement without gap may be needed.

For the measurement requirements of inter-RAT NR measurement, the inter-frequency measurement requirements can be reused since the measurement requirements is based on the target resources and less related to the working carrier and operation.

Proposal 4: The inter-frequency measurement requirements can be reused for inter-RAT NR measurement.

2.2 Inter-RAT LTE measurements

For case b-1 in inter-RAT LTE measurement, similar as the discussion in section 2.1, since the requirements can be defined based on NeedForNCSG-InfoEUTRAN, there is no need to extend other capability to reach the same effect.

Proposal 5: There is no need to extend the capability of NeedForGaps to inter-RAT LTE measurement.

For case b-2, we think the capability of the existing signalling for gap information report (e.g. NeedForGap, NeedForNCSG etc.) is not needed and a new capability indicating whether UE support measurement without gap may be needed.

Proposal 6: For case b-2, a new capability indicating whether UE support measurement without gap may be needed.

3 Summary

This contribution discusses RRM requirements for Inter-RAT measurement without gap. The following proposals are made.

Inter-RAT NR measurements

Proposal 1: Consider MR-DC for case a-2 and b-2.

Proposal 2: All the four cases (case a-1, a-2, b-1 and b-2) can be considered to define the inter-RAT measurement requirements without gap.

Proposal 3: There is no need to extend the other capability to inter-RAT NR measurement for case a-1.

Proposal 4: The inter-frequency measurement requirements can be reused for inter-RAT NR measurement.

Inter-RAT LTE measurements

Proposal 5: There is no need to extend the capability of NeedForGaps to inter-RAT LTE measurement.

Proposal 6: For case b-2, a new capability indicating whether UE support measurement without gap may be needed.

4 References

- R4-2217253 WF on inter-RAT measurement without gap, Intel, RAN4#104bis-e

- R4-2215428, Discussion on RRM requirements for Inter-RAT measurement without gap, CATT, RAN4#104bis-e