3GPP TSG-RAN WG4 Meeting #105                                                                                  R4-2218465

Toulouse, FR, November 14 – November 18, 2022

Source: 	CATT

Title: 	Discussion on remaining issues about simulation assumptions for above 10GHz NTN co-existence study

Agenda item:	8.23.2

Document for:	Approval

- Introduction

In RAN4#104-bis-e meeting, A WF [1] on simulation assumptions for above 10GHz NTN co-existence study was agreed. Some remaining issues in WF need further discussion in next meeting.

This contribution provides our views on remaining issues about simulation assumptions for above 10GHz NTN co-existence study.

- Discussion

2.1 Coexistence scenario

NTN DL simulation

As per the WF [1], the remaining issue concerning NTN DL simulation is shown as below:

Issue 1-1: NTN DL simulation - Option1: Consider NTN UL cases first and further discuss NTN DL cases

Generally, NTN SAN ACLR is dependent on coexistence scenario of NTN DL as aggressor, and NTN UE ACS is dependent on coexistence scenario of NTN DL as victim. For NTN DL case, currently there is no 3GPP defined TN bands adjacent to Ka band DL of NTN, so there is not TN with NTN co-existence scenario for Ka band DL. From previous experience, in this case, the NTN with NTN for Ka band DL needs to be considered. For Ka band DL (17.7-20.2 GHz), there are 2.5GHz RF bandwidth, which is not like small RF bandwidth of FR1 NTN. May be 2.5GHz RF bandwidth can be allocated to more than one satellite operator in future. If more than one satellite operator is located in Ka band DL, so the NTN with NTN coexistence for Ka band DL can be considered as coexistence case for study.

Proposal 1: RAN4 may consider NTN with NTN coexistence for Ka band DL.

Aggressor and victim table

As per the WF [1], the remaining issue concerning aggressor and victim table is shown as below:

Issue 1-9: Aggressor and victim table Further discuss cases in the table below. Noting that: - R4-2215352 (Thales et al): case 1, 3, 4, applicable for n258 - R4-2216557 (ZTE): case 1,2,3,4 applicable for n257, n258 and n261 - There’s also a proposal to remove the Notes. No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL; Applicable for n257, n258 and n261; Phase 1. 2; TN with NTN; TN UL; NTN UL; Applicable for n257, n258 and n261; Phase 1. 3; TN with NTN; NTN UL; TN DL; Applicable for n257, n258 and n261; Phase 1. 4; TN with NTN; TN DL; NTN UL; Applicable for n257, n258 and n261; Phase 1. 5; TN with NTN; TN DL; NTN DL; FFS; Phase 2. 6; TN with NTN; NTN DL; TN DL; FFS; Phase 2. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. 9; NTN with NTN; NTN DL; NTN DL; FFS. NTN UL; NTN UL; FFS. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

For No.2 case in the table, the aggressor is TN UL and victim is NTN UL, it means that the NTN SAN receive the interference signal from TN UE when it receives satellite signals from NTN UE. Referring to FR1 NTN co-existence study, this coexistence case is also considered. So for Ka band coexistence, this case (No.2 case) can be considered. Because NTN Ka band UL (27.5 – 30GHz) is adjacent to NR band n258 (24.250 - 27.5 GHz, TDD). The bands (NTN Ka band UL, and NR band n258) need to be protected and co-existence analysis is required to determine the Adjacent Channel Interference Ratio (ACIR). So, we think case 1,2,3,4 are applicable for NTN operating in e.g.  Ka band UL, for e.g. coexistence with TN operating in n258 TDD.

Observation 1: Case 1, 2, 3, and 4 are applicable for NTN operating in e.g. Ka band UL, for e.g. coexistence with TN operating in n258 TDD.

Proposal 2: Case 1,2,3,4 and 9 can be considered for coexistence study in phase 1.

2.2 System parameters

Number of active UE (UL)

As per the WF [1], the remaining issue concerning number of active UE (UL) is shown as below:

Issue 3-4: Number of active UE (UL) - Option 1: 9 UEs and nRBs per UE for GEO and LEO - Option 2: FFS

Figure 2-1: Scheduled PRB position for UEs per satellite beam

The option 1 is similar to the number of active UE (UL) for FR1 NTN coexistence, where UEs are equally split inside the channel bandwidth into 3 ACIR regions. From Figure 2-1, Scheduled PRB position for UEs per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. So option 1 is reasonable.

Proposal 3: Adopt Option 1(9 UEs and nRBs per UE for GEO and LEO).

NTN Fixed VSAT UE

As per the WF [1], the remaining issue concerning NTN fixed VAST UE is shown as below:

Issue 3-5: NTN Fixed VSAT UE Further discuss Option 1 & Option 2 Characteristics; Option 1 (Ericsson, Samsung, ZTE); Option 2 (Thales). Antenna type and configuration; Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter; Directional (see [2]) with 60 cm equivalent aperture diameter (R4-2215348). Polarisation; Circular; Circular. Efficiency; UL 60%, DL 57%. Rx Antenna gain; 39.7 dBi; 39 dBi. Noise figure; 1.2 dB; 1.2 dB. Rx Feeder loss; -0.5 dB. Antenna temperature; 150 K; 40K. Sky temperature; 30 K. Ground temperature; 10. G/T figure of merit; 16.5 (dB/K). Rx Feeder loss; -0.5 dB. Tx transmit power; 2 W (33 dBm); 2W (3dBW). Tx antenna gain; 43.2 dBi; 42.9 dBi. Output loss; -1.0. EIRP; 44.9 dBm. UE height; FFS; FFS. NOTE: VSAT terminal characteristics could be implemented with phased array antenna.

Option 1 and option2 are same which have same antenna type and configuration, they are both Directional (see [2]) with 60 cm equivalent aperture diameter. So the difference in Rx antenna gain and Tx antenna gain between the two options is small which is caused by the antenna aperture efficiency and the calculated frequency. The Rx antenna gain and Tx antenna gain in both options are reasonable. The biggest difference between two options is Antenna temperature. The option 2(40K) is from ITU-R Rec. P372 and Rec. P618 in Thales’s contribution, and the option 1(150K) is from TR 38.811. Referring to TR 38.811, Antenna temperature is  typically 290 K with 0 dBi gain and 150 K with >30 dBi gain, here, Rx antenna gain is 39.7dBi, so 150 K antenna temperature in option 1 is OK. And the G/T equation for option 1 is from Table 4.4-1 of TR 38.811.  We slightly prefer the antenna temperature in option1 since TR 38.811 is widely accepted by RAN4.

Comparing the difference between two options, the detailed parameters for option 1 are updated in the following table 2-1.

Table 2-1: UE characteristics for NTN Fixed VSAT UE

| Characteristics | Option 1 (Ericsson, Samsung, ZTE) | Option 2 (Thales) |
| --- | --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (see [2]) with 60 cm equivalent aperture diameter (R4-2215348) |
| Polarisation | Circular | Circular |
| Efficiency | Antenna aperture efficiency (a dimensionless parameter with typical values for parabolic antennas from 0.55 to 0.70) from TR 38.821. | UL 60%, DL 57% |
| Rx Antenna gain | 39.7 dBi | 39 dBi |
| Noise figure | 1.2 dB | 1.2 dB |
| Rx Feeder loss | -0.5dB | -0.5 dB |
| Antenna temperature | 150 K | 40K |
| Sky temperature | N/A | 30 K |
| Ground temperature | N/A | 10 K |
| G/T figure of merit | 15.35 (dB/K) (Note 1) | 16.5 (dB/K) |
| Tx transmit power | 2 W (33 dBm, or 3dBW) | 2W (3dBW) |
| Tx antenna gain | 43.2 dBi | 42.9 dBi |
| Output loss | -1dB | -1.0 |
| EIRP[dBW] | 45.2 | 44.9 |
| UE height | FFS | FFS |
|  | NOTE: VSAT terminal characteristics could be implemented with phased array antenna |  |
| NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |  |  |

From above Table 2-1, the G/T for option 1 and option 2 are 15.35 and 16.5dB/K respectively, and EIRP for option 1 and option 2 are 45.2 and 44.9 dBW respectively. The difference in G/T and EIRP between the two options is 1.15dB, from perspective of coexistence simulation, this difference is very small. Both options are acceptable. And we prefer option 1.

Proposal 4: To adopt Option 1(from TR 38.821), and update the parameters in following table.

| Characteristics | Option 1 (Ericsson, Samsung, ZTE) |
| --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 39.7 dBi |
| Noise figure | 1.2 dB |
| Rx Feeder loss | -0.5dB |
| Antenna temperature | 150 K |
| Sky temperature | N/A |
| Ground temperature | N/A |
| G/T figure of merit | 15.35 (dB/K) (Note 1) |
| Tx transmit power | 2 W (33 dBm, or 3dBW) |
| Tx antenna gain | 43.2 dBi |
| Output loss | -1dB |
| EIRP[dBW] | 45.2 |
| UE height | FFS |
|  | NOTE: VSAT terminal characteristics could be implemented with phased array antenna |
| NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |  |

NTN movable VSAT UE

As per the WF [1], the remaining issue concerning NTN movable VAST UE is shown as below:

Issue 3-6: NTN movable VSAT UE - Option 1: Characteristics; ESIM (TR 38.821); ESIM (ITU). Antenna type and configuration; Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg); Antenna pattern: S.580-6. Polarisation; Linear: +/-45°X-pol. Rx Antenna gain; TBD dBi per element; 45 dBi. Antenna temperature; TBD K. Noise figure; TBD dB. Tx transmit power; [TBD W (TBD dBm)]; Transmit power density（dBW/Hz）-46.3. Tx antenna gain; TBD dBi per element; 45 dBi. - Option 2: With respect to VSAT UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-5: - FFS: UE height

The option 1 is from TR 38.821, and the option 2 is from ITU-R S.465-5 in Thales’ contribution. It seems that option 2 is more straightforward. But option 1 is more familiar for RAN4. So we prefer to option 1.  For option 1, referring to 45dBi for ESIM (ITU), we can design the  UE characteristics for NTN movable VSAT UE as figure 2-2.

Figure 2-2, UE characteristics for NTN movable VSAT UE

| Characteristics | ESIM (TR 38.821) |
| --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (128,128,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=90 deg) |
| Polarisation | Linear: +/-45°X-pol |
| Element gain including loss | 5.5dBi |
| Rx Antenna gain (per polarization) | 45.3dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain (per polarization) | 45.3dBi |

Proposal 5: Adopt Option 1, and update parameters in following table.

| Characteristics | ESIM (TR 38.821) |
| --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (128,128,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=90 deg) |
| Polarisation | Linear: +/-45°X-pol |
| Element gain including loss | 5.5dBi |
| Rx Antenna gain (per polarization) | 45.3dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain (per polarization) | 45.3dBi |

Horizontal/vertical 3 dB beam width of single element (degree)

As per the WF [1], the remaining issue concerning Horizontal/vertical 3 dB beam width of single element (degree) is shown as below:

Issue 3-11-1 Horizontal 3dB /vertical 3dB 3 dB beam width of single element (degree) - Option1: 65 - Option2: 90

Generally, the Horizontal/vertical 3dB beamwidth is dependent on antenna element spacing, referring to TR 38.921[6], it is reasonable to set the horizontal and vertical element spacing for Macro Urban as 0.5 wavelengths. For 0.5 wavelengths Horizontal/vertical element spacing, the 90 degree 3dB beam width is reasonable.

Proposal 6: Adopt option 2(90degre) for 0.5 wavelengths Horizontal/Vertical element spacing.

Maximum directional gain of an antenna element GE,max

As per the WF [1], the remaining issue concerning Maximum directional gain of an antenna element GE,max is shown as below:

Issue 3-11-2 Maximum directional gain of an antenna element GE,max - Option1: 5.5 dBi - Option2: 8 dBi

The antenna element gain is dependent on Horizontal/vertical 3dB beamwidth, and it should include loss. For 90 degree horizontal/vertical 3dB beamwidth, it is reasonable to set the antenna element GE,max including loss as 5.5dBi.

Proposal 7: Adopt Option 1(5.5dBi) for 90 degree horizontal/vertical 3dB beamwidth.

- Conclusion

This contribution provides our general consideration on remaining issues about simulation assumptions for above 10GHz NTN co-existence study. The following observations and proposals are concluded as follows:

Proposal 1: RAN4 may consider NTN with NTN coexistence for Ka band DL.

Observation 1: Case 1, 2, 3, and 4 are applicable for NTN operating in e.g. Ka band UL, for e.g. coexistence with TN operating in n258 TDD.

Proposal 2: Case 1,2,3,4 and 9 can be considered for coexistence study in phase 1.

Proposal 3: Adopt Option 1(9 UEs and nRBs per UE for GEO and LEO).

Proposal 4: To adopt Option 1(from TR 38.821), and update parameters in following table.

| Characteristics | Option 1 (Ericsson, Samsung, ZTE) |
| --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 39.7 dBi |
| Noise figure | 1.2 dB |
| Rx Feeder loss | -0.5dB |
| Antenna temperature | 150 K |
| Sky temperature | N/A |
| Ground temperature | N/A |
| G/T figure of merit | 15.35 (dB/K) (Note 1) |
| Tx transmit power | 2 W (33 dBm, or 3dBW) |
| Tx antenna gain | 43.2 dBi |
| Output loss | -1dB |
| EIRP[dBW] | 45.2 |
| UE height | FFS |
|  | NOTE: VSAT terminal characteristics could be implemented with phased array antenna |
| NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |  |

Proposal 5: Adopt Option 1, and update parameters in following table.

| Characteristics | ESIM (TR 38.821) |
| --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (128,128,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=90 deg) |
| Polarisation | Linear: +/-45°X-pol |
| Element gain including loss | 5.5dBi |
| Rx Antenna gain (per polarization) | 45.3dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain (per polarization) | 45.3dBi |

Proposal 6: Adopt option 2(90degre) for 0.5 wavelengths Horizontal/Vertical element spacing.

Proposal 7: Adopt Option 1(5.5dBi) for 90 degree horizontal/vertical 3dB beamwidth.

- References

- R4-2217468, WF on [313] NR_NTN_enh_Part2, Samsung, RAN4#104-bis-e

- R4-2217469, Simulation assumptions for NTN co-existence study in bands above 10GHz, Samsung, RAN4#104-bis-e

- TR38.863

- TR 38.821

- TR 38.811

- TR38.921