3GPP TSG-RAN WG4 Meeting #105	R4-2218574

Toulouse, France, 14th Nov. – 18th Nov., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2698 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on cell reselection requirements with relaxed measurement criterion for satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The minimum requirements at transitions for NTN cell reselection was defined, and the requirements in clause 4.2.2.8 is applied. However, in GSO scenario, NTN UE measure the target cell either with the legacy requirement or with relaxed requirement. And NTN UE is configured with the relaxed measurement configuration via relaxedMeasurement without criteria, and no transition is expected when the relaxed measurement is configured. - In GSO scenario, the relaxed cell reselection requirements for NTN UE was introduced, and the requirements in clause 4.2.2.9 and 4.2.2.10 are applied for intra-frequency and inter-frequency respectively. However, according to RAN2 LS (R2-2210866), RAN2 understands the relaxed measurements only apply to GSO neighbour cells and can only be configured by GSO serving cell. RAN4 need to reflect this understanding in sepc. In addition, different requirement for different scenario are defined in clause 4.2.2.9 and 4.2.2.10, which requirement is applied to NTN UE need to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the minimum requirement at transitions for NTN UEs. - Clarify that the requirements in clause 4.2C.2.7 and 4.2C.2.8 only apply to GSO neighbour cells and can only be configured by GSO serving cell. - Clarify the requirement in clause 4.2.2.9.2 and 4.2.2.10.2 is applied to the relaxed requirement for NTN UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The transition scenario for NTN cell reselection is not needed; - The scenarios for the relaxed requirements for NTN UE is unclear; - Which requirement in clause 4.2.2.9 and 4.2.2.10 is applied to the relaxed requirement for NTN UE is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 4.2C.2.6, 4.2C.2.7 and 4.2C.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2C.2.6	Void

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

The requriements in clause 4.2.2.9.2 apply provided that:

- Serving cell’s satellite is GSO;

- Target neighbour cell’s satellite is GSO;

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

The requriements in clause 4.2.2.10.2 apply provided that:

- Serving cell’s satellite is GSO;

- Target neighbour cell’s satellite is GSO;