3GPP TSG-RAN WG4 Meeting # 105.                                   R4-2218695

Toulouse, France, 14th – 18th November 2022

Source: 	Apple

Title: 	TP for TR 38.876: frequency error

Agenda Item:	8.13.2.1

Document for:	Discussion

# Introduction

This contribution presents a text proposal on frequency error for ATG UE for TR 38.876.

# Reference

[1] R4-2218693, Discussion on ATG general aspects, Apple

[2] R4-2218694, ATG UE Tx RF requirements, Apple

# Text proposal

----------------------------------------------------------Start of Text Proposal------------------------------------------------------------

## 7.1 ATG CPE specific

### 7.1.1 ATG CPE power class and requirement type

### 7.1.2 Tx requirements

7.1.2.x Frequency error

The doppler frequency for ATG UE can be determined by,

fD =  cosθ*vc*f_c

Where,

- f_D is Doppler frequency, f_c is the carrier frequency, which is set as 5GHz for n79.

- v is the speed of the aircraft which is 1200km/h.

- θ is the elevation angle between UE and gNB.

Since the agreed maximum cell radius is ~100km (corresponding to 200km ISD) [x] and the normal commercial airplane altitude is 10km, the angle is only about 6 degrees. When ATG UE is performing initial access at cell edge, the DL doppler frequency is approaching 5.5kHz and the UL doppler frequency is approaching 11kHz which may cause not negligible impact for link performance and access successful rate. ATG UE may need to consider frequency compensation during initial access.

The ATG BS location information need to be broadcasted for UE to do frequency compensation. It can be broadcasted by the IE EphemerisInfo in NTN SIB 19 in 38.331. Where, Ephemeris may be expressed either in format of position and velocity state vector or orbital parameters. When it comes to ATG, velocityVX, velocityVY and velocityVZ can be set as zero.

7.1.2.y	xxxxxx

### 7.1.3 Rx requirements

---------------------------------------------------------End of Text Proposal--------------------------------------------------------------