3GPP TSG-RAN WG4 Meeting # 105			R4-2218753

# Toulouse, France, November 14 – November 18, 2022

Source:	Sony

Title:	Views on UL 256-QAM for FR2-1

Agenda item:	8.7.2

Document for:	Discussion

- Background

At the 3GPP RAN #97-e meeting, the Work Item on “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3” was revised for Rel-18 [1]. The objectives of the core part of the Work Item include the following:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

RAN4 #104 bis-e discussed UL 256-QAM  [2] and approved a way forward [3]:

### Sub-topic #2-1: EVM evaluation by link level simulation

- Issue 2-1-1: EVM requirement for 29GHz

Agreement:

- 3.5% EVM for 29GHz, FFS for operating SNR.

  - The target power class is PC1, PC2, and PC5.

- Issue 2-1-2: EVM requirement for 39GHz

Agreement:

- 3.5% EVM for 39GHz and using average value FFS for operating SNR with limited MCS.

  - The limited MCS is the subset of MCS with 256QAM

  - FFS on the list of MCS

  - Decide the operating SNR based on list of MCS

  - The target power class is PC1, PC2, and PC5.

Simulation assumptions for link-level simulations were already agreed upon in RAN4 #104-e [4] and are shown below in Table 1.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS: 2 (every 2 RBs), LPTRS: 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS: 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modeled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2. |

Table 1. Simulation assumptions according to [3].

Regarding the power classes, it was agreed to keep the priorities specified in the WI.

This tdoc is an update of [5] where we present further link-level simulations. We discuss the performance gain of UL 256QAM under the impact of phase noise and its feasibility in FR2-1.

- Simulation

We performed simulations and compared the throughput between 64QAM and 256QAM for an EVM value of 3.5%, as agreed in WF [3]. TX EVM has been the same as RX EVM in each simulation. The simulations follow the agreed simulation assumptions [4]. Rank-1, and AWGN for 29 GHz and 39GHz are simulated. 50MHz channel BW and no HARQ have been used. Phase noise is added according to option 2 in the simulation assumptions, i.e., “Example2,” at both the UE and BS.

## 29 GHz

We performed simulations to determine whether 256-QAM works for FR2-1 and demonstrate its performance gain. Figure 1 shows the results.

Figure 1 Throughput comparing 64QAM and 256QAM with AWGN for 29GHz, EVM 3.5%.

Figure 1 shows that throughput for 64QAM (red curves) has already saturated at an SNR level of 16dB. For 256QAM, SNR levels of 20 dB for MCS21 and 25 dB for MCS23 (EVM 3.5%) are required to exceed the throughput performance of 64QAM. To reach throughput saturation for 256QAM, SNR levels of 21dB for MCS21 and 28 dB for MCS23 are needed.

Observation 1	For 256QAM, to reach saturation in throughput performance, SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz.

## 39 GHz

Simulations have also been performed for 39GHz with the same setup as for 29GHz. Figure 2 shows the results.

Figure 2	 Throughput comparing 64QAM and 256QAM with AWGN for 39GHz, EVM 3.5%.

Figure 2 shows 256QAM performance for 39GHz compared to 64QAM. SNR levels of 21.5dB are needed to exceed the throughput performance of 64QAM for MCS21. For MCS23, an SNR of 36dB is required to exceed the throughput performance of 64QAM. An SNR level of 24dB is needed to reach throughput saturation for 256QAM for MCS21. For MCS23, we didn’t see saturation within the simulated SNR range.

Observation 2	For 256QAM, to reach saturation in throughput performance, SNR levels of 24dB for MCS21 are needed for 39GHz. To exceed the performance of 64QAM, an SNR of 21.5dBm is required.

For good performance (i.e., improvements over 64QAM), the EVM and phase noise must be kept low. Therefore, 256QAM is better suited for PC1, PC2, and PC5, which apply to devices with higher-performing RF components. Those devices usually operate on a power supply rather than a battery. In particular, the power amplifier (PA) is a significant contributor to the phase noise, and a power supply-operated PA could be biased in a way less contributing to the phase noise. We should also note that a relatively high SNR is needed for 256QAM to be superior to 64QAM. This could be achieved for  PC1, PC2, and PC5, where higher EIRP is assumed. It is also worth mentioning that phase noise (PN) significantly influences the result. The PN used in the simulation is based on the PN model in the TR and depends on semiconductor technology. For other semiconductor technologies, better performance can be expected.

- Conclusion

In this contribution, we have derived the spectral efficiency performance of 256QAM for FR2-1. It is seen that there are clear benefits of introducing 256QAM for FR2-1 in the high SNR range, and 256QAM is, therefore, well suited for PC1, PC2, and PC5, where higher EIRP is assumed. The following observations have been made:

Observation 1	For 256QAM, to reach saturation in throughput performance, SNR levels of 21dB for MCS21 and 28dB for MCS23 are needed for 29GHz.

Observation 2	For 256QAM, to reach saturation in throughput performance, SNR levels of 24dB for MCS21 are needed for 39GHz. To exceed the performance of 64QAM, an SNR of 21.5dBm is required.

- References

RP-222228,		“Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Moderator (Nokia, Xiaomi)

R4-2217783		“Email discussion summary for [104-bis-e][131] FR2_enh_req_Ph3_part2”, Moderator ( Xiaomi)

R4-2217729		“WF on UL 256 QAM”, Xiaomi

R4-2214453		“WF on UL 256QAM”, Xiaomi

R4-2216251		“Views on UL 256-QAM for FR2-1”, Sony