3GPP TSG-RAN WG4 Meeting # 105												R4-2218869

Toulouse, France, November 14 – November 18, 2022

Source: 	vivo

Title: 	Further evaluation for FR2-1 UL 256QAM based on SLS

Agenda Item:	8.7.2

Document for:	Approval

- Introduction

In RAN4#104e-bis, companies have submitted many link level simulation results for FR2-1 UL 256QAM evaluation and we agreed that the EVM for FR2-1 is 3.5% which is the same as FR1. However, the operating SNR is still pendent, and some companies try to lower the operating SNR by limiting the MCS. In this contribution, we provide more evaluation based on system level simulation.

- Discussion

  - Simulation assumption

The following simulation assumption is used in our evaluation:

Table I system level simulation assumption

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | Uma LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH – Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | First priority: PC1/PC2/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi Second priority: PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| Target SNR at BS side |  | FFS |  |
| UE max output power |  | PC1: 35 dBm/PC2: 23dBm/PC3: 23 dBm/PC5: 23 dBm |  |
| Power control |  | PC1: P0 = -60 / PC2&PC5: P0 = -70 / PC3: -76 alpha: Urban macro =0.8 / Indoor = 0.6 |  |
| Traffic model |  | Full Buffer |  |

Compared to the assumption in [1], the only difference is the addition of power control and traffic model. The P0 is the target received power at the BS side which is modified for different types of UE, and the alpha is the factor of path loss compensation which is different for Indoor and Uma to avoid large interference between cells.

  - SINR distribution

The SINR distribution for Indoor and Uma are shown in Figure 1:

Figure 1 SINR distribution

Except for PC1, The SINR of UE is hard to larger than 25 dB at the BS side.

Observation 1:

- For PC1, 20% of UE’s SINR can be larger than 25 dB.

- For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB.

We further counted the percentage of UEs that had used 256QAM in the simulation and the throughput performance gain. The throughput gain is derived from comparing whether the 256QAM is allowed. The results are shown in below:

Figure 2 proportion and performance gain for 256 QAM

For the indoor scenario, PC1 UE can have obvious performance gain, and about 70% UE is once scheduled with 256QAM during the simulation time, but for other power classes, the throughput performance even become worse, this is because the SINR for these UE cannot support such high MCS and the BLER is bad. For the Uma scenario, even though there are many UEs scheduled with 256 QAM, the performance gain is tiny.

Observation 2:

- For the indoor scenario, only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM.

- For the Uma scenario, the performance gain of all PC UEs is tiny.

Based on the simulation results above, we think only PC1 UE has the chance to achieve performance gain and is worth to be further studied.

Proposal 1: Conclude the UL 256QAM for PC2/PC3/PC5 is not feasible in FR2-1, and focus on PC1 only in the future discussion.

  - Limited MCS

In [1], the limited MCS was raised and the intention is to lower the operating SNR so that UE can archive the performance gain. However, we think the limited MCS will also restrict the performance gain of UL 256QAM which makes the 256QAM meaningless. To prove this, we further do the system level evaluation and the results are shown in Table II:

Table II throughput gain for UL 256 QAM with limited MCS

| Limited MCS | No limitation | Max MCS = 22 | Max MCS = 20 |
| --- | --- | --- | --- |
| Indoor, PC1 | 9.4% | 4.2% | 2.3% |

Observation 3: Even though limiting the MCS can lower the operating SNR, the system performance gain will also be restricted which makes the UL 256QAM meaningless.

Proposal 2: No need to limit the max MCS for UL 256QAM, it depends on NW whether the higher MCS can be used.

- Conclusion

In this contribution, we provide further evaluations based on SLS, and the observations and proposals are listed below:

Observation 1:

- For PC1, 20% of UE’s SINR can be larger than 25 dB.

- For PC2/PC3/PC5, <5% UE’s SINR can larger than 25 dB.

Observation 2:

- For the indoor scenario, only PC1 shows obvious throughput gain, and about 69.3%UEs are scheduled with 256 QAM.

- For the Uma scenario, the performance gain of all PC UEs is tiny.

Observation 3: Even though limiting the MCS can lower the operating SNR, the system performance gain will also be restricted which makes the UL 256QAM meaningless.

Proposal 1: Conclude the UL 256QAM for PC2/PC3/PC5 is not feasible in FR2-1, and focus on PC1 only in the future discussion.

Proposal 2: No need to limit the max MCS for UL 256QAM, it depends on NW whether the higher MCS can be used.

- References

- R4-2217729,	WF on UL 256 QAM, Xiaomi, RAN4#104e-bis