3GPP TSG-RAN WG4 Meeting # 105 					 				    		                 R4- 2218942

Toulouse, France, November 14 – November 18, 2022

Agenda item:			8.18.2.1

Source:	Samsung,

Title:	Simulation assumptions for SBFD adjacent channel co-existence study

Document for:	Approval

# Introduction

In previous RAN4#104-e and 104bis-e meetings, the detail items of simulation assumptions were discussed and agreed. The agreements and way forwards (WF) were approved in R4-2214378, R4-2214379 and R4-2217466[1-3]. These were great achievements for this study item to further progress. However, the detailed agreements, especially those affecting the co-ex study, were separately discussed and recorded in different t-docs.

To better describe and organize the discussion, we would like to propose this document as a draft skeleton to capture all co-ex study required assumptions along with the associated agreements and way forwards. This document is proposed as a baseline and living document, so that it will also capture the future agreements and way forwards in future RAN4 meetings.

Proposal 1: This document is proposed as a baseline and living document, so that it will capture the existing and future agreements and way forwards for RAN4 adjacent channel co-ex study.

# Discussion

## Scenarios

The frequency range, deployment scenarios of sub-band full-duplex and legacy TDD, and its associated priority for co-ex study is described in the Table 2.1-1 below.

[Editor’s Note: The Table 2.1-1 refers to the R4-2214378 and R4-2214379.]

Table 2.1-1: Scenarios for SBFD co-ex study

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Indoor -> Indoor | Low |
| FR2 (30GHz) | 3 | Urban Macro -> Urban Macro | High |
|  | 4 | Urban Micro -> Urban Micro | Low |
|  | 5 | Indoor -> Indoor | Low |
| Note: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. |  |  |  |

The aggressor and victim combination is listed in Table 2.1-2 below.

[Editor’s Note: Table 2.1-2 refers to R4-2217466. The color of figure would be updated for clarification purpose later.]

Table 2.1-2: Victim, aggressor and aggressor baseline for SBFD co-ex study

| Victim | Aggressor | Figures | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DUD) | Case 1 | NR TDD DL | High |
|  | SBFD (DU) | Case 2 | NR TDD DL | High |
|  |  | Case 3 | NR TDD DL | Low |
| NR TDD UL | SBFD(DUD) | Case 4 | NR TDD UL | Low |
|  | SBFD(DU) | Case 5 | NR TDD UL | Low |
|  |  | Case 6 | NR TDD UL | Low |
| SBFD (DUD) | NR TDD DL | Case 1 | No system in adjacent channel | High |
| SBFD (DU) | NR TDD DL | Case 2 |  | High |
|  |  | Case 3 |  | Low |
| SBFD(DUD) | NR TDD UL | Case 4 |  | Low |
| SBFD(DU) | NR TDD UL | Case 5 |  | Low |
|  |  | Case 6 |  | Low |
| Note: The above combination sets may be down-scaled if some sets are equivalent in SLS study perspective after agreed on other assumptions. |  |  |  |  |

## Simulation assumptions

### FR1

#### Network layout model

##### Urban Macro

The details on urban macro network layout model are listed in Table 2.2.1.1.1-1.

[Editor’s Note: Table 2.2.1.1.1-1 refers to the R4-2214379, R4-2217466 and TR 38.828.]

Table 2.2.1.1.1-1: Network layout for urban macro in FR1 (4GHz)

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Path-loss model | - Macro(Aggressor) → Macro(Victim): - Macro-to-UE: UMa see TR 38.803 - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - Option 2: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.803. - X = [0.75] - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.803. - UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 - UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable. |
| BS Tx power | For legacy TDD: 49 dBm For SBFD antenna configuration 1: 46 dBm For SBFD antenna configuration 2: 49 dBm Note 1,2 |
| UE Tx power | 23 dBm |
| BS antenna configurations | - Baseline: Reuse TR 38.828 antenna model as in 2.2.1.5 For legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,4,8,2) (dH,dV)=(0.5,0.8)λ For SBFD antenna configuration 1: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Note 1,2,3,4 - Optional: Extended AAS model Section 5.2.3.2.4 of TR 38.803 For legacy TDD: TBA For SBFD: TBA |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 5 dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | Omni |
| UE antenna height | - Baseline: Reuse TR 38.828 UE dropping assumption hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 - Optional: FFS UE height if cluster-based method is used for UE dropping |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| Note 1: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 2: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. Note 3: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 4: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |  |

##### Indoor

Details on indoor network layout model are listed in Table 2.2.1.1.2-1.

[Editor’s Note: Table 2.2.1.1.2-1 refers to the R4-2214379, R4-2217466 and TR 38.828.]

Table 2.2.1.1.2-1: Network layout for Indoor scenarios in FR1 (4 GHz)

| Layouts | Total 12 BSs (operator A: 6 BSs & operator B: 6 BSs) 120 m x 50 m |
| --- | --- |
| Inter-BS distance | 20 m |
| Minimum BS-UE (2D) distance | Indoor-to-Indoor: 0 m |
| Minimum UE-UE (2D) distance | Indoor-to-Indoor: 1 m [~ 3 m] [Editor’s Note: In order to align the SLS implementation, let’s simply use 1m as min UE-UE] |
| Carrier frequency | 4G Hz |
| BS TX power | For legacy TDD: [24] dBm For SBFD antenna configuration 1: [21] dBm For SBFD antenna configuration 2: [24] dBm Note 1,2 |
| UE TX power | 23 dBm |
| Path-loss model | - Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: InH-office see TR 38.803 - BS-to-UE: InH-office see TR 38.803 - UE-to-UE: InH-office see TR 38.803 |
| BS antenna | FFS Option 1: Legacy TDD: (Mg,Ng,M,N,P) =(1,1,4,4,2), = (0.5, 0.5)λ SBFD antenna configuration 1: (Mg,Ng,M,N,P) =(1,1,2,4,2), = (0.5, 0.5)λ SBFD antenna configuration 2: (Mg,Ng,M,N,P) =(1,1,4,4,2), = (0.5, 0.5)λ Note 1,2 |
| BS antenna height: | 3 m |
| BS antenna element gain + connector loss | 3.5 dBi (assuming antenna 1.8dB loss) |
| UE antenna | Omni |
| UE antenna height | 1.5 m |
| Antenna gain of UE | 0 dBi |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5 dB |
| UE receiver noise figure | 9 dB |
| Note 1: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 2: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |

#### Adjacent channel ACLR and ACS

[Editor’s Note: This section refers to R4-2214379, R4-2217466 and TR 38.828.]

Table 2.2.1.2-1: ACLR and ACS for FR1

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 45 dBc |
| BS ACS | 46 dBc |
| UE ACLR | 30 dBc (ACLR1), 43dBc (ACLR2) Note 1 |
| UE ACS | 33 dBc |
| Note 1: Evaluate the UE-UE CLI with ACLR 1 only first, revisit ACLR2 if UE-UE CLI becomes significant. |  |

#### UE distribution

[Editor’s Note: Table 2.2.1.3-1 refers to the R4-2217466 and TR 38.828.]

Table 2.2.1.3-1: UE distribution for FR1

| Scenarios | UE distribution |
| --- | --- |
| Indoor (Indoor-to-Indoor) | 100% indoor |
| Urban Macro (Macro-to-Macro) | - Baseline: 20% indoor and 80% outdoor - Optional: 80% indoor and 20% outdoor |

#### Other simulation parameters

Table 2.2.1.4-1: Other simulation parameters for FR1

| Parameters | Indoor | Urban macro |
| --- | --- | --- |
| Channel bandwidth | 100 MHz | 100 MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 100 MHz For SBFD {DUD}: 40MHz + 40MHz For SBFD {DU}: 80MHz Note 1, 2, 3 | For legacy TDD: 100 MHz For SBFD {DUD}: 40MHz + 40MHz For SBFD {DU}: 80MHz Note 1, 2, 3 |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 100 MHz For SBFD {DUD} and {DU}: 20MHz Note 1, 2, 3 | For legacy TDD: 100 MHz For SBFD {DUD} and {DU}: 20MHz Note 1, 2, 3 |
| Traffic model | Full buffer, Note 4 | Full buffer, Note 4 |
| DL power control | NO | NO |
| UL power control | YES | YES |
| UE max TX power in dBm | 23 dBm | 23 dBm |
| UE min TX power in dBm | -33 dBm (100 MHz CBW) see TS 38.101-1 | -33 dBm (100 MHz CBW) see TS 38.101-1 |
| BS Noise figure in dB | 5 dB | 5 dB |
| UE Noise figure in dB | 9 dB | 9 dB |
| Handover margin in dB | 3 dB (Same as FR2) | 3 dB (Same as FR2) |
| BS mechanical downtilt angle in degrees | 90 degrees | 6 degrees |
| Note 1: Above sub-band BW assumption used for simulation not aligned existing RAN4 agreed CHBW sets. Note 2: Above parameters used for simulation purpose only. Note 3: Companies are encouraged to provide the assumption they used for simulation (whether guard-band assumed and the values of guard-band if any) Note 4: Start with full buffer while other RU is not precluded. Companies are encouraged to provide simulation results while indicating their RU assumption used. If the lower RU other than full buffer is suggested or implemented, the explanation of how this RU or traffic model is implemented in simulation should be provided. |  |  |

#### Baseline antenna model (TR 38.828)

##### Urban macro scenario

Table 2.2.1.5.1-1: FR1 BS antenna element pattern for Urban Macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi (assuming 1.8dB loss) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree horizontal element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

##### Indoor scenario

Table 2.2.1.5.2-1: FR1 BS antenna element pattern for Indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | [Editor’s Note: Should this be 90-deg beamwidth along with 5.5 dBi according to TS 38.921?] |
| Antenna element horizontal radiation pattern (dB) | [Editor’s Note: Should this be 90-deg beamwidth along with 5.5 dBi according to TS 38.921?] |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3.5 dBi (assuming 1.8dB loss) [Editor’s Note: Should this be 90-deg beamwidth along with 5.5 dBi according to TS 38.921?] |

##### UE antenna element pattern

The UE antenna element is assumed to be omnidirectional with 0dBi gain.

### FR2

#### Network layout model

##### Urban Macro

The details on urban macro network layout model are listed in Table 2.2.2.1.1-1.

[Editor’s Note: Table 2.2.2.1.1-1 refers to the R4-2214379, R4-2217466 and TR 38.828.]

Table 2.2.2.1.1-1: Network layout for urban macro in FR2 (30GHz)

| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| --- | --- |
| Inter-site distance | 200 m |
| BS antenna height | 25 m |
| Path-loss model | - Baseline: TR 38.828 model - Macro (Aggressor) – Macro (Victim) - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 For LoS probability for Macro-to-Macro case: - Option 1: Reuse the same model as in TR 38.828 with h_UT equals to 25m; - Option 2: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.803. - X = [0.75] - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.803. - Macro-to-UE(V): Uma + penetration loss see TR 38.803 - UE-to-UE: UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss see TR 38.803 - UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable. - Optional: TR 38.901 model |

##### Urban Micro

Details on urban micro network layout model are listed in Table 2.2.2.1.2-1.

[Editor’s Note: Table 2.2.2.1.2-1 refers to the R4-2217466 and TR 38.828.]

Table 2.2.2.1.2-1: Network layout for urban micro in FR2 (30 GHz)

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | Fixed cluster circle within a macro cell. | Note 1 |
| Number of micro BSs per macro cell | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell | < 28.9 m |  |
| BS antenna height | 10 m |  |
| Pathloss model | - Baseline: TR 38.828 model: Micro (A) – Micro (V) see TR 38.803 [5] - Micro-to-Micro: UMi (h_UE=10 m) - Micro-to-Micro UE: UMi + penetration loss - Micro (UE)-to-Micro (UE): UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss between UEs - UMi model is not applicable when 2D distance is less than 10m, instead free space model is applicable. - Optional: TR 38.901 model |  |
| Shadowing correlation | Between cite: 0.5 |  |
| Multi operator layout | Cluster circle is coordinated | Note 2 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 200 m is assumed. Note 2: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

##### Indoor

Details on indoor network layout model are listed in Table 2.2.2.1.3-1.

[Editor’s Note: Table 2.2.2.1.3-1 refers to the R4-2214379, R4-2217466 and TR 38.828.]

Table 2.2.2.1.3-1: Network layout for Indoor scenarios in FR2 (30 GHz)

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | Total 12 BSs (operator A: 6 BSs & operator B: 6 BSs) 120 m x 50 m |  |
| Inter-site distance | 20 m |  |
| BS antenna height | 3 m | ceiling |
| Path-loss model | - Baseline: TR 38.828 model: Indoor(Aggressor) → Indoor(Victim) - BS-to-BS: InH-office see TR 38.803 [5] - BS-to-UE: InH-office see TR 38.803 [5] - UE-to-UE: InH-office see TR 38.803 [5] - Optional: TR 38.901 model |  |
| Shadowing correlation | N/A |  |

#### Adjacent chanenl ACLR and ACS

Table 2.2.2.2-1: ACLR and ACS for FR2

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 28 dBc |
| BS ACS | 23.5 dBc |
| UE ACLR | 23 dBc (Note 1) |
| UE ACS | 23 dBc |
| Note 1: FR2 UE ACLR uses OBW 23 dBc as baseline. |  |

#### UE distribution

[Editor’s Note: Below tables refers to the R4-2217466 and TR 38.828.]

##### Urban Macro (Macro-to-Macro)

Table 2.2.2.3.1-1: UE distribution for Urban Macro case in FR2

| UE location | Outdoor/indoor | Outdoor and indoor |
| --- | --- | --- |
|  | Indoor UE ratio | 0% |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5 m ≦ hUT ≦ 22.5 m |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m |

##### Dense Urban (Micro-to-Micro)

Table 2.2.2.3.2-1: UE distribution for Dense Urban case in FR2

| UE location | Outdoor/indoor | Outdoor and indoor |
| --- | --- | --- |
|  | Indoor UE ratio | 80 % |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5m ~ 22.5m |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 3m |

##### Indoor-to-Indoor

Table 2.2.2.3.3-1: UE distribution for Indoor cases in FR2

| Scenarios | UE distribution |
| --- | --- |
| Indoor-to-Indoor | 100% indoor |

#### Other simulation parameters

Table 2.2.2.4-1: Other simulation parameters for FR2

| Parameters | Indoor | Urban macro | Urban micro |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | For legacy TDD: 200 MHz For SBFD {DUD}: 80MHz + 80MHz For SBFD {DU}: 160MHz Note 1, 2, 3 | For legacy TDD: 200 MHz For SBFD {DUD}: 80MHz + 80MHz For SBFD {DU}: 160MHz Note 1, 2, 3 | For legacy TDD: 200 MHz For SBFD {DUD}: 80MHz + 80MHz For SBFD {DU}: 160MHz Note 1, 2, 3 |
| Scheduled channel bandwidth per UE (UL) | For legacy TDD: 200 MHz For SBFD {DUD} and {DU}: 40MHz Note 1, 2, 3 | For legacy TDD: 200 MHz For SBFD {DUD} and {DU}: 40MHz Note 1, 2, 3 | For legacy TDD: 200 MHz For SBFD {DUD} and {DU}: 40MHz Note 1, 2, 3 |
| Traffic model | Full buffer, Note 4 | Full buffer, Note 4 | Full buffer, Note 4 |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| UE Peak EIRP in dBm | 22.4 dBm, Note5 | 22.4 dBm, Note5 | 22.4 dBm, Note5 |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 10 | 10 | 10 |
| UE Noise figure in dB | 10 | 10 | 10 |
| Handover margin in dB | 3dB | 3dB | 3dB |
| BS mechanical downtilt in degrees | 90 degrees | 6 degrees | 10 degrees [or other value] |
| Note 1: Above sub-band BW assumption used for simulation not aligned existing RAN4 agreed CHBW sets. Note 2: Above parameters used for simulation purpose only. Note 3: Companies are encouraged to provide the assumption they used for simulation (whether guard-band assumed and the values of guard-band if any) Note 4: Start with full buffer while other RU is not precluded. Companies are encouraged to provide simulation results while indicating their RU assumption used. If the lower RU other than full buffer is suggested or implemented, the explanation of how this RU or traffic model is implemented in simulation should be provided. Note 5: Peak EIRP of 22.4 dBm is the baseline assumption, other higher EIRP is optional and companies could provide simulation results with statement of higher EIRP. |  |  |  |

#### Baseline antenna configuration (TR 38.828)

##### Urban macro scenario

Table 2.2.2.5.1-1: FR2 BS antenna modelling for urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS TX power | For legacy TDD: [30] dBm For SBFD antenna configuration 1: [27] dBm For SBFD antenna configuration 2: [30] dBm Note 4,5 |
| (Mg, Ng, M, N, P) | For legacy TDD: (1, 1, 8, 16, 2) For SBFD antenna configuration 1: (1, 1, 4, 16, 2) For SBFD antenna configuration 2: (1, 1, 8, 16, 2) Note 1,2,4,5 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. Note 4: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 5: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |

##### Dense urban scenario

Table 2.2.2.5.2-1: FR2 BS antenna element pattern for dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS TX power | For legacy TDD: [30] dBm For SBFD antenna configuration 1: [27] dBm For SBFD antenna configuration 2: [30] dBm Note 4,5 |
| (Mg, Ng, M, N, P) | For legacy TDD: (1, 1, 8, 16, 2) For SBFD antenna configuration 1: (1, 1, 4, 16, 2) For SBFD antenna configuration 2: (1, 1, 8, 16, 2) Note 1,2,4,5 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results Note 4: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 5: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |

##### Indoor scenario

Table 2.2.2.5.3-1: FR2 BS antenna element pattern for indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 2dB loss) |
| BS TX power | For legacy TDD: [TBD/24] dBm For SBFD antenna configuration 1: [TBD/21] dBm For SBFD antenna configuration 2: [TBD/24] dBm Note 4,5 [Editor’s Note: The Tx power of 24 and 21 with 4x8 elements were derived from [30] dBm with 8x16 elements in Urban Macro configuration.] |
| (Mg, Ng, M, N, P) note | For legacy TDD: (1, 1, 4, 8, 2) For SBFD antenna configuration 1: (1, 1, 2, 8, 2) For SBFD antenna configuration 2: (1, 1, 4, 8, 2) Note 1,2,4,5 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 90 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation Note 4: SBFD antenna configuration 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. Note 5: SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. |  |

##### UE antenna element pattern

Table 2.2.2.5.4-1: FR2 UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |
| UE antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees Note 3 Fixed elevation: 90 degrees |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note3: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. Note 4: A 90 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation |  |

### Common assumptions

#### Baseline propagation model (TR 38.828)

The Path loss model is summarized in Table 2.2.3.1-1. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 2.2.3.1-1.

Table 2.2.3.1-1: Path-loss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

#### LOS model

The Line-Of-Sight (LOS) probabilities are given in Table 2.2.3.2-1.

Table2.2.3.2-1: LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for Uma |  |

#### O-to-I penetration loss

The Path loss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 2.2.3.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 2.2.3.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 2.2.3.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 2.2.3.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 2.2.3.3-2: O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 2.2.3.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

#### Transmission power control model

[Editor’s Note: This sectoin refers to R4-2214379.]

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure + 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2 are 15 dB.

#### Received signal power model

[Editor’s Note: This sections refers to TR 38.828.]

The following model is applied.

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

RX_PWR is the received power.

TX_PWR is the transmitted power.

G_TX is the transmitter antenna gain (directional array gain).

G_RX is the receiver antenna gain (directional array gain).

#### Evaluation metric

[Editor’s Note: This section refers to R4-2214379, R4-2217466, TR 38.828.]

Follow the TR 38.828’s evaluation criteria to check the 50% and 5% throughput loss compared to the baseline scenario defined.

Table 2.2.3.6-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

#### Grid shift consideration

[Editor’s Note: This section refers to R4-2217466.]

Grid shift uses 100% as baseline for simulation while other grid shifts less than 100% FFS. 0% is not precluded.

  - If other grid shift than 100% is proposed, the definition and implementation of such grid shift should be provided accordingly.

  - Other values less than 100% would wait for the feasibility study conclusion of co-site case.

    - FFS the minimum implementable grid-shift value to be studied in simulation. It would depend on the definition of the grid shifts, the co-site and/or inter-site gNB-gNB isolation value agreed, etc.

#### Simulation description

[Editor’s Note: This section refers to R4-2214379.]

Adopt following simulation steps, while the victim, aggressor and aggressor baseline are given in Table 2.1-2:

1.	Aggressor and victim network are generated.

- UEs are distributed according to the agreements on UE distribution. (NOTE 1)

2.	UE associations: UEs are associated to base station based on coupling loss.

- Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.  (NOTE 2)

When legacy TDD system is victim, follow steps 4a:

4a.	Throughput is computed considering ACI from another static TDD system, with the same TDD UL/DL configuration, as baseline aggressor:

- , where  is the inter-cell interference and  is the adjacent channel interference from the baseline aggressor.

When SBFD system is victim, follow steps 4b:

4b.	Throughput is computed in the victim system without considering ACI as below:

- , where  is the inter-cell interference.

5.	Throughput is computed considering ACI below:

- , where  is the adjacent channel interference. . Throughput shall be reported separately for aligned TDD slots (i.e. legacy slots) and SBFD slots.

NOTE 1: 	UE dropping methods agreements:

    - FFS following UE dropping method options

  - Option 1: Both evenly random dropping and cluster-based method considered in Urban Macro scenario;

  - Option 2: Only evenly random dropping considered in Urban Macro scenario.

    - Consider cluster-based method in a dedicate scenario, [Urban Hotspot] scenario, than Urban Macro scenario.

    - FFS the details of the cluster-based UE dropping methodology.

NOTE 2: 	The noise floor of SBFD as {N = noise floor + XdB}, and for the value of X, taking 1dB sensitivity degradation due to self-interference of DL transmission as starting point for system level evaluation and feasibility study.

- Other values lower than 1dB e.g. 0.1dB/0.8dB not precluded pending on the feasibility study

- Final values used in co-existence evaluation shall be aligned with feasibility analysis conclusion.

# Conclusion

It is proposed to use the simulation assumptions in this paper for SBFD adjacent channel co-existence study.

Proposal 1: This document is proposed as a baseline and living document, so that it will capture the existing and future agreements and way forwards for RAN4 adjacent channel co-ex study.

# Reference

[1] R4-2214378, “WF on adjacent channel co-existence study”, Samsung.

[2] R4-2214379, “WF on simulation assumption for adjacent co-existence study”, CMCC.

[3] R4-2217466, “WF for adjacent channel co-existence evaluation of SBFD operation”, Samsung, CMCC.

[4] TR 38.828

[5] TR 38.921

[6] TS 38.101-1

[7] TS 38.101-2

[8] TR 38.803

[9] TR 38.901

[10] TR 36.942