3GPP TSG-RAN WG4 Meeting #105              	R4-2219019

Toulouse, France, 14th Nov-18th Nov, 2022

Agenda item:	6.2.6.2.3

Source:	Samsung

Title:	Simulation results on PRACH demodulation requirement for Rel-17 NTN

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the detail test setup for PRACH demodulation requirement with NTN was agreed. The related agreement was captured into the WF [1].

In this contribution, the ideal and impairment results are provided for requirement derivation.

# 2	Initial simulation results

In this section, based on the agreed simulation assumption in the last meeting, the simulation results for different are provided for simulation alignment purpose

Table 1. Ideal and impairment simulation results for PRACH

| Case | Tx/Rx | Format | SCS | PRACH sequence length | Ncs | Logical Sequence index | v | Propagation condition | Frequency offset | Time estimation tolerance | Timing offset | SNR (ideal) | SNR (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 0 | 1.25KHz | 839 | 13 | 22 | 32 | AWGN | 0 | 1.04us | 0.9us | -13.8 | -11.8 |
| 2 | 1T2R | 0 | 1.25KHz | 839 | 13 | 22 | 32 | AWGN | 0 | 1.04us | 0.9us | -16.3 | -14.3 |
| 3 | 1T1R | 2 | 1.25KHz | 839 | 13 | 22 | 32 | AWGN | 0 | 1.04us | 0.9us | -18.9 | -16.9 |
| 4 | 1T2R | 2 | 1.25KHz | 839 | 13 | 22 | 32 | AWGN | 0 | 1.04us | 0.9us | -21.2 | -19.2 |
| 5 | 1T1R | B4 | 15KHz | 139 | 23 | 0 | 0 | AWGN | 0 | 0.52us | 0.8us | -16.3 | -14.3 |
| 6 | 1T2R | B4 | 15KHz | 139 | 23 | 0 | 0 | AWGN | 0 | 0.52us | 0.8us | -19.1 | -17.1 |
| 7 | 1T1R | C2 | 15KHz | 139 | 23 | 0 | 0 | AWGN | 0 | 0.52us | 0.8us | -11.5 | -9.5 |
| 8 | 1T2R | C2 | 15KHz | 139 | 23 | 0 | 0 | AWGN | 0 | 0.52us | 0.8us | -14.3 | -12.3 |
| 9 | 1T1R | B4 | 30KHz | 139 | 46 | 0 | 0 | AWGN | 0 | 0.26us | 0.8us | -16.2 | -14.2 |
| 10 | 1T2R | B4 | 30KHz | 139 | 46 | 0 | 0 | AWGN | 0 | 0.26us | 0.8us | -18.9 | -16.9 |
| 11 | 1T1R | C2 | 30KHz | 139 | 46 | 0 | 0 | AWGN | 0 | 0.26us | 0.8us | -11.4 | -9.4 |
| 12 | 1T2R | C2 | 30KHz | 139 | 46 | 0 | 0 | AWGN | 0 | 0.26us | 0.8us | -14.1 | -12.1 |
| 13 | 1T1R | 0 | 1.25KHz | 839 | 13 | 22 | 32 | NTN-TDLA100-200 | 200 | 1.32us | 0.9us | -1.8 | 0.2 |
| 14 | 1T2R | 0 | 1.25KHz | 839 | 13 | 22 | 32 | NTN-TDLA100-200 | 200 | 1.32us | 0.9us | -8.4 | -6.4 |
| 15 | 1T1R | 2 | 1.25KHz | 839 | 13 | 22 | 32 | NTN-TDLA100-200 | 200 | 1.32us | 0.9us | -11.1 | -9.1 |
| 16 | 1T2R | 2 | 1.25KHz | 839 | 13 | 22 | 32 | NTN-TDLA100-200 | 200 | 1.32us | 0.9us | -16.5 | -14.5 |
| 17 | 1T1R | B4 | 15KHz | 139 | 23 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.8us | 0.8us | -4.6 | -2.6 |
| 18 | 1T2R | B4 | 15KHz | 139 | 23 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.8us | 0.8us | -11.3 | -9.3 |
| 19 | 1T1R | C2 | 15KHz | 139 | 23 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.8us | 0.8us | -0.2 | 1.8 |
| 20 | 1T2R | C2 | 15KHz | 139 | 23 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.8us | 0.8us | -7.0 | 5.0 |
| 21 | 1T1R | B4 | 30KHz | 139 | 46 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.55us | 0.8us | -6.6 | -4.6 |
| 22 | 1T2R | B4 | 30KHz | 139 | 46 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.55us | 0.8us | -12.5 | -10.5 |
| 23 | 1T1R | C2 | 30KHz | 139 | 46 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.55us | 0.8us | -2.0 | -0.0 |
| 24 | 1T2R | C2 | 30KHz | 139 | 46 | 0 | 0 | NTN-TDLA100-200 | 200 | 0.55us | 0.8us | -7.9 | -5.9 |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results of PRACH are provided for requirement derivation.

# References

R4-2217348, WF on NTN SAN demodulation performance requirements, Huawei