# 3GPP TSG-RAN WG4 Meeting # 105                                    R4-2219046

# Toulouse, France, Nov. 14-Nov. 18, 2022

# Title: 	Discussion on 8Rx UE demodulation and CSI

# Source: 	ZTE Corporation

# Agenda item:	8.6.6.1

# Document for:	Approval

# 1 Introduction

There were several objectives in the Rel-18 FR1 enh WID [1], in which for 8Rx, the objectives are listed in the following:

Enable 8Rx for CPE/FWA/vehicle/industrial devices [RAN4]

- Objective of Performance part WI

The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include:

Example bands:

- TDD bands: n41, n77/ n78

- FDD bands: n7

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

- Specify RLM test cases to support 8Rx

  - Investigate if the existing 4Rx RLM test can be reused or the new test will be specified

- Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

  - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

- Specify the SDR requirements with 8 MIMO layers

In this contribution, we give some initial discussions on performance part related to 8Rx.

# 2	Discussion

The benefit to support 8Rx in NR have already been identified, such as improve the downlink throughput for a single user and enhance the coverage for the uses located at the cell edge. Also to keep the similar MIMO performance as LTE due to 8Rx have already been supported for LTE.

2.1 Static propagation conditions

In TS 38.101-4, the existing model of static propagation conditions are specified for 2Rx and 4Rx. In our understanding, it is becoming essential to introduce static propagation conditions for 8Rx. Also had specified the propagation conditions in LTE for 8Rx. For those conditions, it can keep similar MIMO performance as LTE. As regards to static propagation conditions, could be reused as LTE for 8Rx requirements.

Observation 1: Define static propagation conditions for 8Rx, include 1 port、2 port、4 port and 8port.

According to the existing model of LTE 8Rx antenna ports, the static propagation conditions for NR 8Rx can be defined as following:

Proposal 1: Define the following H matrix as 8Rx static channel matrices.

For 1 port transmission the channel matrix is defined in the frequency domain by

For 2 port transmission the channel matrix is defined in the frequency domain by

For 4 port transmission the channel matrix is defined in the frequency domain by

For 8 port transmission the channel matrix is defined in the frequency domain by

2.2 MIMO Channel Correlation Matrices

Like the same as the static propagation conditions, MIMO channel correlation matrices had specified for 2Rx and 4Rx in TS 38.101-4. So it is necessary to specified the MIMO channel correlation matrices for 8Rx.

Observation 2: Define MIMO channel correlation matrices RgNB and RUE for 8Rx.

Proposal 2: Define the following matrix RgNB and RUE as MIMO channel correlation matries.

In terms of WID, it had clearly specified the CPE/FWA/vehicle/industrial devices for 8Rx. Considering the size and implementation for those equipment, low correlation between antennas can be satisfied to a certain extent. On the other hand, only low correlation and medium correlation were specified for LTE 8Rx. Therefore, can we consider low correlation for NR 8Rx in the initial stage.

Proposal 3: Define 8Rx performance with MIMO channel of low correlation.

2.3 Simulation assumptions

In RAN#96e meeting, the work item [1] on study on RF requirements enhancement for NR frequency range was approved as one of Rel-18 RAN4 package. In this section, we want to provide initial simulation assumptions for 8Rx.

2.3.1 PDSCH

From the WID[1] of objective, TDD bands(e.g. n41, n77/ n78) and FDD bands(e.g. n7) have been specified for 8Rx. Thus, we want to provide simulation assumptions for PDSCH in this section.

Table 2-1: Simulation assumptions for PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel Bandwidth/SCS |  | MHz/KHz | 40/30 for TDD and 10/15 for FDD |
| TDD pattern |  |  | 7D1S2U S=6D+4G+4U |
| MCS |  |  | 13 for low rank |
|  |  |  | 19 for high rank |
| MIMO configuration |  |  | 1x8, 2x8, ULA Low for Rank 1 2x8, ULA Low for Rank 2 4x8, 4x8 ULA Low for Rank 3, 4 8x8, ULA Low for high Rank 5,6 7, 8 |
| Propagation conditions |  |  | TDLA30-10 |
| HARQ process number |  |  | 4 for FDD and 8 for TDD |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Antenna ports indexes |  | {1000} for Rank 1 {1000, 1001} for Rank 2 {1000-1002} for Rank 3 {1000-1003} for Rank 4 {1000-1004} for Rank 5 {1000-1005} for Rank 6 {1000-1006} for Rank 7 {1000-1007} for Rank 8 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |

Proposal 4. To consider Table 2-1 as PDSCH simulation assumptions for 8Rx requirements.

2.3.2 PDCCH

In this section, we want to provide some simulation assumptions for PDCCH. In our understanding, we can reuse simulation assumptions of 2Rx and 4Rx for PDCCH. Thus, we give some assumptions for 8Rx as follows.

2.3.2.1  TDD

1 Tx Antenna

Table 2-2: Simulation assumptions for PDCCH

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 102 | 1 | 2 | R.PDCCH. 2-1.1 TDD | TDLA30-10 | 1x8 Low |
| 2 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300-100 | 1x8 Low |
| 3 | 40 | 48 | 2 | 16 | R.PDCCH. 2-2.1 TDD | TDLA30-10 | 1x8 Medium A |

2 Tx Antenna

Table 2-3: Simulation assumptions for PDCCH

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40 | 90 | 1 | 8 | R.PDCCH. 2-1.3 TDD | TDLC300-100 | 2x8 Low |

The parameters for the fixed reference channel R.PDCCH TDD were list in the Table 2-4.

Table 2-4: Parameters for fixed reference channel R.PDCCH TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-1.1 TDD | R.PDCCH.2-1.2 TDD | R.PDCCH.2-1.3 TDD | R.PDCCH.2-2.1 TDD |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 90 | 48 |
| CORESET time domain allocation |  | 1 | 1 | 1 | 2 |
| Aggregation level |  | 2 | 4 | 8 | 16 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 1_0 |
| Payload (without CRC) | Bits | 41 | 53 | 53 | 41 |

2.3.2.2  FDD

1 Tx Antenna

Table 2-5: Simulation assumptions for PDCCH

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x8 Low |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x8 Low |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x8 Low |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x8 Low |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x8 Medium A |

2 Tx Antenna

Table 2-6: Simulation assumptions for PDCCH

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x8 Low |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x8 Low |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x8 Low |

The parameters for the fixed reference channel R.PDCCH FDD were list in the Table 2-7 and Table 2-8.

Table 2-7: Parameters for fixed reference channel R.PDCCH FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD |
| Subcarrier spacing | kHz | 15 | 15 |
| CORESET frequency domain allocation |  | 48 | 48 |
| CORESET time domain allocation |  | 1 | 1 |
| Aggregation level |  | 4 | 4 |
| DCI Format |  | 1_0 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 |

Table 2-8: Parameters for fixed reference channel R.PDCCH FDD

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 |

Proposal 5. To consider Table 2-2 - Table 2-8 as PDCCH simulation assumptions for 8Rx requirements.

2.3.3 PBCH

In this section, we want to provide initial simulation assumptions for PBCH to support 8Rx.

2.3.3.1 TDD

Table 2-9: simulation assumptions for PBCH

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth / Subcarrier spacing | MHz /kHz | 40/30 |
| TDD UL-DL pattern |  | 7D1S2U S=6D+4G+4U |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| Propagation condition |  | TDLA30-10 |
| Antenna configuration and correlation matrix |  | 1 x 8 Low |
| Modulation |  | QPSK |
| Target coding rate |  | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 |

2.3.3.2 FDD

Table 2-10: simulation assumptions for PBCH

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth / Subcarrier spacing | MHz/KHz | 10/15 |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| Propagation condition |  | TDLC300-100 |
| Antenna configuration and correlation matrix |  | 1 x 8 Low |
| Modulation |  | QPSK |
| Target coding rate |  | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 |

Proposal 6. To consider Table 2-9 and Table 2-10 as PBCH simulation assumptions for 8Rx requirements.

# 3 Conclusion

In this contribution, we give some initial discussion on UE Performance requirements to support 8Rx, including static propagation conditions, MIMO channel correlation matrices and simulation assumptions for PDSCH, PDCCH and PBCH. The conclusions are:

Observation 1: Define static propagation condition for 8Rx, include 1 port、2 port、4 port and 8port.

Proposal 1: Define the following H matrix as 8Rx static channel matrices.

For 1 port transmission the channel matrix is defined in the frequency domain by

.

For 2 port transmission the channel matrix is defined in the frequency domain by

.

For 4 port transmission the channel matrix is defined in the frequency domain by

.

For 8 port transmission the channel matrix is defined in the frequency domain by

Observation 2: Define MIMO channel correlation matrices RgNB and RUE for 8Rx.

Proposal 2: Define the following matrix RgNB and RUE as MIMO channel correlation matries.

Proposal 3: Define 8Rx performance with MIMO channel of low correlation.

Proposal 4. To consider Table 2-1 as PDSCH simulation assumptions for 8Rx requirements.

Proposal 5. To consider Table 2-2 ~Table 2-8 as PDCCH simulation assumptions for 8Rx requirements.

Proposal 6. To consider Table 2-9 and Table 2-10 as PBCH simulation assumptions for 8Rx requirements.

# 4	Reference

[1]  RP-221496, WID revision: RF requirements enhancement for NR frequency range, Huawei, HiSilicon.