3GPP TSG-RAN WG4 Meeting #105	R4-2219144

Toulouse, France, November 14 – November 18, 2022

Source: 	Huawei, HiSilicon

Title: 	Link level simulation results for FR2 UL 256QAM

Agenda Item:	8.7.2

Document for:	Discussion

- Introduction

For FR2 NR RF requirements enhancement phase 3 [1], one of the objective is UL 256QAM.

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

In this contribution, we provide a link level simulation to study the gain and operating SNR, taking phase noise and other RF impairment into account.

- Discussion

2.1	Simulation assumption

With the purpose to evaluate the impact of RF impairments, we did link level simulation to check the performance benefit of 256QAM. The following simulation assumptions are used as agreed in [2].

Table 2-1 Simulation assumptions for throughput comparison of 256QAM and 64QAM

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

The link lever simulation results are shown in Figure 2-1, Figure 2-2, Figure 2-3 and Figure 2-4 for TDL-D in different frequency ranges. From the simulation results, it is shown that support 256 QAM can provide significant performance gain over 64QAM where the UE is in good propagation condition.

Figure 2-1 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_29 GHz

Figure 2-2 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_29 GHz

Figure 2-3 Throughput comparison of MCS 21 256QAM and 64QAM for RANK1_39GHz

Figure 2-4 Throughput comparison of MCS 23 256QAM and 64QAM for RANK1_39GHz

- Conclusion

In the contribution, we provide proposals for the simulation assumption and preliminary simulation results to study the gain and operating SNR for UL 256QAM.

- Reference

- RP-221862, “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi, RAN Meeting #96, June 2022

- R4-2214453, “WF on UL 256QAM”, Xiaomi