3GPP TSG-RAN WG4 Meeting # 105 	R4-2219150

# Toulouse, France, November 14 – November 18, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for definition of FR2 multi-band BS

Agenda Item:	8.4.2

Document for:	Approval

1 Introduction

New SID on NR BS RF requirement evolution was approved in RAN#95-e [1]. The TR skeleton for the SI dealing with FR2 multi-band BS deployments was approved in [2]. On the definition of FR2 multi-band BS, the following agreements are made in [3].

## Definition of multi-band RIB

Agreement:

  - To reuse the current FR1 definition of multi-band RIB for FR2.

## Definition of multi-band BS

Agreement:

  - No need to introduce the definition of multi-band BS for FR2 into specification

In this contribution, we provide a TP on deployment scenarios to capture these agreement into the TR 38.877.

2 TP for TR 38.877

6.1	Definition of FR2 multi-band BS

As with FR1 there is no explicit definition of a MB BS needed for FR2, the existing definitions on multi-band RIB and explanations on BS capable of supporting operation in multiple operating bands in 38.104 [xx] for FR1 are sufficient. The same descriptions can be applied to FR2 i.e.

BS type 1-O and BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

For single-band RIB and multi-band RIB, the same definitions as for FR1 could be reused, i.e.

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band.

single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB.

3 Conclusion

It is proposed to approve the TP for TR 38.877.

References

[1] RP-221001, “New SID: NR BS RF requirement evolution”, Huawei, HiSilicon

[2] R4-2214779, “TR skeleton for NR mmWave MB-BS”, Huawei, HiSilicon

[3] R4-2217450, “WF on definition of FR2-1 multi-band BS”, CATT