3GPP TSG-RAN WG4 Meeting #105	R4-2219255

Toulouse, France, November 14 – 19, 2022

Title: 	Discussion on RRM impacts due to support intra-band non-collocated CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	8.11.3

Document for:	Discussion

Introduction

In R18, new WI [1] for support of intra-band non-collocated EN-DC/NR-CA deployment was approved, and the objectives of this WI are described as follows:

- Phase I: - Study the feasibility to support non-co-located scenarios for both FR1 inter-band EN-DC with overlapping and FR1 intra-band NR-CA, except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17 - Investigate the tolerable power imbalance between carriers - Investigate the required arrival time difference between CCs - Evaluate the UE performance under the power imbalance and arrival time difference - Discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78 - Investigate whether the power imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the power imbalance based on the outcome of the investigation. - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS - NOTE: Revisit whether or not to study the feasibility for support of 4Rx chains with up to 4-layer MIMO at RAN#97. This 4-layer is only for DL in Rel-18. - NOTE: Revisit whether or not to study the technical solutions can be applied to EN-DC case regardless of band contiguity at RAN#97. - Phase II: - Phase II work will get started after the feasibility in phase I is confirmed - Specify MRTD and MTTD requirements in non-collocated deployment - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

In this contribution, we will provide the discussion on MRTD and MTTD requirements for FR1 intra-band EN-DC/NR-CA in non-collocated deployment.

Discussion

In RAN4, the UE RF architecture for supporting intra-band EN-DC/NR-CA in non-collocated deployment was discussed in RF session.

For 2 layer MIMO case, the following agreements were captured in WF [2]

<RAN4#104e> Issue 2-2-1 (a): UE RF architecture Agreement: - Total four RF antenna is assumed. - Reuse UE RF architecture of inter-band non-contiguous DC_42_n77/78 EN-DC Type-2 (i.e. 2 layer/2 Rx Chain per CC, total 4 Rx Chain) as the baseline.

It can be observed that the UE RF architecture of Type 2 UE can be reused for supporting intra-band non-collocated EN-DC/NR-CA, where Type 2  UE is assumed to have separate Rx chains for different CCs.

For EN-DC band combination DC_42_n77/78, UE capability of interBandMRDC-WithOverlapDL-Bands-r16 is defined as follows in TS38.306:

| interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

For Type 2 UE, the MRTD requirements defined in 38.133 for inter-band case shall be applied. With the same UE RF architecture, the MRTD requirements defined in 38.133 for inter-band case also can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

Proposal 1: For 2 layer MIMO case, the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

For 4 layer MIMO case, the following agreements were captured in WF [3]

< Issue 2-1-1: Possible UE RF architecture candidates on new Type UE> Agreement: - Agree the following table as UE RF architecture candidates on new Type UE for non-collocated non-contiguous intra-band NR-CA and inter-band EN-DC - Check whether there are other restrictions except for power imbalance. If some other restrictions are found, moderator will add them to the table immediately - Prioritize Type 3a/3b discussion for 4MIMO layer in Rel-18. UE Type; CC#; antenna / LNA; Mixer; Analog BB; #Rx; Frequency Separation between 2cc; NRCA/ENDC; power imbalance; comment. 1; 1; 4 shared; 4 shared; 4 shared; 4Rx; ≤ X MHz; NRCA,ENDC; 6dB full range; Baseline architecture (i.e. legacy architecture). 2; 4Rx. 2; 1; 2; 4 total; 2; 2; 2Rx; No limitation or ≤ X MHz; NRCA,ENDC; 25dB full range; Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies. 2; 2; 2; 2; 2Rx. 3a; 1; 4 shared; 4; 4; 4Rx; No limitation or ≤ X MHz; ENDC; 6<P≤25dB partial range; Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range. 2; 2; 2; 2Rx. 3b; 1; 4 shared; 4; 4; 4Rx; No limitation or ≤ X MHz; NRCA,ENDC; 6<P≤25dB partial range; Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range. 2; 4; 4; 4Rx. 4a; 1; 4; 6 total; 4; 4; 4Rx; No limitation or ≤ X MHz; ENDC; 25dB full range; Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only. 2; 2; 2; 2; 2Rx. 4b; 1; 4; 8 total; 4; 4; 4Rx; No limitation or ≤ X MHz; NRCA,ENDC; 25dB full range; Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only. 2; 4; 4; 4; 4Rx.

Type 3a/3b UE is prioritized for 4 layer MIMO. It can be observed that the UE RF architecture of Type 3a/3b UE have partially separate RF chains for different CCs but with the shared LNA. Type 3a/3b UE can be assumed to perform separate time/frequency tracking for different CCs. However, the impact of larger RTD on the shared LNA needs to be further studied.

Proposal 2: For 4 layer MIMO case, RAN4 further study whether the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

Conclusions

In this paper we provided our initial analysis on MRTD/MTTD requirements for FR1 intra-band EN-DC/NR-CA in non-collocated deployment. The followings are provided:

Proposal 1: For 2 layer MIMO case, the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

Proposal 2: For 4 layer MIMO case, RAN4 further study whether the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for intra-band EN-DC/NR-CA in non-collocated deployment.

Reference

- RP-222309,  “New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment”, KDDI

- R4-2217733, “WF on NonCol_intraB_ENDC_NR_CA for NR-CA Type-2 UE”, KDDI

- R4-2217733, “WF on NonCol_intraB_ENDC_NR_CA for New Type UE”, KDDI