3GPP TSG-RAN WG4 Meeting #105                                                            R4-2219276

Toulouse, France, 14 Nov. – 18 Nov. 2022

Agenda Item:	6.5.3.1.1

Source:	Ericsson

Title:	Update simulation results for PDSCH requirement for HST-SFN

Document for:	Information

# 1	Introduction

After the discussion in RAN 104bis-e, companies agreed to consider MCS13 for defining requirement for HST-SFN scheme B.

### Issue 2-1-2: MCS for HST-SFN scheme B Agreement: - Introduce HST-SFN scheme B with MCS 13 for both FDD and TDD

According to the agreement under the discussion of SNR value for PDSCH requirement with HST-SFN scheme A and B, companies are encouraged to further check the simulation results and update (if needed) in the next meeting.

### Issue 2-1-4: SNR value for PDSCH requirement with HST-SFN scheme A and B Agreement: - Specify the SNR requirement = average impairment results+ extra margin based on simulation summary R4-2215728 - Extra margin =0.5 dB for QPSK and 16QAM - For the test case with larger span than [2.5] dB, further check the details of simulation assumption and further discuss extra margin on a case-by-case basis, leave TBD for SNR requirement in this meeting if no further updated. - Companies are encouraged to further check the simulation results and updated in the next meeting

In this contribution, we are going to re-submit our simulation results (including ideal and impairment) for HST-SFN scheme A & B with MCS13 for alignment.

# 2	Discussion

## 2.1 SFN scheme A

Following the simulation assumptions and parameter configurations agreed previously, we have provided our simulations results:

Table 2.1-1 Summary for SFN scheme A results

| Case Nunmber | Duplex & Antenna | MCS | SNR @ 70% max Tput (Ideal) | SNR @ 70% max Tput (Impairment) |
| --- | --- | --- | --- | --- |
| 1 | FDD, 2x2 | 13 | 8.6dB | 10.6dB |
| 2 | FDD, 2x4 |  | 5.7dB | 7.7dB |
| 3 | TDD, 2x2 |  | 10.1dB | 12.1dB |
| 4 | TDD, 2x4 |  | 7.6dB | 9.6dB |

## 2.2 SFN scheme B

Simulation results summary for SFN scheme B:

Table 2.2-1 Summary for SFN scheme B results

| Case Nunmber | Duplex & Antenna | MCS | SNR @ 70% max Tput (Ideal) | SNR @ 70% max Tput (Impairment) |
| --- | --- | --- | --- | --- |
| 1 | FDD, 2x2 | 13 | 8.1dB | 10.1dB |
| 2 | FDD, 2x4 |  | 4.8dB | 6.8dB |
| 3 | TDD, 2x2 |  | 9.2dB | 11.2dB |
| 4 | TDD, 2x4 |  | 6.9dB | 8.9dB |

# 3	Summary

In summary, we shared our simulation results for both scheme A and scheme B including 2Rx and 4Rx for FDD and TDD.

# References

[1]	R4-2214397: WF on UE demodulation and CSI requirements for FeMIMO, Samsung

## Annex: Simulation configurations

Here we provide our simulation assumptions for information.

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern | / | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |
| Maximum Doppler shift | Scheme A: 870Hz |  |
| MCS | 13 |  |
| Modeling of TRP pre-compensation | Assume perfect modeling of TRP pre-compensation |  |