3GPP TSG-RAN WG4 Meeting # 105	R4-2219333

Toulouse, France, November 14-18, 2022

Source:	Ericsson

Title:	On timing requirements

Agenda item:	8.13.4.3

Document for:	Approval

# 1	Introduction

In this contribution we discuss issues related to timing and frequency adjustments based on the RRM and co-existence WF from previous meeting RAN4#104bis-e [1,2].

# 2	Discussion

## 2.1 	Initial transmit timing requirements

Regrading Initial transmit timing requirements, we have this issue from WF [1]:

Issue 3-3-1: UE transmit timing Issue 3-3-1-1: Initial transmit timing requirements Te - Option 1: The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline. (CATT) - Option 2: Reuse the legacy R15 TN requirement for initial transmit timing requirement Te with the assumption of ISD 100-200km. The issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session. (CMCC, HW, ZTE, Ericsson)

As stated above, RRM agreement of ISD assumption of 100-200 km translates to cell ranges (BS to UE max distance) from 50 km to > 200 km. Coexistence ISD assumption of 200 km translate to ranges from 100 km to > 200 km. Only NTN covers the entire ISD and cell range set agreed, so far.

Observation 1	Only NTN cover the entire ISD and cell range set agreed, so far in RRM (100-200 km) and UE RF coexistence (up to 200 km).

Proposal 1	The requirements for NTN UE can be used as baseline for the initial transmit timing Te.

## 2.2 	Gradual timing adjustment

Regrading Gradual timing adjustment, we have this issue from WF [1]:

Issue 3-3-1-2: Gradual timing adjustment - Option 1: The requirements of gradual timing adjustment for TN or NTN in Rel-17 can be used as baseline if UE TA pre-compensation is used, otherwise Tp and Tq may be need updated. (CATT) - Option 2: Tp and Tq shall be updated. (HW, CMCC, Ericsson, ZTE) - Option 2-1: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS. (CMCC, Ericsson, HW)

In [3] WI Rapporteur CMCC analyze Tp and Tq parameters of Gradual timing adjustment:

Regarding the exact Tp and Tq value, Tq should include time drift due to frequency error per 200ms, time drift due to UE movement per 200ms, and 1.5Ts DigRF error. We provide the calculation in Table 3 with the assumption of max UE speed 1200km/h.

Table 3. Time drift calculation

| Factors | Time drift |
| --- | --- |
| Frequency error | [0.1]PPM |
| Time drift due to frequency error | [20]ns |
| speed of UE movement | 1200km/h aircraft |
| Time drift due to UE movement per [200]ms | 222.2ns |
| Sum of time drift above | 7.45Ts |
| DigRF error | 1.5Ts |

Considering of the sample period of UL transmission is 4Ts in 15kHz and 30kHz SCS, the Tq used by UE should be (4*7.45/4+1.5)Ts, which is 9.5Ts.

Proposal 22: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS.

Observation 2 	The analysis in R4-2215505, Discussion on RRM requirements for ATG, CMCC is correct.

Proposal 2 	For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Only NTN cover the entire ISD and cell range set agreed, so far in RRM (100-200 km) and UE RF coexistence (up to 200 km).

Observation 2 	The analysis in R4-2215505, Discussion on RRM requirements for ATG, CMCC is correct.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The requirements for NTN UE can be used as baseline for the initial transmit timing Te.

Proposal 2 	For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS.

# References

- R4-2217256, WF on NR ATG RRM core requirements, CMCC

- R4-2217736, WF on FR1 co-existence evaluation for ATG, CMCC

- R4-2215505, Discussion on RRM requirements for ATG, CMCC