3GPP TSG-RAN WG4 Meeting # 105	R4-2219340
Toulouse, France, November 14 – November 18, 2022

Agenda item:	8.15.2

Source:	CAICT

Title:	Framework for FR2 MIMO OTA performance requirements development

Document for:	Approval

1	Introduction

Defining the framework for FR2 MIMO OTA requirements development is listed as the 1st priority core part objective is the Rel-18 MIMO OTA WID [1]. In the last meeting, the group reached plenty of agreements and specified several way-forward actions [2].

The framework is scheduled to be revisited in RAN#98 to check the feasibility and progress of hybrid simulation and measurement approach. It was agreed that RAN4 should reach consensus on detailed framework/procedure on how to perform hybrid approach before RAN#98.

Issue 2-1-1: General views on the FR2 MIMO OTA requirements development Agreements: - Proposal 1: RAN4 should reach consensus on detailed framework/procedure on how to perform hybrid approach before RAN#98, to ensure and confirm the feasibility of the framework.

This is the last meeting before RAN#98 plenary, thus, we propose a framework for FR2 MIMO OTA performance requirements development based on the agreements in the WF [2]. The framework can be updated based on the discussion outcomes during this meeting.

2	Framework for FR2 MIMO OTA performance requirements development (for approval)

2.1 Overall work flow

The overall work flow of FR2 MIMO OTA performance requirements development is illustrated in Fig. 1.

Fig. 1.  Work flow of FR2 MIMO OTA performance requirements development

In general, either the hybrid approach (simulation and measurement) or the pure measurement approach will be adopted to define the FR2 MIMO OTA performance requirements. Once [8] or more measurement results of different commercial devices per band are collected, the pure measurement approach will be adopted and simulation results will only be provided for information and not included in the data pool for requirement development. The detailed working procedures for specifying FR2 MIMO OTA performance requirements are described in Section 2.2.

The simulation efforts and measurement efforts can be conducted in parallel. To establish valid and trustable simulation and measurement data pools for defining FR2 MIMO OTA requirements, the following activities are required:

- Simulation platform validation activity: Companies shall complete simulation platform validation before submitting simulation results into the data pool for defining FR2 MIMO OTA requirements, validation results should be submitted to RAN4 for review. Details of the simulation platform validation activity is specified in 2.2.1.

- Channel model validation activity: Companies shall complete channel model validation before submitting measurement results, validation results should be submitted to RAN4 for review. Details of the channel model validation is specified in 2.2.2.

- [Lab alignment activity: Only aligned labs can share measurement results into the data pool for defining FR2 MIMO OTA requirements. Details of the lab alignment activity is specified in 2.2.3.]

- RAN4 should discuss and determine whether to perform an FR2 MIMO OTA lab alignment activity, make the decision on or before RAN4 106 meeting (Feb. 2023). At least [3] participating labs and at least [2] Performance Alignment Devices (PADs) per band are required.

2.2 Detailed working procedures

2.2.1 Simulation Platform Validation Activity

- The purpose of Simulation Platform Validation Activity is to ensure that simulation results can be aligned or correlated with measurement results.

- Method: Validate the simulation platform by comparing the simulation results with the measurement results using the same set of parameters as UE’s implementation. The simulation platform is allowed to be adjusted and improved during the activity towards meeting the pass/fail criteria.

- Band: [n261]

- The minimum number of devices (i.e., different sets of parameters of UE implementations for simulation) required per band: [2]

- Pass/fail criteria:

  - (1st priority) Each simulation result has a gap less than X dB with the corresponding measurement result. The value of X is FFS. If Criterion a. is met, the simulation results generated by the simulation platform(s) can be considered as aligned with measurement.

  - (If Criterion a. cannot be met, Criterion b. is acceptable.) The simulation results have a stable and reasonable gap with the measurement results. Detailed acceptable values of the gaps are FFS. If Criterion b. is met, the simulation results generated by the simulation platform(s) can be correlated with measurement.

2.2.2 Channel Model Validation

- The purpose of Channel Model Validation is to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone of the 3D-MPAC system.

- The channel model validation measurements shall be performed as described in Annex D.3 of TS 38.151, including:

  - Power delay profile (PDP)

  - Doppler/Temporal correlation

  - PAS similarity percentage (PSP)

  - Cross-polarization

  - Power validation

- Channel model: FR2 UMi CDL-C, as specified in Annex D.1 of TS 38.151

- Test band: [n261]

- Pass/fail limits: as defined in Annex D.2 of TS 38.151

[2.2.3 Lab Alignment Activity]

- The purpose of Lab Alignment Activity is to ensure there is no unexpected lab deviation and establish full trust and confidence on the measurement results. At least [3] participating labs and [2] PADs for each band are required. If RAN4 decides to perform the lab alignment activity, follow the working procedures as below.

- Test labs are invited to participate in the lab alignment activity, the following conditions should be fulfilled:

  - Participating labs shall complete channel model validation.

  - Participating labs should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TS 38.151

  - Test system: 3D-MPAC

- Test cases for Lab Alignment Activity:

  - Test band: n261

  - Number of test cases: 2~3 devices per-band

  - Operation mode: NR Standalone (SA)

- Test results submission:

  - Use the same worksheet template to submit the measurement results (a template will be submitted to RAN4 meetings for approval)

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - The reference value is derived based on the per-band averaging approach of lab alignment data pool from ≥ 3 labs, whether apparent outliers will be considered in averaging process, or not, is FFS

  - Pass/fail limit for lab alignment should be derived from the preliminary MU value. RAN4 should complete preliminary FR2 MU assessment before the end of Lab alignment activity.

2.2.4 Simulation Campaign

- The purpose of Simulation Campaign is to collect simulation results with different UE antenna assumptions which follow practical implementations from valid simulation platforms after the Simulation Platform Validation Activity for specifying FR2 MIMO OTA performance requirements.

- Simulation assumptions for FR2 MIMO OTA Simulation Campaign: TBD

  - The simulation assumptions agreed in R17 MIMO OTA WI can be considered as the baseline.

  - Companies are encouraged to provide inputs on different antenna configurations following practical UE implementations for collecting more simulation data.

- Simulation cases:

  - Band: [n261] (first stage)

  - Operation mode: NR Standalone (SA) (first stage)

  - Powe class: PC3 (first stage)

- Simulation results submission:

  - Use the same worksheet template to submit the simulation results (a template will be submitted to RAN4 meetings for approval)

2.2.5 Measurement Campaign

- The purpose of Measurement Campaign is to collect measurement results of commercial devices [from permitted labs after the Lab Alignment Activity] for specifying FR2 MIMO OTA performance requirements.

- Test cases for FR2 MIMO OTA Measurement Campaign:

  - Test band: [n261] (first stage)

  - Operation mode: NR Standalone (SA) (first stage)

  - Powe class: PC3 (first stage)

- Measurement results submission:

  - Use the same worksheet template to submit the measurement results (a template will be submitted to RAN4 meetings for approval)

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed). The following information should be provided:

    - All FR2 bands supported by each UE

    - Other information that should be disclosed is FFS

2.2.6 Specifying Performance Requirements

2.2.6.1 Pure measurement approach

- Minimum number of commercial devices for defining requirements: [8] per band

- Method: Derive the requirements from the pure measurement data pool

2.2.6.2 Hybrid approach

Starting point: one of the following three approaches can be adopted based on different applicable conditions.  Revisit and refine the approaches when some simulation and measurement results are available.

- Hybrid approach 1:

  - Applicable conditions: the simulation results are verified to be aligned with the measurement results

  - Method: Treat the simulation and measurement results equally and define the requirements based on the hybrid data pool including both simulation results and measurement data

  - Minimum amount of data in the hybrid data pool for defining requirements: [10] per band

- Hybrid approach 2:

  - Applicable conditions: the simulation results are verified to be aligned with the measurement results, or can be correlated with the measurement

  - Method: Define the requirements based on the hybrid data pool, and focus more on measurement results than simulation results

    - How to focus more on the measurement results to define FR2 requirements, is FFS

    - The minimum number of measurement results required for this approach, is FFS.

  - Minimum amount of data in the hybrid data pool for defining requirements: [10] per band

- [Hybrid approach 3 (with the lowest priority):]

  - Applicable conditions: the simulation results can be correlated with the measurement, and the amount of measurement results is less than [3]

  - Method: Define a range or tentative values with square brackets for FR2 MIMO OTA requirements based on simulations, further adjust the tentative requirements based on the measurement results to obtain final requirements

  - Minimum amount of data in the hybrid data pool for defining requirements: [10] per band

Proposal 1: Approve the Framework defined in this contribution to guide the FR2 MIMO OTA performance requirements related work.

3	Conclusion

In this paper, we propose the Framework for FR2 MIMO OTA performance requirements development.

Proposal 1: Approve the Framework defined in this contribution to guide the FR2 MIMO OTA performance requirements related work.

References

- R4-222668, “R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs”, CAICT, OPPO, Keysight Technologies, 3GPP RAN#97e, Sep. 2022.

- R4-2217461, “WF on NR MIMO OTA enhancement”, CAICT, 3GPP RAN4#104bis-e, Oct. 2022.