3GPP TSG-RAN WG4 Meeting#105                               R4-2219355

Toulouse, France,14th Nov – 18th Nov,2022

Agenda item:	8.13.2.3

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Rx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the last RAN4 meeting, we have reached good progress for ATG UE in [5], however there are still some open issues left for further discussion. In this contribution, we want to share further views on ATG UE RF requirements.

- Specify features to core specifications for ATG BS and UE [RAN4]

  - Scenario:

    - BS on the ground, and the CPE type of UE mounted in the aircraft

    - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft

    - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion.

  - Specify core requirements for coexistence between ATG and IMT terrestrial network

    - Example bands include n1, n78 and n79.

    - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS)

    - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs

      - Aim to reuse existing requirements for BS and UE where possible, e.g.,

        - Reuse TN BS requirements for ATG BS

    - Specify RF requirements for ATG UE/BS

      - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

      - Taking into account identified differences between ATG and fully ground based systems

      - Consider BS type 1-C/1-H/1-O and specify the requirements

      - Consider conductive requirements for UE

    - Specify RRM core requirements for ATG UE

      - Taking into account identified differences between ATG and fully ground based systems

      - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects

    - Specify new UE/BS type(s) for ATG network if necessary

Discussion

## 2.1. ATG UE requirements

In addition, in the following table 2, we share some further views on the ATG UE Rx RF requirement and its details could be found as following:

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No | During the last RAN4 meeting, there were some discussions for 2Rx or 4Rx for ATG UE, however no conclusion was draw on the end. First of all, as we highlighted for power class part, to use similar approach as IAB-MT, this could give more implementation freedom in practice. For IAB-MT introduced in Rel-16, it’s also not mandatory to support 4Rx for IAB-MT and its reference requirement for IAB-MT is defined per antenna connector FR1 IAB-MT type 1-H. In addition, if we consider the 4Rx for ATG CPE, at least two panels with cross-polarization on each panel are needed, however due to the limitation of installation on ATG UE on the aircraft, it will be difficult to have 4Rx in practice at least based on our experience. Based on the above clarifications, we propose to have 2 Rx branches as baseline and it’s not necessary to mandate 4Rx for band n1/n78 and n79 since antenna assumptions for ATG UE and handheld smartphone are different. |
| Reference sensitivity | Yes | - To follow the existing Reference requirements of band n1/78/79 for 2Rx in TS 38.101-1. |
| Maximum input level | No | This depend on the simulation results input to verify whether this requirement could be further relaxed similar as NR over NTN in Rel-17. |
| ACS | No | This depends on the coexistence study. |
| Blocking characteristics |  |  |
| - In-band | No | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Out-of-band | Partially Yes | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Narrow band | No | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be much lower compared with the handheld smartphone |
| Spurious response | No | Expect the same as TN and its power level of interfering signal could be further investigated similar as in-band blocking and out of band blocking. |
| Intermodulation | No | Not applicable |
| Spurious emissions | No | To reuse General requirements defined in TS 38.101-1 |

Conclusions

In this contribution, we shared some further views for ATG UE Rx RF requirements and proposals are made as following:

Proposal 1: to further discuss the proposals in Table 1 for ATG UE Rx RF requirements.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,	Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738	WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.