3GPP TSG-RAN WG4 Meeting#105-e                               R4-2219380

Toulouse, France,14th Nov – 18th Nov,2022

Agenda item:	8.23.3

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In this contribution, we want to share some further views on the SAN RF requirements for NTN in Ka-band.

The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022.

- Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4]

  - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4]

  - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4]

  - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4]

  - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4].

  - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4]

Discussion

## 2.1. NTN SAN type and SAN class

First of all, since Ka-band DL is close to low boundary 24.25GHz of FR2 definition and Ka-band UL is largely overlapping with TN band n257. Based on the experience for FR2 BS in the past, only BS type 2-O need to be defined otherwise insertion loss of connector between transceiver and antenna array would be quite high which will result in the reduced coverage. Based on the above considerations, we propose to define SAN type 2-O for NTN Ka-band unless other technical reasons from satellite vendors. In addition, as commented by other satellite vendors, conducted requirement is also possible in Ka-band which are different from the terrestrial network. From our understanding, for satellite antenna, there might be phase antenna array and reflection antenna with steerable spot antenna or multiple spot antenna.

Proposal 1: to define SAN type 2-O for NTN Ka-band and further discuss other antenna types e.g. reflection antenna architecture for Ka-band.

Table 2.1-1: Set-1 satellite parameters for system level simulation

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

## 2.2. NTN SAN RF requirements

Table 2.2-1. Summary of related RF requirement

| Tx requirements | Applicability notes |
| --- | --- |
| SAN output power | up to the declaration and no need to define any requirements |
| Output power dynamics |  |
| RE power control dynamic range | N/A |
| Total power dynamic range | N/A |
| Transmit ON/OFF power | N/A since the Ka-band is FDD band. |
| Transmitted signal quality |  |
| Frequency error | to reuse the same requirement defined in TS 38.108 |
| Modulation quality | to reuse the same requirement defined in TS 38.104. For DL 256QAM and 64QAM, this need more discussion similar as Rel-17 NR over NTN. |
| Time alignment error | this is not applicable for NTN SAN since its channel characteristic is difficult to support the MIMO which is similar as Rel-17 NR over NTN. |
| Unwanted emissions |  |
| Occupied bandwidth | to reuse the same requirement defined in TS38.108 which is following ITU-R Recommendation SM.328 |
| Adjacent Channel Leakage Power Ratio | this depends on the outcome of coexistence study. |
| Operating band unwanted emissions | this depends on the outcome of coexistence study if needed and ITU recommendation SM.1541-6 . Further study for FSS if the corresponding FSS spectrum could be included. |
| Transmitter spurious emissions | to reuse the same spurious emission requirement defined in TS 38.108, however the upper frequency limit for DL need to be updated as 2nd harmonic instead of 5th harmonic to follow the principle defined in ITU-R SM.329 |
| Transmitter intermodulation | Not applicable since it’s supposed to have no surrounding interfering gNB next to gNB |
| Rx requirement |  |
| Reference sensitivity level | Declaration basis according to the noise figure of SAN, antenna array configuration, the target SNR, FRC. The existing FRC for TN BS in FR2 could be reused if system parameter in other agenda is agreeable. It should be noted that antenna array between transmitter and receiver could be different which are different from the legacy TN BS. |
| Dynamic range | more evaluation results for IoT level in the uplink are needed similar as Rel-17 NR over NTN.. |
| ACS | This depends on the outcome of coexistence study. |
| In-band blocking | This depends on the outcome of coexistence study. |
| Out of band blocking | to start with OOBB power level as -44dBm and this should be changed to V/m in the OTA chamber. |
| Receiver spurious emission | Not applicable for FDD band operation |
| Receiver intermodulation | Not applicable since it’s supposed to have no surrounding interfering gNB next to gNB which is similar as transmitter intermodulation requirements, |
| In-channel selectivity | more evaluation results for IoT level in the uplink are needed similar as Rel-17 NR over NTN. |

Conclusions

In this contribution, we want to share some further views on NTN SAN RF requirements.and proposals are made as following:

Proposal 1: to define SAN type 2-O for NTN Ka-band and further discuss other antenna types e.g. reflection antenna architecture for Ka-band.

Proposal 2: to define SAN RF requirement for NTN in Ka-band as proposed in table 2.2-1.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2217496, Email discussion summary for [104-bis-e][312] NR_NTN_enh_Part1

- R4-2217467, WF on NR NTN Enhancements Part1, Thales, approved.

- https://artes.esa.int/projects/array-fed-reflector-antenna-fafr