3GPP TSG-RAN WG4 Meeting # 105	R4-2219401

Toulouse, France, November 14 – November 18, 2022

Title: 	Discussion on DL interruption for Tx switching across 3/4 bands

Source: 	Huawei, HiSilicon

Agenda item:	8.20.3.1

Document for:	Discussion

Introduction

In last meeting, R18 WI on Multi-carrier enhancements was discussed from RRM perspective. The agreements and progress are captured in WF [1]. This contribution further provides on DL interruption requirements for Tx switching across 3/4 bands.

Discussion

In Rel-16 and Rel-17, the mechanism and RAN4 requirements were specified for UE Tx switching between two bands. In Rel-18, Tx switching among 3 bands or 4 bands are to be supported. RF session agreed that the UL switching period across 3 or 4 bands in single TAG is the same as R16/R17 [2].

Agreement: On the length of switching period: - For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. - The length of switching period is applied per band pair for each band combination. - For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17. - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured. - For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17. On the UE complexity: RAN4 has not identified any technical difficulty for UE to prevent realizing Tx switching across 3 or 4 bands. RAN4 would like to recommend the UE memory sharing issue to be further discussed in RAN1 if necessary.

It is agreed in last meeting [1] that DL interruption length and starting point with single TAG are the same as that of R-17 TX switching between 2 bands. In other words, the discussion on DL interruption requirements for TX switching across 3/4 band with single TAG are completed.

Issue 1-2: DL interruption length <Agreement>: For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption. <Agreement>: Issue 1-3: Starting point of the DL interruption For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse the Rel-16/17 agreement on the starting symbol of DL interruption

Regarding multiple TAG, in last RF session it is further agreed that UL switching time with 2 TAG is the same for single TAG [3].

- Issue 2-1-1: UL switching time WF UL switching time (Oct 12 GTW agreement): the UL switching time is the same for single TAG and dual-TAG cases

Then the open issue of DL interruption length with 2 TAGs case is the length of MTRD. We think it is reasonable to consider non-collocated scenario for 2 TAGs. As we know UL TX switching across 3/4 band is typically applicable for urban scenario where the inter-site distance is not larger than 1km. Considering 3us TAE for inter-band CA and inter-site distance (about 900m), the receive time difference at UE is about 6us. To accommodate the non-collocated inter-band scenario for 2 TAGs, the RTD used to derive DL interruption length is 6us.

Issue 1-2: DL interruption length < Way forward >: For Rel-18 Tx switching across 3/4 bands with 2 TAGs, - Option 1: reuse Rel-16/Rel-17 values for length of DL interruption. - Option 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as: Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1 Where - Switching period is being discussed in RF session and the outcome can be reused in interruption design. - TA adjustment uncertainty remains same as legacy. - MRTD=3us which is used to derive R16/R17 DL interruption length is not valid for non-collocated case, further discuss the concrete value of MTRD

Proposal 1: Regarding the non-collocated inter-band scenario for Tx switching across 3/4 bands with 2 TAGs, the RTD used to derive DL interruption length is 6us.

Following the below formula of deriving DL interruption in R-16, the DL interrupted OFDM symbols for Tx switching across 3/4 bands with 2 TAGs is shown in Table 1.

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Table 1. DL interruption length in the unit of OFDM symbols (X) for switching across 3/4 uplink carriers with 2 TAGs

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | 8 |
| 2 | 0.25 | 4 | 10 | 14 |

It is observed that only the value which is marked in red is different with R16/R17 interruption, other values are the same. Therefore to simplify the requirements, the legacy DL interruption length is recommended to be reused for switching across 3/4 uplink carriers with 2 TAGs.

Proposal 2: For Rel-18 Tx switching across 3/4 bands with 2 TAGs, reuse Rel-16/Rel-17 values for length of DL interruption.

Conclusions

This contribution provides the analysis on DL interruption for Multi-carrier enhancements. The following proposals are provided:

Proposal 1: Regarding the non-collocated inter-band scenario for Tx switching across 3/4 bands with 2 TAGs, the RTD used to derive DL interruption length is 6us.

Proposal 2: For Rel-18 Tx switching across 3/4 bands with 2 TAGs, reuse Rel-16/Rel-17 values for length of DL interruption.

Reference

- R4-2217258, WF on UL Tx switching with multiple TAGs, Huawei, HiSilicon

- R4-2214464, Reply LS on UL Tx switching across 3 or 4 bands

- R4-2217742, WF on UL Tx switching with multiple TAG, Ericsson