3GPP TSG-RAN WG4 Meeting #105 	R4-2219483

Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Test Case Configuration for UE Transmit Timing

Agenda item:	6.2.5.5

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. For the past meetings, RAN4 has made progress in developing core requirements for NTN new features, as well as creating the performance testing configuration and specification to test such requirements. In this WI we aim at providing clarifications and addressing possible inconsistencies in the UE requirements. In the past meeting, more progress was made, and the way forward was captured in [2], which includes definitions and agreements over specification text regarding the mobility of the UE in idle mode.   This contribution proposes further clarification for issues regarding the performance requirements for timing related issues at the UE side.

Discussion

In the past meeting, the discussions amount performance requirements for UE transmit timing progressed, but nonetheless there are still some open issues that require RAN4 attention. In the WF provided in [2], one of the open issues regards the test configuration for UE transmit timing.

- Issue 5-2: Acquisition of UE location in UE timing test cases - Open issue: - Option 1: - UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused. - Option 2: - Use AT command approach to acquire UE location - AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates. - The exact UE position should be defined in such a way that the smallest elevation angle between the UE and satellite(s) is not smaller than 45 deg.

The main difference between both options is that, whereas in Option 1 the UE has to perform GNSS positioning for estimating its own position, in the second one the UE is provided by the test equipment with the “UE reference position” for the test.

From our point of view the option 2 does not satisfy the objectives of the present test. In special, regarding the following aspects:

- UE positioning for RRM purposes is a new functionality: For some other aspects in NTN, the test cases were developed assuming the baseline legacy behavior of terrestrial UEs plus modifications regarding the specificities of the NTN scenario. However, timing pre compensation should not be under similar assumption. In previous releases, there was no feature where the reliance of the Timing Advance procedure was solely at the UE side. Before, the timing advance was a closed-loop feature, controlled by network. In NTN, this is the first time the UE is in charge of calculating and implementing timing advance pre-compensation. Moreover, the UE has first to acquire its own position, the satellite ephemeris, perform some calculations and share this information with the lower layers for timing pre-compensation. This sort of functionality was not tested before, and it is a fundamental part of UE operation.

- GNSS inaccuracy and relaxation of the requirements: In RAN4 #101 [3], companies have agreed to relax the transmit timing accuracy requirement, on the grounds of the GNSS inaccuracy observed in NTN scenarios. This agreement shows the intention to consider the inaccuracy of GNSS positioning into NTN a relevant source of error for the UE procedure. If the exact UE position to be used in the test is conveyed to the UE, there is no inaccuracy in the UE positioning, and therefore the reason for the timing accuracy relaxation is no longer present. The relaxed requirement in this case would only allow a poorer UE implementation, on top of a new feature.

- Test of UE implementation of the feature: In LTE (NB-IoT and eMTC) discussions for NTN, there was an implementation of a validity timer for the GNSS fix of the UE in RAN1. The reason behind this decision is that some UEs may not have the capacity to update the reception of GNSS signals while monitoring the DL without gaps. We understand this is necessary due to the limited processing capabilities of IoT UEs. On the other hand, if the test setup provides the location for the UE, the UE with similar implementation  -  which is not expected under NR over NTN – may succeed the test, which may be prejudicial for the network performance.

- UE refresh rate of GNSS position: There are no requirements about how often and how UEs acquire their GNSS position. However, NTN UEs need to be capable to update their position as the UE moves, in order to maintain a tight control of timing advance. If the UE position is informed by the test equipment, the test case loses the capacity to test if a moving UE is updating its GNSS position. Some UEs might pass the test case, even if their implementation does not update GNSS position as often as necessary.

Based on the aforementioned, we then provide the following set of observations and proposals

For other RRM features, RAN4 has compromised developing test cases where the UE acquires its position from the test equipment.

For the transmit timing test, the usage of the AT command to convey UE position does not fully test the new functionality introduced in NTN.

For the acquisition of UE location in UE transmit timing test cases, we prefer Option 1: UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused.

Conclusion

In this paper we discussed aspects related to the test configuration and performance requirements for the cell reselection procedures, regarding the acquisition of SIB19.

In the paper, the following Observations and Proposals were made:

Observation 1: For other RRM features, RAN4 has compromised developing test cases where the UE acquires its position from the test equipment.

Observation 2: For the transmit timing test, the usage of the AT command to convey UE position does not fully test the new functionality introduced in NTN.

Proposal 1: For the acquisition of UE location in UE transmit timing test cases, we prefer Option 1: UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused.

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2217185, WF on performance part for NR, RAN4 #104-bis-e, Xiaomi

- R4-2120310, WF on GNSS-related and timing requirements for NR NTN, Xiaomi