3GPP TSG-RAN WG4 Meeting # 105	R4-2219502

# Toulouse, France, November 14 – November 18, 2022

Title: 	Discussions on remain issues on FR2-2 PDSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       6.3.6.2.1

Document for:	Discussion

Background

As per [1], RAN4 left some issues on FR2-2 UE demodulation and CSI requirements including single CC and FR1+FR2-2 CA. In this paper, we provide our views on these issues.

Discussions

Single CC requirements

Maximum Testable SNR

There are two options for this issue which are listed as below:

- Option 1: CBW (MHz); Num RBs; Test method. IFF. Single band UE; 100; 66; [9.8]. [32]; [13.2]. 400; 66; [2.6]. [32]; [6.6]. [20]; [8.9]. [16]; [10]. - Option 2: CBW (MHz)/SCS(kHz); Num RBs; Test method. IFF. Single band UE; 100/120; 66; [9.8]. 32; [13.2]. 400/480; 66; [2.6]. 32; [6.6]. 20; [8.9]. 16; [10].

It is noted that we removed the row about 400MHz/120kHz of option 2 since it was agreed that not to define PDSCH requirements for 400MHz/120kHz for single CC. Based on our understanding, option 2 is more reasonable since maximum testable SNR depends on actual transmission bandwidth which can be calculated approximately as product of SCS and allocated RB number rather than channel bandwidth. Hence we think it is clearer to specify SCS in this table because we define partial RB allocation requirements for different SCS and maximum testable SNR is different for different SCS for given RB allocation. Therefore, we suggest to use option 2.

Proposal 1: Use table specified in option 2 as maximum DL testable SNR derivation. I.e. Use following table:

|  | CBW (MHz)/SCS(kHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100/120 | 66 | 9.8 |
|  |  | 32 | 13.2 |
|  | 400/480 | 66 | 2.6 |
|  |  | 32 | 6.6 |
|  |  | 20 | 8.9 |
|  |  | 16 | 10 |

Detailed cases

The simulation results of target SNR[2] vs maximum testable SNR are captured in Table 2-1:

Table 2-1: Target SNR vs Maximum testable SNR

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB | Ideal SNR | Impairment SNR | Maximum Testable SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120kHz/100MHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 | -2.2 | -0.2 | 9.8 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 | 6.0 | 8.0 | 9.8 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 | -0.9 | 1.1 | 9.8 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 66 | 8.6 | 10.6 | 9.8 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 32 | 8.8 | 10.8 | 13.2 |
| 480kHz/400MHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 | -2.7 | -0.7 | 2.6 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 20 | 5.2 | 7.2 | 8.9 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 32 | 4.9 | 6.9 | 6.6 |

We can observe that target SNR is higher than maximum testable SNR in following cases:

- 120kHz/100MHz, MCS17, TDLD-300

- 480 kHz/400MHz, MCS13. TDLD10-200

Observation 1: Target SNR is larger than maximum testable SNR in following cases:

- 120kHz/100MHz, MCS17, TDLD-300

- 480 kHz/400MHz, MCS13. TDLD10-200

Considering the test coverage and testability, we propose to define PDSCH requirements for following cases listed in Table 2-2:

Table 2-2: Proposed cases for FR2-2 PDSCH requirements

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB |
| --- | --- | --- | --- | --- | --- |
| 120kHz/100MHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 32 |
| 480kHz/400MHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 20 |

Proposal 2: Define FR2-2 PDSCH requirements for cases listed in Table 2-2.

TRS configuration

The options of this issue are listed as follows:

Whether the TE transmit TRS in the full CBW during PDSCH testing - Option 1: Yes, further discuss the impact on TRS SNR; - Option 2: Transmit TRS in unallocated PRB only if PDSCH is not multiplexed with TRS (see Issue 1-2-2), or otherwise if there is no impact on maximum SNR; - Option 3: Schedule same RB size for TRS and PDSCH; - Option 4: Refer to TE recommendation;

In last meeting, some companies concern that whether it is feasible to transmit TRS+PDSCH with full bandwidth allocation. However, TE vendor didn’t provide clear feedback in last meeting. Hence we provide our analysis as follows: Assume TRS is transmitted with full bandwidth (66RBs) occupying 3 REs per RB per symbol. (See Figure 2-1).

Figure 2-1: TRS symbol transmission

For case with 120kHz/100MHz, MCS17, TDLD30-200, PDSCH is allocated with 32RBs, the number of RE allocated for TRS which is out of  PDSCH transmission bandwidth is (66-32) *3=102, which can be approximately equivalent to 9 RBs .Therefore, the actual total number of RBs used for TRS+PDSCH transmission is 32+9=41RBs. The maximum testable SNR for 41RBs is 12.1dB based on the link budget calculation which leaves enough margin for the test. (The impairment target SNR according to our simulation results is 10.8dB).  Hence, for this case, we think there is no problem for testability when TRS are transmitted with full bandwidth in PDSCH scheduled slots.

Observation 2: For case with 120 kHz/100MHz, MCS17, TDLD30-200 that TRS is transmitted with full bandwidth and PDSCH is transmitted with 32RBs, it can be approximately equivalent to 41 RBs used for TRS and PDSCH transmission, the maximum testable SNR of which is 12.1dB, which leaves enough margin between impairment SNR and maximum testable SNR.

For case with 480kHz/400MHz, MCS13, TDLD10-200. PDSCH is allocated with 20RBs, the number of RE allocated for TRS which is out of PDSCH transmission bandwidth is (66-20) *3=138, which can be approximately equivalent to 12 RBs. Therefore, the actual total number of RBs used for TRS+PDSCH transmission is 20+12=32RBs. The maximum testable SNR for 32RBs is 6.8dB which is much closed to our impairment results. (The impairment target SNR according to our simulation results is 6.9dB). Hence, for this case, we think there is problem for testability when TRS are transmitted with full bandwidth in PDSCH scheduled slots.

Observation 3: For case with 480kHz/400MHz, MCS17, TDLD30-200 that TRS is transmitted with full bandwidth and PDSCH is transmitted with 20RBs, it can be approximately equivalent to 32 RBs used for TRS and PDSCH transmission, the maximum testable SNR of which is 6.8 dB, which leads to the problem that impairment SNR is very closed to maximum testable SNR.

Based on the above analysis, we propose the following:

For case with 120 kHz/100MHz, MCS17, TDLD30-200 (32RBs allocation), TRS is full bandwidth transmitted in PDSCH scheduled slot.

For case with 480 kHz/400MHz, MCS13, TDLD10-200(20RBs allocation), FFS for following candidate options:

- Option 1: Reduce the PDSCH RB allocation to 16RBs

- Option 2: Not transmit PDSCH in TRS slot

- Option 3: Schedule same RB size for TRS and PDSCH.

Proposal 3: For case with 120 kHz/100MHz, MCS17, TDLD30-200 (32RBs allocation), TRS is full bandwidth transmitted in PDSCH scheduled slot. For case with 480kHz/400MHz, MCS13, TDLD10-200(20RBs allocation), FFS for following candidate options:

- Option 1: Reduce the PDSCH RB allocation to 16RBs

- Option 2: Not transmit PDSCH in TRS slot

- Option 3: Schedule same RB size for TRS and PDSCH.

FR1+FR2-2 CA requirements

We spent less effort on FR1+FR2-2 requirements discussion in past meetings. However, FR1+FR2-2 CA is commonly used and mandatory for UE to support. We list the corresponding open issues according to [1] as follows:

Bandwidth choice and test setup for FR1+FR2-2 CA requirements - Option 1: - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is 66RBs - Option 2: - 100MHz/120KHz - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is 66RBs - 400MHz/480KHz - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration. - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). - CQI requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is 66RBs Applicability rule for FR1+FR2-2 CA requirements - Option 1: - Copy all cases of single CC to CA and define applicability rules as follows: - If UE support standalone operation, UE can only be tested cases with single CC. - If UE don’t support standalone operation, UE can only be tested cases with CA Configurations for FR1 CC for FR1+FR2-2 CA requirements - Companies can provide configurations for FR1 CC for FR1+FR2-2 CA requirements next meeting. Test cases for FR1+FR2-2 CA requirements - Option 1: Reuse all test cases of single CC to CA requirements How to define FR1+FR2-2 CA requirements - Only define the requirements for FR2-2 CC - Option 1: Apply all the requirements of single CC to CA requirements - Option 2: Define separate requirements for CA - Companies can provide simulation results or analysis next meeting to support your views

Bandwidth combination for FR1+FR2-2 CA requirements

We copy all FR1+FR2-2 CA combination configurations in Table 5-1 which is part of Table 5.5A.1-1k of 38.101-3 and all FR2-2 CA configuration in Table 5-2 which is part of Table 5.5A.1 of 38.101-2  in appendix. We can observe that the minimum bandwidth of FR2-2 CC is 400MHz for CA configuration “FR1(Single CC)+FR2-2(Single CC)” . Therefore, we think option 2 for 120kHz SCS conflict the RF agreements, i.e. no 100MHz CBW for FR2-2 single CC in FR1+FR2-2 CA.

Observation 4: The minimum bandwidth of FR2-2 CC is 400MHz for CA configuration “FR1(Single CC)+FR2-2(Single CC)” . Option 2 for 120kHz SCS conflict the RF agreements.

We can observe that RF defined too many FR1+FR2-2 CA configurations, if we follow the Rel-15 PDSCH requirements, we need to define requirements for all FR2-2 mandatory channel bandwidth (100MHz and 400MHz) and choose all bandwidth defined for FR1 for test setup to cover all possible CA combinations. However, this will cause heavy workload for simulation work and complex test setup. Based on our understanding, we can choose the simplest CA combination which can be supported by most UE.We can observe that CA_n48A-n263A is the simplest CA configuration among all FR1+FR2-2 CA configurations which has only one FR1 CC and one FR2-2 CC. Regarding all possible bandwidth combinations of CA_n48A-n263A, we suggest to only select 40MHz for FR1 CC and 400MHz for FR2-2 CC which is typical CA combination and no need to define requirements for all channel bandwidth like did for FR1 CA in Rel-16..

Observation 5: CA_n48A-n263A is the simplest CA configuration among all FR1+FR2-2 CA configurations. Meanwhile, 40MHz+400MHz is the most typical CA configuration regarding to all bandwidth combination of CA_n48A-n263A which will be supported by every UE.

Based on the above analysis, we propose the following:

Proposal 5: RAN 4 to choice following bandwidth configuration to define FR1+FR2-2 CA requirements:

- FR1 single CC with 40MHz+FR2-2 single CCs with 400MHz

Scheduled and feedback pattern

Different from single CC scheduling pattern, FR1+FR2-2 CA uses UL slot of FR1 to transmit HARQ-ACK, which means the periodicity of scheduling of FR2-2 should be aligned with that of FR1 TDD pattern. Hence, we propose the following:

For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1. (See Figure 2-2)

Figure 2-2: Schedule pattern of FR1+FR2-2 with 120kHz

For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

Proposal 6: Considering following scheduling and feedback pattern for FR1+FR2-1 CA requirements:

- For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

- For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

How to define requirements for FR1+FR2-2 CA requirements

The difference of test setup between CA requirements and single CC are summarized as follows:

- Test setup for single CC:

- 120kHz

- Bandwidth: 100MHz

- Schedule pattern:

  - PDSCH is scheduled with one TDD pattern periodicity (DDDSU).

- 480kHz

- Bandwidth: 400MHz

- Schedule pattern:

  - One scheduling pattern includes two TDD pattern period which is 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U.

  - There are 7 DCIs in every scheduling pattern and each DCI schedules 4 PDSCHs if not overlapped with SSB, otherwise, each DCI schedules 3 PDSCHs. PDSCH is scheduled in every DL slots except slot #33 and #34 and slot contain SSB

- Test setup for CA:

- 120kHz

- Bandwidth: 400MHz

- Schedule pattern: See proposal 4

- 480kHz

- Bandwidth: 400MHz

- Schedule pattern: See proposal 4

The simulation results are captured in Table 2-3 for single CC and CA with same RB allocation:

Table 2-3 Requirements for single CC vs CA

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB | Ideal SNR for Single CC | Ideal SNR for CA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120kHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 | -2.3 | -2.2 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 | 6.3 | 6.0 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 | -1.1 | -0.9 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | 32 | 8.5 | 8.8 |
| 480kHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 | -2.3 | -2.7 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | 20 | 5.3 | 5.2 |

We can observe that there is negligible difference of performance between CA and single CC. Therefore we think it is feasible to apply requirements of single CC to CA.

Observation 6: There is negligible difference of performance between CA and single CC.

Proposal 7: Apply the requirements of single CC to CA.

Configuration of Pcell CC

In order to simplify the test setup, we suggest to not schedule PDSCH and any DL reference signal except SSB in Pcell CC. I.e. Pcell CC is only used for initial access and HARQ-ACK reception. The proposed test parameters are listed in Table 2-4:

Table 2-4: Proposed test parameters for Pcell CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | 7D1S2U S=6D:4S:4U |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | See FR1.30-1 as defined in Annex A.1.2 |

Proposed 8: Use parameters listed in Table 2-4 for Pcell CC for CA test.

Conclusion

In this paper, we provide our views on FR2-2 open issues. The observations and proposals are:

Proposal 1: Use table specified in option 2 as maximum DL testable SNR derivation. I.e. Use following table:

|  | CBW (MHz)/SCS(kHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100/120 | 66 | 9.8 |
|  |  | 32 | 13.2 |
|  | 400/480 | 66 | 2.6 |
|  |  | 32 | 6.6 |
|  |  | 20 | 8.9 |
|  |  | 16 | 10 |

Observation 1: Target SNR is larger than maximum testable SNR in following cases:

- 120kHz/100MHz, MCS17, TDLD-300

- 480 kHz/400MHz, MCS13. TDLD10-200

Proposal 2: Define FR2-2 PDSCH requirements for cases listed in Table 2-2.

Observation 2: For case with 120 kHz/100MHz, MCS17, TDLD30-200 that TRS is transmitted with full bandwidth and PDSCH is transmitted with 32RBs, it can be approximately equivalent to 41 RBs used for TRS and PDSCH transmission, the maximum testable SNR of which is 12.1dB, which leaves enough margin between impairment SNR and maximum testable SNR.

Observation 3: For case with 480kHz/400MHz, MCS17, TDLD30-200 that TRS is transmitted with full bandwidth and PDSCH is transmitted with 20RBs, it can be approximately equivalent to 32 RBs used for TRS and PDSCH transmission, the maximum testable SNR of which is 6.8 dB, which leads to the problem that impairment SNR is very closed to maximum testable SNR.

Proposal 3: For case with 120 kHz/100MHz, MCS17, TDLD30-200 (32RBs allocation), TRS is full bandwidth transmitted in PDSCH scheduled slot. For case with 480kHz/400MHz, MCS13, TDLD10-200(20RBs allocation), FFS for following candidate options:

- Option 1: Reduce the PDSCH RB allocation to 16RBs

- Option 2: Not transmit PDSCH in TRS slot

- Option 3: Schedule same RB size for TRS and PDSCH.

Observation 4: The minimum bandwidth of FR2-2 CC is 400MHz for CA configuration “FR1(Single CC)+FR2-2(Single CC)” . Option 2 for 120kHz SCS conflict the RF agreements.

Observation 5: CA_n48A-n263A is the simplest CA configuration among all FR1+FR2-2 CA configurations. Meanwhile, 40MHz+400MHz is the most typical CA configuration regarding to all bandwidth combination of CA_n48A-n263A which will be supported by every UE.

Proposal 5: RAN 4 to choice following bandwidth configuration to define FR1+FR2-2 CA requirements:

- FR1 single CC with 40MHz+FR2-2 single CCs with 400MHz

Proposal 6: Considering following scheduling and feedback pattern for FR1+FR2-1 CA requirements:

- For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

- For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1.

Observation 6: There is negligible difference of performance between CA and single CC.

Proposal 7: Apply the requirements of single CC to CA.

Proposed 8: Use parameters listed in Table 2-4 for Pcell CC for CA test.

Reference

[1]   R4-2217391 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated

[2]   Simulation results for FR2-2 PDSCH demodulation requirements. Huawei HiSilicon

Appendix

Table 5-1: CA combination for FR1+FR2-2 CA

| CA_n48A-n263A | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48A-n263G | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48A-n263H | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48A-n263I | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48A-n263J | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48A-n263K | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48A-n263L | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48A-n263M | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(2A)-n263A | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(2A)-n263G | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(2A)-n263H | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(2A)-n263I | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(2A)-n263J | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(2A)-n263K | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(2A)-n263L | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(2A)-n263M | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48B-n263A | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48B-n263G | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48B-n263H | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48B-n263I | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48B-n263J | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48B-n263K | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48B-n263L | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48B-n263M | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(A-B)-n263A | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(A-B)-n263G | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(A-B)-n263H | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(A-B)-n263I | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(A-B)-n263J | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48C-n263A | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48C-n263G | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48C-n263H | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48C-n263I | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48C-n263J | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48C-n263K | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48C-n263L | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48C-n263M | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(3A)-n263A | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(3A)-n263G | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(3A)-n263H | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(3A)-n263I | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(3A)-n263J | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(3A)-n263K | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(3A)-n263L | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(3A)-n263M | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(4A)-n263A | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(4A)-n263G | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(4A)-n263H | CA_n48A-n263A | n48 | CA_n48(4A) |  |
|  |  | n263 | CA_n263H | 0 |
| CA_n48(4A)-n263I | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(4A)-n263J | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(4A)-n263K | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(4A)-n263L | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(4A)-n263M | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263M |  |

Table 5-2: Configurations for FR2-2 CA

| CA_n263B | CA_n263A | 400 | 400 |  |  |  |  |  |  | 800 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n263C | CA_n263A | 400 | 400 | 400 |  |  |  |  |  | 1200 | 0 | 1 |
| CA_n263G | CA_n263A | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n263H | CA_n263A | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 | 3 |
| CA_n263I | CA_n263A | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 | 3 |
| CA_n263J | CA_n263A | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 | 3 |
| CA_n263K | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 | 3 |
| CA_n263L | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 | 3 |
| CA_n263M | CA_n263A | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 | 3 |