3GPP TSG-RAN WG4 Meeting # 105	R4-2219508

# Toulouse, France, November 14 – November 18, 2022

Title: 	Simulation results on FR2-2 PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:        6.3.6.3.1

Document for:	Discussion

Background

In this paper, we provide our simulation results for FR2-2 PUSCH demodulation requirements based on simulation assumptions derived from [1].

Simulation results

The simulation assumptions are captured in Table 2-1. Based on our previous analysis, performance only benefit from ICI compensation under the condition of large number of PTRS samples and high MCS. Therefore, we provide the CPE compensation results for cases with 66RBs allocation and 16QAM. For cases with 264RB allocation and 64QAM, we provide results for both CPE and CPE+ICI compensation.

Table 2-1 Common simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled, Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0}, {0, 1} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | 2, Disabled |
|  | Time density (LPT-RS) | 1, Disabled |
| Phase noise model |  | Set 2 in TR 38.808 |
| Phase noise compensation |  | For case with 120kHz SCS, 400MHz bandwidth, MCS 18 and 20: CPE+ICI For other cases: CPE only |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

The simulation results for CP-OFDM for minimum bandwidth for each SCS are captured in Table 2-2:

Table 2-2: Simulation results for CP-OFDM for minimum bandwidth for each SCS

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP (WO PN) | 70% of max TP (With PN) | Performance Loss (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | -2.5 | -2.1 | 0.4 |
|  |  |  |  | 2x2 Low | 2.0 | 2.4 | 0.4 |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 8.8 | 9.6 | 0.7 |
|  |  |  |  | 2x2 Low | 17.4 | 19.1 | 1.7 |
| 120 | 100 | 16 | TDLD30-200 | 2x2 Low | 10.5 | 11.3 | 0.8 |
| 120 | 100 | 18 | TDLD30-200 | 2x2 Low | 11.6 | 12.6 | 1.0 |
| 120 | 100 | 20 | TDLD30-200 | 1x2 Low | 9.7 | 10.9 | 1.2 |
|  |  |  |  | 2x2 Low | 13.6 | 15.2 | 1.6 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | -3.8 | -3.5 | 0.3 |
|  |  |  |  | 2x2 Low | -0.3 | 0.2 | 0.5 |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 8.0 | 8.7 | 0.7 |
|  |  |  |  | 2x2 Low | 15.4 | 17.1 | 1.7 |
| 480 | 400 | 16 | TDLD10-200 | 2x2 Low | 10.2 | 11.0 | 0.8 |
| 480 | 400 | 18 | TDLD10-200 | 2x2 Low |  | 12.4 |  |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 9.7 | 10.9 | 1.2 |
|  |  |  |  | 2x2 Low | 13.4 | 15.1 | 1.7 |

The simulation results for CP-OFDM for 120kHz/400MHz are captured in Table 2-3.

Table 2-3: Simulation results for CP-OFDM for 120kHz/400MHz

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP (WO PN) | 70% of max TP (With PN CPE) | 70% of max TP (With PN CPE+ICI) | Performance Loss (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 400 | 4 | TDLA30-650 | 1x2 Low | -2.3 | -2.1 | / | 0.2 |
|  |  |  |  | 2x2 Low | 2.1 | 2.3 | / | 0.2 |
| 120 | 400 | 16 | TDLA30-650 | 1x2 Low | 8.9 | 9.5 | / | 0.6 |
|  |  |  |  | 2x2 Low | 17.9 | 19.4 | / | 1.5 |
| 120 | 400 | 16 | TDLD30-200 | 2x2 Low | 10.6 | 11.4 | / | 0.8 |
| 120 | 400 | 18 | TDLD30-200 | 2x2 Low | 11.7 | 12.7 | 13.4 | 1.0 |
| 120 | 400 | 20 | TDLD30-200 | 1x2 Low | 9.9 | 11.1 | 11.1 | 1.2 |
|  |  |  |  | 2x2 Low | 13.7 | 15.4 | 15.4 | 1.7 |

The simulation results for DFT-S-OFDM are captured in Table 2-4:

Table 2-4 Simulation results for DFT-S-OFDM

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | -1.8 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | -2.3 |

Conclusion

In this paper we provide our simulation results for FR2-2 PUSCH requirements.

Reference

[1]   R4-2217372_WF_PUSCH_FR2-2. Nokia, Nokia Shanghai Bell.