3GPP TSG-RAN WG4 Meeting # 105	R4-22xxxxx

# Toulouse, France, Novemeber 14 – Novemeber 18, 2022

Title: 	Simulation results on Adaptation Absolute Physical Layer requirements

Source: 	Huawei, HiSilicon

Agenda item:       8.17.1

Document for:	Discussion

Background

RAN 4 has studied the feasibility of defining requirements for Adaptation Absolute Physical Layer requirements and related study outcome can be found in [1]. It was agreed that it is feasible to define the absolute physical layer throughput requirements under link adaptation. This meeting is the first meeting for WI stage and we will provide our corresponding simulation results.

Simulation results

The simulation results are captured in Table 2-2 based on the simulation assumptions in Table 2-1

Table 2-1: Simulation assumptions Table

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

Table 2-2: Simulation results of SNR@ T% of max TP

|  | FR1 FDD |  |  |  | FR1 TDD |  |  |  | FR2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  | 4 Rx UE |  | 2 Rx UE |  |
| % of Max T-put | Ideal | Impariment | Ideal | Impariment | Ideal | Impariment | Ideal | Impariment | Ideal | Impariment |
| 10 | 3.8 | 5.3 | 0.7 | 2.2 | 3.5 | 5.0 | 0.1 | 1.6 | 1.6 | 3.1 |
| 15 | 6.6 | 8.1 | 4.0 | 5.5 | 6.9 | 8.4 | 3.5 | 5.1 | 4.4 | 5.9 |
| 20 | 9.5 | 11.0 | 5.4 | 6.9 | 10.3 | 11.8 | 5.5 | 7.0 | 6.7 | 8.2 |
| 25 | 12.3 | 13.8 | 7.4 | 8.9 | 13.4 | 14.9 | 7.1 | 8.6 | 10.3 | 11.8 |
| 30 | 14.8 | 16.3 | 9.1 | 10.6 | 15.7 | 17.2 | 8.9 | 10.4 | 12.2 | 13.7 |
| 35 | 16.6 | 18.1 | 10.6 | 12.1 | 17.7 | 19.2 | 10.6 | 12.1 | 14.4 | 15.9 |
| 40 | 18.8 | 20.3 | 12.1 | 13.6 | 19.8 | 21.3 | 12.4 | 13.9 | 16.2 | 17.7 |
| 45 | 3.8 | 5.3 | 13.9 | 15.4 |  |  | 14.5 | 16.0 | 17.9 | 19.4 |
| 50 |  |  | 15.6 | 17.1 |  |  | 16.5 | 18.0 | 18.9 | 20.4 |
| 55 |  |  | 17.1 | 18.6 |  |  | 18.3 | 19.8 | 19.9 | 21.4 |
| 60 |  |  | 18.7 | 20.2 |  |  | 19.8 | 21.3 |  |  |

Reference

[1]   37901-5

[2]   R4-2113123-Summay UE Application Layer Data Throughput. Intel